Prilog IV

Surovini i pomo[ni materijali i energii upotrebeni ili proizvedeni vo instalacijata

Sodr`ina:

IV.1 Opis na surovini	2

IV.1.1 Dolomit	3

IV.1.2 Kalcit	4

IV.1.3 Siv varovnik	4

IV.2 Pomo{ni materijali	5

IV.2.1 Materijali za minerski raboti	5

IV.2.1.1 Plasti~ni eksplozivi	5

IV.2.1.2 Pra{kasti eksplozivi	6

IV.2.1.3 Bavnogore~ki fitil	6

IV.2.2 Odr`uvawe na mehanizacija	6

IV.2.3 Sredstva za higiena i za{tita pri rabota	7

IV.3 Energensi	7

IV.3.2 Dizel gorivo	7

IV.3.3 Elektri~na energija	7

IV.4 Voda	7

IV.5 Proizvodi	9

IV.6 Prilozi	10

IV.6.1 Potvrda za rakuvawe so dizel gorivo pri polnewe na cisterni	11

IV.6.2 Karakteristiki na surovini/proizvodi	12

IV.6.2.1 Izve{taj od ispituvawe na fizi~ko hemiski karakteristiki na siv varovnik	13

IV.6.2.2 Fizi~ko hemiski karakteristiki na supstanciite sodr`ani vo eksplozivot	15

IV.6.3 Safety Data sheets	24

IV.6.3.1 Kalcium karbonat	24

IV.6.3.2 Silicium Dioksid	25

IV.6.3.3 Dizel ekstra lesno gorivo	26

IV.6.3.4 Glicerol - maziva	27

IV.6.3.5 Trinitrotoluol	29

IV.6.3.6 Dinitrotoluol	30

IV.6.4 Dispozicija na objekti na lokacijata-sega{na sostojba	0

### IV.1 Opis na surovini

Vo procesot na proizvodstvo i prerabotka se koristat nemetalni mineralni surovini koi se dobivaat preku iskopuvawe i predhodno primarno drobewe.  Surovinite koi se koristat i mo`at da se prerabotat imaat tvrdina 8 po Mosovata skala.

Dostavuvaweto na surovinite e od sopstveniot iskop na mermerizirani varovnici na mesnosta “Dobaraski @eden ” vo krugot na samata instalacija.

Kalcium (Ca)  gi ima slednite alotropski modifikacii:

- 	kalcium karbonat

- 	mermeri i

- 	gips

^istiot kalcium najmnogu se javuva vo vselenata i slu`i za za{tita na obvivkata. Drugi karakteristiki se slednite:

- reden broj po Mendelej		20

- atomska te`ina			40,08

- specifi~na te`ina		1,58

Srebreno bel, lesen alkalen nemetal. Vo prirodata se nao|a vrzan vo mnogu soedinenija (varovnik, mermer gips i dr.). ajva`en element za razvitokot i opstanokot na `iviot i rastitelniot svet na zemjata, samo mal broj organizmi moraat da e razvivaat i opstanat vo sredini kade nema Ca (primer: gabite Aspergilus niger).

Kaj ~ovekot i site sisari e neophoden za razvitokot na koskeniot skelet, kade 25% go so~inuva Ca, a ostanatiot del opa|a na muskulnata masa na koja isto taka presuden faktor za razvojot ima kalciumot.

Vo organizmot se vnesuva preku hranata i vodata vo oblik na organski soli i minerali.

Dnevnata potreba na organizmot za Ca za pravilen razvoj iznesuva od 0,9-1,5 gr/24~asa za vozrasni, dodeka za deca, mladinci i bremeni `eni ovaa doza e pogolema.

Vo nedostatok na Ca vo organizmot kaj ~ovekot doa|a do pojava na razni zaboluvawa i popre~uvawe na razvitokot kako {to se rahitis, osteoporoza i dr, a kaj rastitelniot svet doa|a do su{ewe na korenovite vlakna i odumirawe na lisnata masa.

Ne mo`e da se zamisli razvojot na industrijata, kako {to se metalurgijata, hemiskata, farmacevtskata, prehrambenata i grade`nata, kade Ca ne na{ol primena kako ~ist ili vo forma na kalciumovi soedinenija.

Op{t e vpe~atokot deka bez prisustvo na kalcium ~ist ili vo forma na soedinenija, neodr`liv bi bil `ivotot i razvojot na planetata Zemja.

Surovinite voglavno se podeleni na dve grupi spored svojata priroda i namena:

- Karbonatni surovini:

  - dolomiti,

  - kalciti

  - siv varovnik

### IV.1.1 Dolomit

Prestavuva sedimentna karbonatna stena so kristalna mineralna struktura sostaven od kalcium magnezium karbonat CaMg(CO3)2.

Karakteristikite mu se sli~ni na tie od kalcitite so razlika {to ne se razlo`uva burno vo razbla`ena hlorovodorodna kiselina. Tvrdinata po Mos e od 3,5 do 4, a specifi~nata te`ina e 2,85. Formira cvrsti rastvori so `elezoto. Mali koli~ini na `elezo vo strukturata dava `olta do kafeava boja na mineralite.

Magneziumot vo strukturata na mineralot vo forma na MnO u~estvuva do 3 %. Visoka sodr`ina na magnezium dava roze boja na mineralot. Olovoto i cinkot vo strukturata na mineralot mo`e da go zamenat magneziumot.

### IV.1.2 Kalcit

Karbonatniot mineral kalcit e hemiski ili biohemiski kalcium karbonat so hemiska formula CaCO3 i e eden od najrasprostranetite minerali vo zemjinata kora. Isto taka delumno vleguva vo sostav na sedimentnite steni i varovnikot.

Isto taka e primaren mineral vo metamorfniot mermer. Se pojavuva vo naslagite od geotermalni vodi, pe{teri kao stalaktit i stalagmit. Kalcitot prestavuva stabilna forma na kalcium karbonat; argonitot preminuva vo kalcit na 4700C , dodeka vateritot μ-CaCO3 e ponestabilen. Se pojavuva vo fibrozna, granularna, lamelarna ili kompaktna forma.

Po Mosovata skala ima tvrdina od 2,71, Bojata mu e bela ili bezbojna so nijansi na siva, crvena. `olta, zelena, sina, violetova, kafeava pa duri i crna vo slu~ai koga mineralot sodr`i primesi.

### IV.1.3 Siv varovnik

Karbonatna stena so siva boja so finozrnesta struktura sostavena od mermerizirani varovnici {to burno reagira so ladna (10%) HCl kiselina {to se karakterizira za sitnozrni kalcitni mermeri.

Hemiskiot sostav na siviot varovnik e prika`an vo Tabelata 1

| Oksidi | % |
| --- | --- |
| CaO | 55.16 - 55.44 |
| MgO | 0.25 – 0.55 |
| P2O3 | 0.18 – 0.20 |
| SiO2 = ner | 0.18 – 0.40 |
| Zag.zarenje | 43.65 – 43.78 |
| Suma | 99.82 – 99.89 |
| MgCO3 | 0.52 – 1.15 |
| CaCO3 | 98.44 – 98.94 |
| SO3 | / |

Tabela 1: Hemiski sostav na siviot varovnik

### IV.2 Pomo{ni materijali

Koristeweto na pomo{nite materijali se odnesuva na:

- Minerski aktivnosti

- odr`uvaweto na mehanizacijata i opremata,

- sredstva za odr`uvawe na higiena kako i

- sredstva za za{tita pri rabota.

### IV.2.1 Materijali za minerski raboti

Stopanskiot eksploziv se nabavuva za potrebite za minirawe na povr{inskiot kop i se izveduva od ovlastena kompanija za izveduvawe na minerski aktivnosti. Godi{nata potro{uva~ka e 30 toni, bavno gore~ki fitil (12000 m), detonatori DK-8 (48 par~iwa), milisekundni zabavuva~i (600 par~iwa), kruni (8 par~iwa), cevki (4 par~iwa).

### IV.2.1.1 Plasti~ni eksplozivi

Plasti~niot eksploziv sodr`i toksi~ni komponenti kako nitroglikol, dinitro i trinitro toluol, koi mo`at da go zagrozat zdravjeto na vrabotenite so vdi{uvawa na isparuvawata i nivnata apsorbcija preku ko`ata. Pra{kastite eksplozivi kako brizantna komponenta sodr`at dinitro i trinitro- toluol. Iako se patronirani na skoro identi~en na~in kako i plasti~nite eksplozivi, mo`no e prisustvo na isparuvawa i one~istuvawa na ambala`ata so eksploziv. Zatoa ne e dozvolena manipulacija so plasti~niot i pra{kastiot eksploziv bez gumeni rakavici.

Po zavr{enata manipulacija so eksplozivot rabotnicite treba da gi isperat racete. Plasti~niot eksploziv e osetliv na zgolemena temperatura nad +30 oS koga mo`e da dojde do izdvojuvawe na nitroglicerinot i nitroglikolot od eksplozivot, a pri pomala temperatura od -20 0C mo`e da dojde do smrznuvawe na nitroestrite. I vo dvata slu~ai eksplozivot e opasen za manipulacija i ne smee da se izdava za koristewe.

### IV.2.1.2 Pra{kasti eksplozivi

Za pra{kastiot eksploziv se nepo`elni temperaturi nad +300C, koga promenata na kristalnata struktura, amonium nitratot stvrdnuva i go pravi eksplozivot neupotrebliv. Ista sostojba e pri dispergiraniot pra{kast eksploziv, a sli~na i pri ANFO eksplozivot, kako kaj pra{kastite eksplozivi. Osnovna eksplozivna komponenta na detonatorskiot fitil e pentritot, koj ne e toksi~en. Dobro ambala`iran ne mo`e da predizvika posledici kaj vrabotenite.

### IV.2.1.3 Bavnogore~ki fitil

Se koristi za inicirawe na rudarskate kapisla i ima jadro od crn dimen barut obmotan so pove}e pamu~ni konci i izolacija od PVC ili bitumen, protiv vlaga. Se pali so pomo{ na kibrit.

### IV.2.2 Odr`uvawe na mehanizacija

Rezervnite delovi kako masti i maziva za podma~kuvawe i odr`uvawe na opremata i mehanizacijata se skladirani vo mehani~arskata rabotilnica.

Motorno maslo i hidrauli~no maslo za odr`uvawe na mehanizacijata se ~uvaat vo mehani~arskata rabotilnica na soodvetno mesto za taa namena.

Vo tek se prevzemawe aktivnosti za izgradba na sklad za rezervi delovi koj ke se nao|a na potegot pome|u magacinot i septi~kata jama.

Vo Tabelata 1 se prika`ani koli~inite na pomo{ni materijali za odr`uvawe na mehanizacijata.

### IV.2.3 Sredstva za higiena i za{tita pri rabota

Sredstvata za higiena kako i sredstvata za za{tita pri rabota se ~uvaat vo magacin za taa namena i se sostojat od sredstva za li~na higiena (detergenti i pasti za odmastuvawe) kako i za{titni rakavici, ~evli i za{titna obleka.

### IV.3 Energensi

### IV.3.2 Dizel gorivo

Dizel gorivoto se koristi za potrebite na mobilnata mehanizacija.  Dizel gorivoto se skladira vo buriwa i se koristi samo za dnevna potro{uva~ka. Pokazatelite na potro{uva~kata se prika`ani vo Tabelata 1

### IV.3.3 Elektri~na energija

Napojuvaweto se elektri~na energija e preku sopstvena trafostanica so snaga od 650 kW. Dovodniot kabel e podzemen a priklu~okot e izveden vo soglasnost so barawata. Prose~nata mese~nata potro{uva~kata na struja e  15 MW/h a anga`iranata mo}nost na elektri~na energija se kontrolira preku vgraden maksigraf.

### IV.4 Voda

Tehnolo{ka i sanitarna voda na lokacijata se koristi od rezervoari za voda od 2x5000litri, postaveni vedna{ do primarnata drobilka i istite se  dopolnuvaat so cisterni od op{tinskata vodovodna mre`a.

Srednata godi{na potro{uva~ka na sanitarna i tehnolo{ka voda iznesuva 320 m3.

Voda se koristi samo za perewe na objektot i okolinata i poprskuvawe na granulatite zaradi prevencija od sozdavawe pra{ina . Otpadnata sanitarna voda se ispu{ta vo sopstvena septi~ka jama za taa namena.

Vo Tabelata 1 se prika`ani koli~inite na pomo{ni materijali za odr`uvawe na mehanizacijata.

Tabela 1: Potro{en materijal za odr`uvawe na mobilnata mehanizacija

| Red. broj | Potro{en materijal | Edin. mera | Dup~ewe i minirawe | BGH-1000 C | KRAZ-6504 | TG-220 | Vkupno |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | Gorivo | L/t |  | 0.7 | 0.29 | 0.84 | 1.83 |
| 2. | M.maslo | L/t |  | 0.035 | 0.0145 | 0.042 | 0.0915 |
| 3. | Difer. Maslo | L/t |  | 0.021 | 0.0087 | 0.025 | 0.0547 |
| 4. | Tov.mast | Kg/t |  | 0.014 | 0.0056 | 0.017 | 0.0366 |
| 5. | Hidr.maslo | L/t |  | 0.07 | 0.029 | 0.054 | 0.183 |
| 6. | Gumi | Par/t |  | / | / | 1.4 h 10-4 | 1.4 h 10-4 |
| 7. | Eksploziv | Kg/t | 0.15 | / | / | / | 0.15 |
| 8. | Detonatorski kapisli DK-8 | Par/t | 24h10-5 | / | / | / | 24h10-5 |
| 9. | Detonatorski fitil | m/t | 0.06 | / | / | / | 0.06 |
| 10. | Milisekundni zabavuva~i | Par/t | 0.003 | / | / | / | 0.003 |
| 11. | Kruni | Par/t | 3.8h10-5 | / | / | / | 3.8h10-5 |
| 12. | Cevki | Par/t | 1.9h10-5 | / | / | / | 1.9h10-5 |

### IV.5 Proizvodi

- Droben kamenit (0 – 63 mm)

- Frakcionen droben agregat

  - 0 – 4 mm

  - 4 – 8 mm

  - 8 – 16 mm

  - 16 – 32 mm

Granulatot se koristi kako finalen proizvod namenet za izrabotka na beton, armiran beton i tamponski sloj za posipuvawe na pati{ta.

Od godi{noto proizvodstvo na droben kamenit od 100.000 toni proizvodite u~estvuvaat so:

- Droben kamenit (0 – 63 mm) so 10 %

- Frakcionen agregat (0 – 4 mm) so 35 %

- Frakcionen agregat (4 – 8 mm) so 20 %

- Frakcionen agregat (8 – 16 mm) so 20 %

- Frakcionen agregat (16 – 32 mm) so 15 %

Gotovite proizvodi se skladiraat na plac za gotovi proizvodi

Skladiraweto na proizvodite i poluproizvodite kako i pomo{nite materijali se obele`ani na mapa  vo Prilog IV.6.3

### IV.6 Prilozi

### IV.6.1 Potvrda za rakuvawe so dizel gorivo pri polnewe na cisterni

### IV.6.2 Karakteristiki na surovini/proizvodi

### IV.6.2.1 Izve{taj od ispituvawe na fizi~ko hemiski karakteristiki na siv varovnik

### IV.6.2.2 Fizi~ko hemiski karakteristiki na supstanciite sodr`ani vo eksplozivot

ETHYLENE GLYCOL DINITRATE 1056

October 1999

CAS No: 628-96-6

RTECS No: KW5600000

EC No: 603-032-00-9

Glycol dinitrate

EGDN

Nitroglycol

C2H4N2O6 / NO2-OCH2CH2O-NO2

Molecular mass: 152.1

TYPES OF

HAZARD/

EXPOSURE

ACUTE HAZARDS/SYMPTOMS PREVENTION FIRST AID/FIRE FIGHTING

FIRE Explosive. Gives off irritating or toxic fumes (or gases) in a fire.

NO open flames, NO sparks, and

NO smoking.

Powder, water spray, foam, carbon dioxide, Note: Evacuate area, fight fires only from an explosion-resistant location.

EXPLOSION Risk of fire and explosion. Prevent build-up of electrostatic charges (e.g., by grounding). Use non-sparking handtools. Do NOT expose to friction or shock. In case of fire: cool drums, etc., by spraying with water but avoid contact of the substance with water. Combat fire from a sheltered position.

EXPOSURE STRICT HYGIENE!

Inhalation Headache. Dizziness. Nausea. Weakness. Flushing of the face. Chest pain. Symptoms may be delayed (see Notes). Ventilation, local exhaust, or breathing protection. Fresh air, rest. Refer for medical attention. Skin MAY BE ABSORBED! (See Inhalation). Protective gloves. Protective clothing. Remove contaminated clothes. Rinse and then wash skin with water and soap. Refer for medical attention. Eyes Face shield, or eye protection in combination with breathing protection. First rinse with plenty of water for several minutes (remove contact lenses if easily possible), then take to a doctor. Ingestion (See Inhalation). Do not eat, drink, or smoke during work. Wash hands before eating. Rinse mouth. Induce vomiting (ONLY IN CONSCIOUS PERSONS!). Refer for medical attention.

SPILLAGE DISPOSAL PACKAGING & LABELLING

Evacuate danger area! Consult an expert! Collect leaking and spilled liquid in sealable containers as far as possible. Absorb remaining liquid in sand or inert absorbent and remove to safe place. Do NOT let this chemical enter the environment. (Extra personal protection: complete protective clothing including self-contained breathing apparatus).

E Symbol

T+ Symbol

R: 2-26/27/28-33

S: (1/2)-33-35-36/37-45

Do not transport with food and feedstuffs.

EMERGENCY RESPONSE STORAGE

Fireproof. Store in a separate building. Separated from acids, food and feedstuffs. Cool. Well closed.

NITROGLYCERIN 0186

October 2005

CAS No: 55-63-0

RTECS No: QX2100000

UN No: 0143 (Desensitized)

EC No: 603-034-00-X

Glyceryl trinitrate

Glycerol trinitrate

1,2,3-Propanetriol trinitrate

Blasting oil

C3H5N3O9 / C3H5(NO3)3

Molecular mass: 227.1

TYPES OF

HAZARD/

EXPOSURE

ACUTE HAZARDS/SYMPTOMS PREVENTION FIRST AID/FIRE FIGHTING FIRE

Explosive. Many reactions may cause fire or explosion. Gives off irritating or toxic fumes (or gases) in a fire. NO open flames, NO sparks, and NO smoking. NO contact with hot surfaces. Powder, water spray, foam, carbon dioxide.

EXPLOSION

Risk of fire and explosion. Closed system, ventilation, explosion-proof electrical equipment and lighting. Use non-sparking handtools. Do NOT expose to friction or shock. In case of fire: keep drums, etc., cool by spraying with water. Combat fire from a sheltered position.

EXPOSURE STRICT HYGIENE! IN ALL CASES CONSULT A DOCTOR!

Inhalation Headache. Flushing of the face. Dizziness. Ventilation, local exhaust, or breathing protection. Fresh air, rest. Refer for medical attention. Skin MAY BE ABSORBED! (Further see Inhalation). Protective gloves. Protective clothing. Remove contaminated clothes. Rinse and then wash skin with water and soap. Refer for medical attention. Eyes Redness. Pain. Face shield or eye protection in combination with breathing protection. First rinse with plenty of water for several minutes (remove contact lenses if easily possible), then take to a doctor. Ingestion Flushing of the face. Headache. Dizziness. Nausea. Vomiting. Shock or collapse. Do not eat, drink, or smoke during work. Wash hands before eating. Rinse mouth. Induce vomiting (ONLY IN CONSCIOUS PERSONS!). Give a slurry of activated charcoal in water to drink. Refer for medical attention immediately.

SPILLAGE DISPOSAL PACKAGING & LABELLING

Evacuate danger area! Consult an expert! Remove all ignition sources. Personal protection: complete protective clothing including self-contained breathing apparatus. Do NOT wash away into sewer. Do NOT let this chemical enter the environment. Collect leaking and spilled liquid in sealable containers as far as possible. Absorb remaining liquid in sand or inert absorbent and remove to safe place.

E Symbol

T+ Symbol

N Symbol

R: 3-26/27/28-33-51/53

S: (1/2-)33-35-36/37-45-61

UN Hazard Class: 1.1D

(Desensitized)

UN Subsidiary Risks: 6.1

(Desensitized)

Do not transport with food and feedstuffs.

EMERGENCY RESPONSE SAFE STORAGE

Transport Emergency Card: TEC (R)-10G1.1

NFPA Code: H2; F3; R4

Store only if stabilized. Store in an area without drain or sewer access. Fireproof. Well closed. Separated from food and feedstuffs.

AMMONIUM NITRATE 0216

March 2001

CAS No: 6484-52-2

RTECS No: BR9050000

UN No: 1942

Nitric acid, ammonium salt

NH4NO3

Molecular mass: 80.1

TYPES OF

HAZARD/

EXPOSURE

ACUTE HAZARDS/SYMPTOMS PREVENTION FIRST AID/FIRE FIGHTING FIRE

Not combustible but enhances combustion of other substances. Explosive. Gives off irritating or toxic fumes (or gases) in a fire. NO contact with combustibles or reducing agents. Water in large amounts. NO other extinguishing agents. In case of fire in the surroundings: use flooding amounts of water in the early stages.

EXPLOSION

Risk of fire and explosion under confinement and high temperatures. Evacuate danger area! In case of fire: keep drums, etc., cool by spraying with water. Combat fire from a sheltered position.

EXPOSURE PREVENT DISPERSION OF DUST!

Inhalation Cough. Headache. Sore throat. See Ingestion. Local exhaust or breathing protection. Fresh air, rest. Artificial respiration may be needed. Refer for medical attention. Skin Redness. Protective gloves. First rinse with plenty of water, then remove contaminated clothes and rinse again. Refer for medical attention. Eyes Redness. Pain. Safety goggles. First rinse with plenty of water for several minutes (remove contact lenses if easily possible), then take to a doctor. Ingestion Abdominal pain. Blue lips or fingernails. Blue skin. Convulsions. Diarrhoea. Dizziness. Vomiting. Weakness. Do not eat, drink, or smoke during work. Rinse mouth. Refer for medical attention.

SPILLAGE DISPOSAL PACKAGING & LABELLING

Evacuate danger area! Consult an expert! Sweep spilled substance into non-combustible containers. Wash away remainder with plenty of water.

UN Hazard Class: 5.1

UN Pack Group: III

EMERGENCY RESPONSE SAFE STORAGE

Transport Emergency Card: TEC (R)-51S1942 or

51GO2-I+II+III

NFPA Code: H 2; F 0; R 3; OX

Provision to contain effluent from fire extinguishing. Separated from combustible and reducing substances. Dry.

2,4-DINITROTOLUENE 0727

April 2005

CAS No: 121-14-2

RTECS No: XT1575000

UN No: 3454

EC No: 609-007-00-9

1-Methyl-2,4-dinitrobenzene

2,4-DNT

C7H6N2O4 / C6H3CH3(NO2)2

Molecular mass: 182.1

TYPES OF HAZARD/ EXPOSURE

ACUTE HAZARDS/SYMPTOMS PREVENTION FIRST AID/FIRE FIGHTING

FIRE Combustible. Gives off irritating or toxic fumes (or gases) in a fire. NO open flames. Powder, water spray, foam, carbon dioxide.

EXPLOSION

Finely dispersed particles form explosive mixtures in air. Risk of explosion on contact with many substances. Prevent deposition of dust; closed system, dust explosion-proof electrical equipment and lighting. In case of fire: keep drums, etc., cool by spraying with water. Combat fire from a sheltered position.

EXPOSURE PREVENT DISPERSION OF DUST! STRICT HYGIENE!

Inhalation Blue lips or finger nails. Blue skin. Headache. Dizziness. Nausea. Confusion. Convulsions. Unconsciousness. Local exhaust or breathing protection. Fresh air, rest. Artificial respiration may be needed. Refer for medical attention. Skin MAY BE ABSORBED! (See Inhalation). Protective gloves. Protective clothing. Remove contaminated clothes. Rinse and then wash skin with water and soap. Refer for medical attention. Eyes Safety goggles. First rinse with plenty of water for several minutes (remove contact

lenses if easily possible), then take to a doctor. Ingestion (See Inhalation). Do not eat, drink, or smoke during work. Wash hands before eating. Rinse mouth. Give plenty of water to drink. Refer for medical attention.

SPILLAGE DISPOSAL PACKAGING & LABELLING

Consult an expert! Personal protection: chemical protection suit including self-contained breathing apparatus. Do NOT let this chemical enter the environment. Sweep spilled substance into containers; if appropriate, moisten first to prevent dusting. Carefully collect remainder, then remove to safe place.

T Symbol

N Symbol

R: 45-23/24/25-48/22-62-68-51/53

S: 53-45-61

Note: E

UN Hazard Class: 6.1

UN Pack Group: II

Do not transport with food and

feedstuffs.

EMERGENCY RESPONSE SAFE STORAGE

Transport Emergency Card: TEC (R)-61S3454; 61GT2-II NFPA Code: H3; F1; R3 Fireproof. Separated from strong bases, food and feedstuffs oxidants, strong reducing agents. Well closed. Keep in a well-ventilated room. Store in an area without drain or sewer access.

2,3-DINITROTOLUENE 0726

April 2005

CAS No: 602-01-7

RTECS No: XT1400000

UN No: 3454

EC No: 609-050-00-3

1-Methyl-2,3-dinitrobenzene 2,3-DNT C6H3CH3(NO2)2 / C7H6N2O4

Molecular mass: 182.1

TYPES OF HAZARD/ EXPOSURE

ACUTE HAZARDS/SYMPTOMS PREVENTION FIRST AID/FIRE FIGHTING

FIRE Combustible. Gives off irritating or toxic fumes (or gases) in a fire. NO open flames. Powder, water spray, foam, carbon dioxide.

EXPLOSION

Finely dispersed particles form explosive mixtures in air. Risk of explosion on contact with many substances.

Prevent deposition of dust; closed system, dust explosion-proof electrical equipment and lighting. In case of fire: keep drums, etc., cool by spraying with water. Combat fire from a sheltered position.

EXPOSURE PREVENT DISPERSION OF DUST!

Inhalation Blue lips or finger nails. Blue skin. Headache. Dizziness. Nausea. Confusion. Convulsions. Unconsciousness. Local exhaust or breathing protection. Fresh air, rest. Artificial respiration may be needed. Refer for medical attention. Skin MAY BE ABSORBED! Redness. (Further see Inhalation). Protective gloves. Remove contaminated clothes. Rinse and then wash skin with water and soap. Refer for medical attention. Eyes Safety goggles. First rinse with plenty of water for several minutes (remove contact lenses if easily possible), then take to a doctor. Ingestion (Further see Inhalation). Do not eat, drink, or smoke during work. Wash hands before eating. Rinse mouth. Give plenty of water to drink. Refer for medical attention.

SPILLAGE DISPOSAL PACKAGING & LABELLING

Consult an expert! Personal protection: chemical protection suit including self-contained breathing apparatus. Do NOT let this chemical enter the environment. Sweep spilled substance into containers; if appropriate, moisten first to prevent dusting. Carefully collect remainder, then remove to safe place.

T Symbol

N Symbol

R: 45-23/24/25-48/22-62-68-50/53

S: 53-45-60-61

Note: E

UN Hazard Class: 6.1

UN Pack Group: II

Do not transport with food and feedstuffs.

EMERGENCY RESPONSE SAFE STORAGE

Transport Emergency Card: TEC (R)-61S3454; 61GT2-II NFPA Code: H3; F1; R3

Fireproof. Separated from strong bases, food and feedstuffs oxidants, strong reducing agents. Well closed. Keep in a well-ventilated room. Store in an area without drain or sewer access.

2,4,6-TRINITROTOLUENE 0967

April 2000

CAS No: 118-96-7

RTECS No: XU0175000

UN No: 0209

EC No: 609-008-00-4

2-Methyl-1,3,5-trinitrobenzene

1-Methyl-2,4,6-trinitrobenzene

TNT

C7H5N3O6 / C6H2(CH3)(NO2)3

Molecular mass: 227.1

TYPES OF HAZARD/ EXPOSURE

ACUTE HAZARDS/SYMPTOMS PREVENTION FIRST AID/FIRE FIGHTING FIRE  Explosive. Many reactions may cause fire or explosion. NO open flames, NO sparks, and NO smoking. Water in large amounts. Do not attempt to extinguish large fire, evacuate area.

EXPLOSION

Risk of fire and explosion upon rapid heating or strong shock. Do NOT expose to friction or shock. Do not expose to heat and keep wet with at least 30% water. In case of fire: keep drums, etc., cool by spraying with water. Combat fire from a sheltered position.

EXPOSURE PREVENT DISPERSION OF DUST!

STRICT HYGIENE!

IN ALL CASES CONSULT A DOCTOR!

Inhalation Headache. Blue lips or finger nails. Blue skin. Cough. Sore throat. Laboured breathing. Vomiting. Abdominal cramps. Unconsciousness. Symptoms may be delayed (see Notes). Local exhaust or breathing protection. Fresh air, rest. Artificial respiration may be needed. Refer for medical attention. Skin MAY BE ABSORBED! Redness. Pain. Yellowish staining. (Further see Inhalation). Protective gloves. Protective clothing. Remove contaminated clothes. Rinse and then wash skin with water and soap. Refer for medical attention. Wear protective gloves when administering first aid. Eyes Redness. Pain. Face shield or eye protection in combination with breathing protection. First rinse with plenty of water for several minutes (remove contact lenses if easily possible), then take to a doctor. Ingestion (Further see Inhalation). Do not eat, drink, or smoke during work. Wash hands before eating. Rinse mouth. Induce vomiting

(ONLY IN CONSCIOUS PERSONS!). Wear protective gloves when inducing vomiting.

SPILLAGE DISPOSAL PACKAGING & LABELLING

Evacuate danger area! Consult an expert! Wet spilled material before picking it up, do not attempt to sweep up dry material. Do NOT wash away into sewer. Carefully collect remainder, then remove to safe place. Do NOT let this chemical enter the environment. Chemical protection suit including self-contained breathing apparatus.

E Symbol

T Symbol

N Symbol

R: 2-23/24/25-33-51/53

S: (1/2-)35-45-61

UN Hazard Class: 1.1D

Unbreakable packaging; put breakable packaging into closed unbreakable container. Do not transport with food and feedstuffs.

EMERGENCY RESPONSE SAFE STORAGE

Transport Emergency Card: TEC (R)-10G1.1

NFPA Code: H 2; F 4; R 4

Fireproof. Separated from initiator explosives, food and feedstuffs, incompatible materials. See Chemical Dangers. Well closed.

2,5-DINITROTOLUENE 1591

June 2006

CAS No: 619-15-8

RTECS No: XT1750000

UN No: 3454

EC No: 609-055-00-0

2-Methyl-1,4-dinitrobenzene 2,5-DNT Toluene, 2,5-dinitro- C7H6N2O4

Molecular mass: 182.14

TYPES OF HAZARD/ EXPOSURE

ACUTE HAZARDS/SYMPTOMS PREVENTION FIRST AID/FIRE FIGHTING

FIRE Combustible. Gives off irritating or toxic fumes (or gases) in a fire. NO open flames. Powder, water spray, foam, carbon dioxide.

EXPLOSION

Finely dispersed particles form explosive mixtures in air. Prevent deposition of dust; closed system, dust explosion-proof electrical equipment and lighting. In case of fire: keep drums, etc., cool by spraying with water. Combat fire from a sheltered position.

EXPOSURE AVOID ALL CONTACT!

Inhalation Cough. Sore throat. Blue lips or finger nails. Local exhaust or breathing protection. Fresh air, rest. Refer for medical attention. Skin MAY BE ABSORBED! Redness. Pain. Protective clothing. Protective gloves. Remove contaminated clothes. Rinse and then wash skin with water and soap. Refer for medical attention. Eyes Redness. Pain. Safety goggles. First rinse with plenty of water (remove contact lenses if easily possible). Ingestion Blue lips or fingernails. Blue skin. Dizziness. Headache. Nausea. Confusion. Convulsions. Unconsciousness. Do not eat, drink, or smoke during work. Rinse mouth. Give a slurry of activated charcoal in water to drink. Refer for medical attention.

SPILLAGE DISPOSAL PACKAGING & LABELLING

Consult an expert! Personal protection: Chemical protection suit including self-contained breathing apparatus. Sweep spilled substance into sealable containers. Carefully collect remainder, then remove to safe place. Do NOT let this chemical enter the environment.

EU classification

T Symbol

N Symbol

R: 45-23/24/25-48/22-62-68-51/53

S: 53-45-61

UN classification

UN Hazard Class: 6.1

UN Pack Group: II

Unbreakable packaging; put breakable packaging into closed unbreakable container. Severe marine pollutant. Do not transport with food and feedstuffs.

EMERGENCY RESPONSE SAFE STORAGE

Transport Emergency Card: TEC (R)-61GT2-II

Transport Emergency Card: TEC (R)-61S3454

NFPA Code: H3; F1; R3

Fireproof. Separated from strong oxidants, food and feedstuffs, and incompatible materials. See Chemical Dangers. Well closed. Keep in a well-ventilated room. Store in an area without drain or sewer access. Provision to contain effluent from fire extinguishing.

2,6-DINITROTOLUENE 0728

April 2005

CAS No: 606-20-2

RTECS No: XT1925000

UN No: 3454

EC No: 609-049-00-8

1-Methyl-2,6-dinitrobenzene

2,6-DNT

C7H6N2O4 / C6H3CH3(NO2)2

Molecular mass: 182.1

TYPES OF HAZARD/ EXPOSURE

ACUTE HAZARDS/SYMPTOMS PREVENTION FIRST AID/FIRE FIGHTING FIRE

Combustible. Gives off irritating or toxic fumes (or gases) in a fire. NO open flames. Powder, water spray, foam, carbon dioxide.

EXPLOSION

Finely dispersed particles form explosive mixtures in air. Risk of explosion on contact with many substances.

Prevent deposition of dust; closed system, dust explosion-proof electrical equipment and lighting. In case of fire: keep drums, etc., cool by spraying with water. Combat fire from a sheltered position.

EXPOSURE PREVENT DISPERSION OF DUST! AVOID ALL CONTACT! AVOID EXPOSURE OF (PREGNANT) WOMEN!

Inhalation Blue lips or finger nails. Blue skin. Headache. Dizziness. Nausea. Confusion. Convulsions. Unconsciousness. Local exhaust or breathing protection. Fresh air, rest. Artificial respiration may be needed. Refer for medical attention. Skin MAY BE ABSORBED! (See Inhalation). Protective gloves. Protective clothing. Remove contaminated clothes. Rinse and then wash skin with water and soap. Refer for medical attention. Eyes Face shield. First rinse with plenty of water for several minutes (remove contact lenses if easily possible), then take to a doctor. Ingestion (See Inhalation). Do not eat, drink, or smoke during work. Wash hands before eating. Rinse mouth. Give plenty of water to drink. Refer for medical attention.

SPILLAGE DISPOSAL PACKAGING & LABELLING

Consult an expert! Personal protection: chemical protection suit including self-contained breathing apparatus. Sweep spilled substance into containers; if appropriate, moisten first to prevent dusting. Carefully collect remainder, then remove to safe place.

T Symbol

R: 45-23/24/25-48/22-62-68-52/53

S: 53-45-61

Note: E

UN Hazard Class: 6.1

UN Pack Group: II

Do not transport with food and feedstuffs.

EMERGENCY RESPONSE SAFE STORAGE

Transport Emergency Card: TEC (R)-61S3454; 61GT2-II

NFPA Code: H3; F1; R3

Fireproof. Separated from strong bases, food and feedstuffs oxidants, strong reducing agents. Well closed. Keep in a well-ventilated room.

3,4-DINITROTOLUENE 0729

April 2005

CAS No: 610-39-9

RTECS No: XT2100000

UN No: 3454

EC No: 609-051-00-9

1-Methyl-3,4-dinitrobenzene

3,4-DNT C7H6N2O4 / C6H3CH3(NO2)2

Molecular mass: 182.1

TYPES OF HAZARD/ EXPOSURE

ACUTE HAZARDS/SYMPTOMS PREVENTION FIRST AID/FIRE FIGHTING FIRE

Combustible. Gives off irritating or toxic fumes (or gases) in a fire. NO open flames. Powder, water spray, foam, carbon dioxide.

EXPLOSION

Finely dispersed particles form explosive mixtures in air. Risk of explosion on contact with many substances.

Prevent deposition of dust; closed system, dust explosion-proof electrical equipment and lighting. In case of fire: keep drums, etc., cool by spraying with water. Combat fire from a sheltered position.

EXPOSURE PREVENT DISPERSION OF DUST!

Inhalation Blue lips or finger nails. Blue skin. Headache. Dizziness. Nausea. Confusion. Convulsions. Unconsciousness. Local exhaust or breathing protection. Fresh air, rest. Artificial respiration may be needed. Refer for medical attention. Skin MAY BE ABSORBED! Redness. (Further see Inhalation). Protective gloves. Remove contaminated clothes. Rinse and then wash skin with water and soap. Refer for medical attention. Eyes Safety goggles. First rinse with plenty of water for several minutes (remove contact lenses if easily possible), then take to a doctor. Ingestion (Further see Inhalation). Do not eat, drink, or smoke during work. Rinse mouth. Give plenty of water to drink. Refer for medical attention.

SPILLAGE DISPOSAL PACKAGING & LABELLING

Consult an expert! Personal protection: chemical protection suit including self-contained breathing apparatus. Do NOT let this chemical enter the environment. Sweep spilled substance into containers; if appropriate, moisten first to prevent dusting. Carefully collect remainder, then remove to safe place.

T Symbol

N Symbol

R: 45-23/24/25-48/22-62-68-51/53

S: 53-45-61

Note: E

UN Hazard Class: 6.1

UN Pack Group: II

Do not transport with food and feedstuffs.

EMERGENCY RESPONSE SAFE STORAGE

Transport Emergency Card: TEC (R)-61S3454; 61GT2-II

NFPA Code: H3; F1; R3

Fireproof. Separated from strong bases, food and feedstuffs oxidants, strong reducing agents. Well closed. Keep in a well-ventilated room. Store in an area without drain or sewer access.

### IV.6.3 Safety Data sheets

### IV.6.3.1 Kalcium karbonat

Safety (MSDS) data for calcium carbonate

Safety data for calcium carbonate

Click here for data on calcium carbonate in student-friendly format, from the

HSci project

Glossary of terms on this data sheet.

The information on this web page is provided to help you to work safely, but it

is intended to be an overview of hazards, not a replacement for a full Material

Safety Data Sheet (MSDS). MSDS forms can be downloaded from the web sites of

many chemical suppliers.

General

Synonyms: limestone, marble, calcite, chalk, carbonic acid calcium salt,

blackboard chalk

Molecular formula: CaCO3

CAS No: 471-34-1

EINECS No: 207-439-9

Physical data

Appearance: white or colourless crystals or white powder or chunks

Melting point: 825 C

Boiling point:

Vapour density:

Vapour pressure:

Density (g cm-3): 2.83

Flash point:

Explosion limits:

Autoignition temperature:

Water solubility: negligible

Stability

Stable. Incompatible with acids, fluorine, ammonium salts, alum.

Toxicology

Dust may cause irritation. Typical TLV/TWA 10 mg m-3.

Risk phrases

R36 R37 R38.

Transport information

Non-hazardous for air, sea and road freight.

Personal protection

Minimise exposure to dust.

Safety phrases

S26 S36.

### IV.6.3.2 Silicium Dioksid

Safety data for silicon dioxide

General

Synonyms: silica, amorphous silica, (sand is predominantly silicon dioxide),

accusand, amorphous quartz, borsil P, cryptocrystalline quartz, Denka F 90,

Denka FB 44, fused quartz, fused silica, fuselex, microcrystalline quartz,

nalcast, optocil, quartz glass, quartz sand, rancosil, siltex, spectrosil,

suprasil, vitreosil, vitreous quartz, vitreous silica, vitrified silica

Use: Used as a suspending and thickening agent in pharmaceutical formulations;

intermediate for glasses, component in abrasives, enamels.

Molecular formula: SiO2

CAS No: 7631-86-9

EC No: 262-373-8

Physical data

Appearance: white crystals or powder

Melting point:

Boiling point:

Vapour density:

Vapour pressure: zero

Specific gravity:

Flash point: n/a

Explosion limits: n/a

Autoignition temperature: n/a

Stability

Stable.

Toxicology

IARC evaluation: dust is probably carcinogenic to humans. Dust can cause

silicosis. Typical 8hr TWA 0.3mg/m3 total dust. In bulk form, not hazardous

according to Directive 67/548/EEC.

Toxicity data

IPN-RAT LD50 50 mg kg-1

IVN-RAT LD50 15mg kg-1

Risk phrases

(The meaning of any risk phrases which appear in this section is given here.)

R20.

Transport information

Non-hazardous for air, sea and road freight.

Personal protection

Safety glasses. Good ventilation if exposed to dust. Normal exposure to bulk

silicon dioxide as sand is not considered likely to be harmful.

### IV.6.3.3 Dizel ekstra lesno gorivo

Safety data for diesel

General

Synonyms: diesel fuel, diesel oil

Molecular formula: depends upon formulation, typically composed of a

hydrocarbon mix together with (often proprietary) additives. May contain a dye

to indicate, for example, whether or not excise duty has been paid on the

product.

CAS No: 68334-30-5

EC No:

Physical data

Appearance: clear colourless or dyed liquid

Melting point:

Boiling point: typically > 149 C

Vapour density:

Vapour pressure: at 20 C typically < 1 mm

Specific gravity:

Flash point: typically > 52 C

Explosion limits:

Autoignition temperature:

Stability

Stable. Flammable. Incompatible with strong acids, strong oxidizing agents,

halogens.

Toxicology

Respiratory and skin irritant. The product may contain polycyclic aromatic

hydrocarbons which may be carcinogenic. Generally regarded as being of low

toxicity unless contact is repeated and/or prolonged.

Toxicity data

ORL-RAT LD50 >2000 mg kg-1

Risk phrases

R10.

Personal protection

Avoid skin contact and inhalation. Ensure good ventilation.

### IV.6.3.4 Glicerol - maziva

Safety data for glycerol

General

Synonyms: glycerin, glycerol USP, glycerine, 1,2,3-propanetriol, propanetriol,

1,2,3-trihydroxypropane, bulbold, citifluor AF 2, cristal, emergy 916, glyrol,

glycerol opthalgan, glyciterol, glycyl alcohol, osmoglyn, pricerine 9091

Use: Widely used as a food additive (emulsifier, thickener, stabilizer),

cosmetic agent, lubricating agent, antifreeze etc.

Molecular formula: C3H8O3 [structural: CH2OHCHOHCH2OH]

CAS No: 56-81-5

EC No: 200-289-5

Physical data

Appearance: viscous colourless or pale yellow liquid

Melting point: 17.8 C

Boiling point: 290 C

Vapour density: 3.17 g/l

Vapour pressure: < 1mm Hg at 20 C

Specific gravity: 1.261

Flash point: 160 C (closed cup)

Explosion limits: lower 0.9%

Autoignition temperature: 370 C

Critical temperature: 492.2 C

Critical pressure: 42.5 atm

Stability

Stable. Incompatible with perchloric acid, lead oxide, acetic anhydride,

nitrobenzene, chlorine, peroxides, strong acids, strong bases. Combustible.

Toxicology

Mist is a respiratory irritant at high concentrations. Repeated contact may

cause dehydration of skin. Typical TLV 10 mg/m3 (nuisance). Not hazardous

according to directive 67/548/EC.

Toxicity data

IPR-RAT LD50 8700 mg kg-1

ORL-RAT LD50 12600 mg kg-1

SCU-RAT LD50 100 mg kg-1

ORL-MUS LD50 8700 mg kg-1

Risk phrases

Personal protection

Minimize contact.

Safety phrases

S26 S36.

Safety (MSDS) data for glycerol-D8

General

Synonyms:

Use:

Molecular formula: C3D8O3

CAS No: 7325-17-9

EINECS No:

Physical data

Appearance: colourless viscous liquid

Melting point: 20 C

Boiling point: 182 C at 20 mm Hg

Vapour density:

Vapour pressure:

Density (g cm-3): 1.37

Flash point: 113 C (closed cup)

Explosion limits: 0.9% (lower)

Autoignition temperature: 370 C

Water solubility: complete

Stability

Stable, but moisture sensitive. Incompatible with strong bases, strong

oxidizing agents.

Toxicology

Not hazardous according to Directive 67/548/EEC.

Toxicity data

(The meaning of any toxicological abbreviations which appear in this section

is given here.)

Risk phrases

Transport information

(The meaning of any UN hazard codes which appear in this section is given

here.)

Non-hazardous for air, sea and road freight.

Personal protection

Minimize exposure.

Safety phrases

### IV.6.3.5 Trinitrotoluol

Safety data for 2,4,6-trinitrotoluene

General

Synonyms: 2-methyl-1,3,5-trinitrobenzene, TNT, alpha-trinitrotoluol, tolite, tritol, entsufon, trotyl 
Use: 
Molecular formula: C6H2(CH3)(NO2)3 
CAS No: 118-96-7 
EC Index No: 609-008-00-4

Physical data

Appearance: yellow crystals 
Melting point: 80 C 
Boiling point: 240 C (decomposes explosively) 
Vapour density: 
Vapour pressure: 
Density (g cm-3): 1.65 
Flash point: 
Explosion limits: 
Autoignition temperature: 
Water solubility: insoluble

Stability

Unstable. Risk of explosion if heated or struck. Reacts violently - potentially explosively - with reducing agents. Reacts with heavy metals.

Toxicology

Harmful if inhaled or swallowed and in contact with the skin. Readily absorbed through the skin. Irritant.

Toxicity data 
(The meaning of any abbreviations which appear in this section is given here.)

Risk phrases 
(The meaning of any risk phrases which appear in this section is given here.) 
R2 R23 R24 R25 R33.

Transport information

(The meaning of any UN hazard codes which appear in this section is given here.) 
Hazard class 1.1D.

Personal protection

Safety glasses, adequate ventilation. Suitable protection against explosion. Ensure a full risk assessment is prepared before work starts.

Safety phrases 
(The meaning of any safety phrases which appear in this section is given here.) 
S35 S44.

### IV.6.3.6 Dinitrotoluol

Safety data for 2,4-dinitrotoluene

General

Synonyms: 1-methyl-2,4-dinitrobenzene 
Molecular formula: C7H6N2O4 
CAS No: 121-14-2 
EC No: 204-450-0

Physical data

Appearance: solid 
Melting point: 67 - 70 C 
Boiling point: 
Vapour density: 
Vapour pressure: 1 mm Hg at 103C 
Specific gravity: 
Flash point: 
Explosion limits: 
Autoignition temperature:

Stability

Stable. Incompatible with oxidizing agents, reducing agents, strong bases.

Toxicology

Highly toxic. Possible carcinogen. Reproductive hazard. Danger of cumulative effects. May cause sensitization by inhalation or skin contact.

Toxicity data 
(The meaning of any toxicological abbreviations which appear in this section is given here.) 
ORL-RAT LD50 268 mg kg-1

Risk phrases 
(The meaning of any risk phrases which appear in this section is given here.) 
R23 R24 R25 R33 R42 R43 R45.

Personal protection

Safety glasses, gloves, work in a fume cupboard. Handle as a possible carcinogen.

Safety phrases 
(The meaning of any safety phrases which appear in this section is given here.) 
S26 S27 S36 S37 S39 S45

Safety data for 2,4-dinitrotoluene

General

Synonyms: 1-methyl-2,4-dinitrobenzene 
Molecular formula: C7H6N2O4 
CAS No: 121-14-2 
EC No: 204-450-0

Physical data

Appearance: solid 
Melting point: 67 - 70 C 
Boiling point: 
Vapour density: 
Vapour pressure: 1 mm Hg at 103C 
Specific gravity: 
Flash point: 
Explosion limits: 
Autoignition temperature:

Stability

Stable. Incompatible with oxidizing agents, reducing agents, strong bases.

Toxicology

Highly toxic. Possible carcinogen. Reproductive hazard. Danger of cumulative effects. May cause sensitization by inhalation or skin contact.

Toxicity data 
(The meaning of any toxicological abbreviations which appear in this section is given here.) 
ORL-RAT LD50 268 mg kg-1

Risk phrases 
(The meaning of any risk phrases which appear in this section is given here.) 
R23 R24 R25 R33 R42 R43 R45.

Personal protection

Safety glasses, gloves, work in a fume cupboard. Handle as a possible carcinogen.

Safety phrases 
(The meaning of any safety phrases which appear in this section is given here.) 
S26 S27 S36 S37 S39 S45

Safety data for 2,6-dinitrotoluene

General

Synonyms: 2-methyl-1,3-dinitrobenzene 
Molecular formula: C7H6N2O4 
CAS No: 606-20-2 
EC No: 210-106-0

Physical data

Appearance: tan crystals 
Melting point: 64 - 66 C 
Boiling point: 
Vapour density: 
Vapour pressure: 
Specific gravity: 
Flash point: 
Explosion limits: 
Autoignition temperature:

Stability

Stable, but shock sensitive. Incompatible with oxidizing agents, reducing agents, strong bases. Heating may cause explosion.

Toxicology

Highly toxic. Possible carcinogen. Reproductive hazard. Neurological hazard. Danger of cumulative effects. May cause sensitization by inhalation or skin contact. Corrosive. Readily absorbed through skin.

Toxicity data 
(The meaning of any toxicological abbreviations which appear in this section is given here.) 
ORL-RAT LD50 177 mg kg-1

Risk phrases 
(The meaning of any risk phrases which appear in this section is given here.) 
R5 R22 R23 R24 R33 R42 R43 R45.

Personal protection

Safety glasses, gloves, fume cupboard.

Safety phrases 
(The meaning of any safety phrases which appear in this section is given here.) 
S26 S27 S36 S37 S39 S45

### IV.6.4 Dispozicija na objekti na lokacijata-sega{na sostojba