THE EUROPEAN UNION'S OPERATIONAL PROGRAMME FOR REGIONAL DEVELOPMENT

Preparation of project studies (FSs, EIAs, CBAs), design and tender documentation for establishing an integrated and financially self-sustainable waste management system in the east and northeast regions

EuropeAid/136070/IH/SER/MK

Contract number 12-9539/1

Annex 5

“ECOLOGICAL BASELINE SURVEY ”

CWMF Meckuevci-Arbasabci

NOVEMBER 2022

A project implemented by

in consortium with EPEM SA

This project is funded by

the European Union

| Project: | Preparing of project studies (FSs, EIAs, CBAs), design and tender documentation for establishing of an Integrated and Financially Self-sustainable Waste Management System in the East and Northeast Regions |
| --- | --- |
| Reference Number: | EuropeAid/136070/IH/SER/MK |
| Document: | Environmental Impact Assessment study Annex 5 - “Ecological Baseline Survey” Central Waste Management Facility (Meckuevci-Arbasanci) for the East and Northeast Region |
| Prepared by: | Marko Acevski, biodiversity expert and Emanuel Lisichanec, bird expert |
| Client: | Ministry of Finance - CFCD |
| Date: | November 2022 |

Contents

1.	INTRODUCTION	6

1.1 Background	6

1.2 Site Description	7

1.2.1 Site Location	7

1.2.2 Site ownership	7

1.2.3 Site Layout	8

1.2.4 Climate	8

1.2.5 Surface Waters	9

1.3 Main objectives	9

2.	METHODOLOGY	10

2.1 Literature preview (Desktop Study)	10

2.2 Ecological baseline survey	10

2.2.1 Duration and seasoning of the survey	11

2.2.2 Habitat Survey	12

2.2.3 Vegetation and Plant Species Survey	13

2.2.4 Mammal Survey	13

2.2.5 Bird Survey	14

2.3. Limitations of Survey	15

3. LEGISLATIVE, REGULATORY AND POLICY FRAMEWORK	17

4.	BASELINE	19

4.1 Designated Sites for Nature Conservation	19

4.1.1 Protected Areas and Proposed Protected Areas under Macedonian	19

legislation	19

4.1.2 Protected Areas under International legislation	23

4.2 Description of habitats	28

4.3 Valorisation	45

4.3.1	Habitats	45

4.3.2	Flora	46

4.3.3	Fauna	46

4.3.4 Ecosystem function of habitats	59

4.4	Significant habitat areas and affected species	60

5. Impact of CWMF on biodiversity	64

5.1 Overview of CWMF elements	64

5.2 Impact on flora and fauna	65

5.2.1 Construction phase	65

5.2.2 Operational phase (waste collection, transport, processing, landfilling)	67

5.2.3 Post-operational phase	73

6. Mitigation measures	74

6.1 Mitigation measures during construction phase	74

6.2 Mitigation measures during operational phase	79

7. Biodiversity Management and monitoring plan	92

7.1 Biodiversity Management consideration	92

7.2 Biodiversity Monitoring Plan	94

8. Disicussion	97

9. ANNEXES	97

References	121

Abbreviations & Acronyms:

| EBS BMP | Ecological Baseline Survey Biodiversity Management Plan |
| --- | --- |
| EIA | Environmental Impact Assessment |
| EMP | Environmental Management Plan |
| ESDD | Environmental & Social Due-Diligence |
| ESIA | Environmental and Social Impact Assessment |
| ESP | Environmental and Social Policy |
| IBA IPA PBA SPA KBA | Important Bird Area Important Plant Area Prime Butterfly Area Special Protected Areas Key Biodiversity Area |
| IUCN | International Union for the Conservation of Nature |
| MoEPP | Ministry of Environment and Physical Planning |
| NTS NNL NPI NPA | Non-Technical Summary No Net Loss Net Positive Impact Net Positive Approach |
| O.G. | Official Gazette |
| EIAO | Environmental Impact Assessment Ordinance |
| CWMF LWMF | Central Waste Management Facility Local Waste Management Facility |

# INTRODUCTION

## 1.1 Background

Environmental Impact Assessment (EIA) is a process by which the potential ecological impacts of a development proposal are assessed. As part of the EIA process it is usually necessary to undertake a combination of desktop and field based surveys in order to establish the baseline conditions of the natural environment in the vicinity of the proposed development. That process (or EIA step) is known as Ecological Baseline Survey (EBS).

This EBS is a part of the EIA study for construction of central landfill for Eastern and Northeastern region of Macedonia i.e. CWMF Meckuevci-Arbasanci. EBS is being as part of the Contract number 12-9539/1. According to the Environmental Impact Assessment Ordinance (EIAO) Guidance Note No. 7/2010, the objective of the baseline study of an ecological assessment is to provide adequate and accurate ecological baseline information. The ecological baseline survey forms an important part of the baseline study to:

- Provide first hand, specific and updated information on the existing ecological characters of the proposed landfill CWMF Meckuevci-Arbasanci and its vicinity;

- Verify information obtained from the review of existing literature; and

- Fill existing information gaps.

Before conducting the ecological baseline surveys, the key ecosystem components and processes as well as the target taxa groups of the study site Meckuevci-Arbasanci  that are considered relevant, important, valuable, susceptible and sensitive to changes or fundamental to the functioning of the ecosystem were selected as the focal points of the survey. The ecological survey was focused on the location of the proposed landfill and taking into consideration the review of the previous investigations in this region, the review of literature about the impact of landfill over wildlife; the fact that it is proposed for Important Bird Area (IBA) and EMERALD site Ovce Pole; and the season for conducting the survey, birds and mammals were chosen as a target taxa groups where impacts are likely to be significant.

This type of methodology is practicable and cost-effective for the baseline survey to provide exhaustive ecological information of the study site, as collection of a great deal of data with little focus does not facilitate subsequent ecological assessment.

This Report will be summarized for inclusion in the Assessment of the Environmental Effects for the project.

## Site Description

### 1.2.1 Site Location

Figure 1.  Site location

The proposed CWMF site is located in the eastern part of Sveti Nikole Municipality (fig.1), in the border area between villages Meckuevci and Arbasanci. The site is a vacant land, owned by the Municipality, and comprises two adjacent plots – one in Meckuevci (0.78 ha) and the other in Arbasanci (37.5 ha). The site is located in a hilly area (at the altitude about 610 m a.s.l.) above the Ovce Pole valley, 15 km north-east of the town of Sveti Nikole.  The road access is provided by a local asphalt road Sveti Nikole – Probistip.

### 1.2.2 Site ownership

The ownership of the proposed site is public currently occasionally used as pastures, like it is registered in the cadaster.

### 1.2.3 Site Layout

This site is a Greenfield location which lies between Meckuevci, which is considered the main location, and Arbasanci, which is considered as potential area for expansion. The wider area is hilly, with average altitude of 550-600 m.a.s.l (fig.2). The site and its wider surroundings are vacant pastures and have not been used for cultivating. No structures are present. The site and its surroundings comprise semi-natural habitats represented by steppe-like grasslands. Woodland is very scarce, represented by individual trees. Scattered shrubs are sparsely present mainly along the gullies. The landscape reminiscent of steppe, so here we find a number of steppe plants and steppe vegetation.Location is surrounded by low hills and valleys with degraded vegetation and the remains from coppices oak forest. The location and its surroundings represent intensive pastures of particular importance.

Figure 2. Site layout

### 1.2.4 Climate

The area of Ovce Pole has hot summers and moderate cold winters, with periodical temperature extremes. It is one of the most arid areas in the country with mean annual precipitation of 473.3 mm. Precipitation is unevenly distributed which causes long periods of drought. Mean annual temperature is 12.8°C with significant variations throughout the year. The coldest month is January (mean temperature is +1.4°C), the hottest is July (mean temperature is +23.7°C). The region is characterized by winds with average speed of 5.7 m/s and maximum speed up to 27 m/s. The average humidity is 67%. In summary, frequent winds, high temperatures and low air humidity, especially during the summer months, result in high evaporation which is the highest in the country, both from surface water and soil (average annual evaporation is 1246 l/m2 of water surface).

### 1.2.5 Surface Waters

The site is located within a hilly to low-mountainous relief with intermittent flows running along gullies towards the Ovce Pole valley. There are no significant surface water bodies on the site or its vicinity, only an intermittent stream named Klobuc, running along the eastern site area, towards south. This is one of several torrential streams running towards the Svetinikolska River (or Azmak) down in a valley. The Svetinikolska River is the right tributary of the Bregalnica River.

## Main objectives

The main objectives of the EBS are:

- Provide baseline information on the current habitats and ecological features both within the Site and in the immediately surrounding area;

- Identify the proximity of any designated sites for nature conservation interest and provide an assessment of any potential effects the proposed time extension may have on these;

- Identify the presence or potential presence of any protected species or habitats and provide an assessment of any potential effects the proposed time extension may have on these; and,

- Provide recommendations for further pre-construction checks and / or mitigation measures, if required, and provide an outline of proposed habitat enhancements, if applicable.

The EBS tasks are described below:

# METHODOLOGY

## 2.1 Literature preview (Desktop Study)

Data about previous ecological surveys in the wider region like Ovce Pole, Sveti Nikole, down part of river Bregalnica Watershed etc. were previewed. A desktop study was undertaken to identify any known existing features or species of ecological importance within the project area (as defined below). The desk study included a review of relevant policy and guidance and thought to identify any statutory designated sites for nature conservation. 3 km search radius was adopted for all statutory designated sites. The following data search parameters were used:

All statutory designated sites within a 3 km radius; and

Proposed IBA, IPA, PBA, EMERALD sites as future possible Special Protected Areas (SPAs) and Special Areas of Conservation (SACs) located within a 10 km radius.

## 2.2 Ecological baseline survey

Taking into account the findings of literature review and previous ecological investigations, biodiversity assessment was used as a survey method for confirmation and extending the existing data on the location for CWMF.

Beside site survey, literature data on the region were used obtained in previous research projects. Namely, from 2003 to 2008 monitoring were carried out throughout the year with a frequency of at least once a week covering the location of CWMF. During these monitoring, data on the complete fauna of birds in the area was collected. Also, since 2004 until now, a project has been implemented to preserve the Imperial eagles (Aquila heliaca) in Republic of Macedonia, and one of the authors of the EBS is active participant of the project implementation team. Therefore, it can be stated that the data provided in this document (EBS) are complete and fully reflects the site situation on the location proposed for CWMF.

In accordance with the methodological guidance on the provisions of Article 6(3) and (4) of the Habitats Directive 92/43/EEC the following information were considered during development of the EIA including EBS:

- a description of the project or plan;

- a description of the baseline environment where it is relevant to the conservation objectives of the Natura 2000 site (e.g. soil, water, flora and fauna, climate and the interrelationships between these factors);

- the identification of impacts and assessment of their significance;

- the recording and reporting of the findings of the assessment.

During the screening process the assessment have comprised four steps:

- determining whether the project or plan is directly connected with or necessary to the management of the site;

- describing the project or plan and the description and characterization of other projects or plans that in combination have the potential for having significant effects on the Natura 2000 site;

- identifying the potential effects on the Natura 2000 site;

- assessing the significance of any effects on the Natura 2000 site.

Further the screening process all predicted steps from appropriate assessment were considered during preparation of EIA including EBS, as follows: information required, impact prediction, conservation objectives and mitigation measures.

Following the the appropriate assessment, it may be concluded that an appropriate assessment report includes the following information:

- describes the project or plan in sufficient detail for members of the public to understand its size, scale and objectives;

- describes the baseline conditions of the Natura 2000 site;

- identifies the adverse effects of the project or plan on the Natura 2000 site;

- explains how those effects will be avoided through mitigation;

- sets out a timescale and identifies the mechanisms through which the mitigation measures will be secured, implemented and monitored.

As it was stressed before the ecological survey was focused on the location of the proposed landfill while birds and mammals were chosen as target taxa groups. Besides that dominant flora and fauna as well as some species of conservation importance were noted. This ecological baseline survey provides an insight into the ecological functions and importance of the habitats in question.

### 2.2.1 Duration and seasoning of the survey

The survey started in November end ended in February, following the additional investigation until July, carried out by personnel with adequate knowledge and field experience of the target taxa groups to be surveyed with the special accent to birds. According to the EIAO Guidance Note No. 7/2010, the duration of an ecological baseline survey for is generally dependent on the following factors:

(a) The geographical coverage of the project area;

(b) The diversity of habitats within the project area;

(c) The diversity of flora and fauna within the project area;

(d) Presence of ecologically important species or habitats which exhibit distinct seasonal patterns (e.g. migratory animals, seasonal wetlands);

(e) Availability of existing ecological information of the project area;

Because the project area Meckuevci-Arbasanci consists of common habitats mostly grasslands and degraded oak forest, a 4-month period plus 4-month additional investigation provide reasonable amount of site information on general wildlife. In the project area there is a natural gully which collects rainwater in winter months it was necessary the surveys to be done in the wet season.

Even it was a winter season, the first months of the survey were not with typical winter conditions like snow, wind, very low temperature etc. so the animals especially the target groups were active. This confirms the statement that different wildlife groups may differ in their activities and hence conspicuousness at different times of the year, as a result of difference in breeding seasons, migratory behavior, or physiological changes (e.g. low temperature restricts activity of poikilothermic but not homeothermic). Therefore one of the reason to use birds and mammals as target group (besides the fact that this is proposed IBA and EMERALD site) was that they are homoeothermic, i.e.  active in the period given for ecological survey (fig.3).

Figure 3. Description of the methodology for habitat, vegetation and plants, mammals and birds survey is in accordance with the EIAO Guidance Note No. 10/2010.

### 2.2.2 Habitat Survey

The purpose of a habitat survey was to identify different types of habitats (fig.4) found within a project area and to delineate their coverage. The results are consolidated in form of a habitat map and photographs shown in Annex 3. Mapping software Anatrack, was used for habitat mapping.

Satellite images (Google Earth) were used, because they provide useful basic information on habitat type/land use of the project area. Through their interpretation the general conditions (e.g., vegetation cover, land use) of the project area and its vicinity were readily visualized and translated into a preliminary habitat map. Because the dominant habitat type is grasslands, there is possibility of mapping error during the interpretation. Especially it was hard to distinguish if it is steppe and halophytic communities or typical pasture.

Figure 4. Different types of habitats

### 2.2.3 Vegetation and Plant Species Survey

Vegetation surveys focus on vegetation types in the project area while plant species surveys aim to provide information on plant species diversity and identify plant species of conservation concern. Vegetation surveys for ecological impact assessment studies are usually restricted to description of physiognomy or the growth form (e.g. forest, shrub or grassland) of the vegetation and conducted in conjunction with the general habitat surveys. A detailed vegetation survey of the project area with floristic analyses at plant community level is very time consuming and is normally not required in the context of EIA. A more direct approach is to produce a plant species list by direct observation while surveying representative parts of the project area. The dominant plant species (fig.5) with the presence of rare, protected and threatened plant species and other species of conservation concern were noted and their description is given in Chapter 4.2 “Description of habitats” as well as in Chapter 4.3“Valorization”.

Figure 5. Plant species

### 2.2.4 Mammal Survey

Mammals vary widely in ease of observation and different survey methods may be applied for different species. Conspicuous and large mammals may simply be counted by direct observation. However, mammals in East region of Macedonia which are of conservation concern are mostly secretive and nocturnal (fig.6). In this study “searching for traits” was method for surveying mammals which are difficult to observe directly. Signs such as dung, feeding signs, footprints, burrows and dens were noted as evidence of the presence of mammals in the project area. Observed burrow and the sounds of the small mammals helped to assess that they are still active.

Figure 6. Mammals in the area (pictures are not taken in study area)

### 2.2.5 Bird Survey

Identification of bird species (fig.7) was done visually or aurally by recognition of unique songs and calls. In addition to identifying the bird species under observation, behaviors of the bird such as feeding, were recorded.

The subject of selective inventory has included an area of ​​1500 meters in a circle around of specified location, or monitoring of area on location of the landfill and its surroundings.

Based on relief of the terrain, in order quality of field activities to be improved and as well as providing the best results, monitoring was carried out using two methods. Where condition on the terrain allows the method of “line transect” has been used; starting in the early morning on a predetermined route, mainly along the line of the ridge. The monitoring in the further phases (construction and operational) can give computational analyses using the data line transects that would be performed on the same routes. On the other hand, where the terrain did not allow research method of line transect, the monitoring was carried out with "point count" of the advanced selected points. Counts were undertaken from fixed locations for a fixed period of time (e.g. 2 to 20 minutes).

Figure 7. Bird species in the area

Line transect and point count monitoring

In line transect surveys (fig.8) the observer follows a predetermined path of known length; for counting from specific location (point count), requires the observer to collect visual and sound data of a particular item for in a particular interval of time. For a better estimations and appropriate representation of the observed bird species, both of the above mentioned methods can be combined.

Figure 8. Line transect

Monitoring of birds, required by the EIA, was performed so photographs of birds in the affected area and its surrounding are presented in Annex 4 of EBS.

During the research the following equipment was used to identify the types of birds:

- Binocular Minox 8x40

- Telescope Svarovski 20-60x100

Additionally, the below mentioned literature and applications were used:

- Collins Birds Guide (Peter James Grant HarperCollins, 2000 - Birds)

- RSPB POCKET BIRDS (Jonathan Elphick et al. 2003)

- Audio recordings (mobile application with sounds of birds)

## 2.3. Limitations of Survey

During the field work access was only available for habitats within the land ownership boundary and therefore habitats outside of this were not subject to a full habitat survey, although broad habitat types and boundary features were recorded and no significant constraints were encountered. It must be noticed too that Meckuevci is a hunting area for small game and the time for conducting ecological survey matches with the hunting season for small game, so it wasn’t safe to walk every ware. Despite of that, winter is the season when sheep’s from the high mountains of western Macedonia are placed in Ovce pole where they have greater availability of food. Because they use site Meckuevci-Arbasanci as a pasture, it was difficult to identify the signs of “resident” mammals, mainly because of the dungs of sheep and sheep dogs.

The preparation of the biodiversity baseline has been mainly limited by the lack of existing detailed data for the project area. Nor vegetation maps or habitat maps exist in the Republic of Macedonia at the national or regional levels. Much of the information, are interpretation of satellite images and aerial photographs, and field surveys. Even bats are widespread, they have not been considered in detail in this assessment, as there is no information on their distribution and behavior in the Project area.

Moreover, the determination of sensitive plant and animal species that are potentially present has been limited by the fact that no Red Data Books and Red Lists for Macedonian flora,  fauna and fungi have been prepared yet at a national or regional level. Thus, the establishment of the presence of sensitive species in the landfill area had to be done on the basis of the most relevant international conventions and treaties (Bern Convention, IUCN red list, Habitats Directive, Bird Directive etc.).

The problem arises from the fact that there may be species that are of low concern at an  international level, but have some level of threat in the Republic of Macedonia (or vice versa), thus underestimating (or over‐estimating) the sensitivity value of the species.

# 3. LEGISLATIVE, REGULATORY AND POLICY FRAMEWORK

The legal basis for nature protection in the Republic of Macedonia is contained within the Constitution, the Law on Nature Protection (Official gazette No. 67/04 and relevant up to date amendments), the Law on Environment (Official gazette No. 53/05, and relevant up to date amendments), and in international agreements signed or ratified by the Country and other laws regulating the use of certain natural resources.

The Law on Environment transposes the requirements of various EU requirements, including those of Directive2003/35/EC2; Council Directive 96/61/EC3; Directive 2001/42/EC4; and Council Directive82/501/EEC5.

The Law on Nature Protection transposes the following Directives: Council Directive 92/43/EEC116, Council Directive 79/409/EEC127, Council Regulation (EC) No 338/97138 etc. This law sets out principles of protection, restrictions regarding use of nature and natural resources, impact assessment, planning, compensation measures, protection of biodiversity, protection of internationally important species, wildlife conservation, genetic diversity, habitats and ecosystems, ecological networks minimum environmental release, restrictions for construction activities in riparian habitats and littoral areas, restriction of fishing in certain conditions, protected areas, management plans for protected areas, rangers, landscape diversity, organisation of nature protection including management of protected areas, financing inspection and supervision, penalties and final and transitional provisions.

Also, several international environmental and conventions are relevant are listed below:

      - Convention on Biological Diversity (CBD)

      - Berne convention on the Conservation of European Wildlife and natural Habitat

      - Bonn Convention: Conservation of Migratory Species of Wild Animals

      - Convention on Wetlands of International Importance;

      - Convention on the International Trade in Endangered Species of Wild Flora and Fauna (CITES);

The mitigation hierarchy (fig.9, below) is crucial for all development projects aiming for No Net Loss (NNL) or Net Positive Impact (NPI) or those adopting a Net Positive Approach (NPA). It is based on a series of essential, sequential steps that must be taken throughout the project’s life cycle in order to limit any negative impacts on biodiversity.

1.  Avoidance: the first step of the mitigation hierarchy comprises measures taken to avoid creating impacts from the outset, such as careful spatial or temporal placement of infrastructure or disturbance. For example, placement of roads outside of rare habitats or key species’ breeding grounds, or timing of seismic operations when aggregations of whales are not present. Avoidance is often the easiest, cheapest and most effective way of reducing potential negative impacts, but it requires biodiversity to be considered in the early stages of a project.

2. Minimization: measures taken to reduce the duration, intensity and/or extent of impacts that cannot be completely avoided. Effective minimization can eliminate some negative impacts

3. Rehabilitation/restoration: measures taken to improve degraded or removed ecosystems following exposure to impacts that cannot be completely avoided or minimized. Restoration tries to return an area to the original ecosystem that occurred before impacts; whereas rehabilitation only aims to restore basic ecological functions and/or ecosystem services (e.g. through planting trees to stabilize bare soil). Rehabilitation and restoration are frequently needed towards the end of a project’s life-cycle, but may be possible in some areas during operation (e.g. after temporary borrow pits have fulfilled their use).

Collectively avoidance, minimization and rehabilitation/restoration serve to reduce, as far as possible, the residual impacts that a project has on biodiversity. Typically, however, even after their effective application, additional steps will be required to deliver No Net Loss or a Net Positive Impact.

4. Offset: measures taken to compensate for any residual, adverse impacts after full implementation of the previous three steps of the mitigation hierarchy. Biodiversity offsets are of two main types: ‘restoration offsets’ which aim to rehabilitate or restore degraded habitat, and ‘averted loss offsets’ which aim to reduce or stop biodiversity loss (e.g. future habitat degradation) in areas where this is predicted. Offsets are often complex and expensive, so attention to earlier steps in the mitigation hierarchy is usually preferable.

Figure 9. Sequential steps of the mitigation hierarchy

# BASELINE

## 4.1 Designated Sites for Nature Conservation

The Law on Nature Protection regulates the protection of nature through protection of biological and landscape diversity and protection of natural heritage within and outside protected areas. Except for already declared protected natural areas, none of these areas have yet a legal status under Macedonian law that fully warranties their protection. These areas include:

- Protected natural areas declared (or anticipated to be declared) under Macedonian law;

- Proposed ecological corridors of the national ecological network to connect protected areas and  ecologically important areas (including future Natura 2000 sites), as required by Article 53 of Law on  Nature Protection;

- Natural areas covered under  the protection regime of European Union legislation or international  conventions (Natura 2000/Emerald sites);

- Proposed areas for the management of species;

- Other areas of natural interest without protection coverage (IBAs, IPAs,PBAs)

### 4.1.1 Protected Areas and Proposed Protected Areas under Macedonian

### legislation

The following maps present an overview of all protected areas and areas proposed for protection in the project region (fig. 10 and 11).

Figure 10.  Protected areas

Figure 11. Proposed Nature Protected Areas

Figure 11. Proposed Nature Protected Areas

Protected areas

The proposed CWMF site is not located within the boundaries of any national nature conservation area as presented on figure 10.

Proposed Protected Areas

There are no protected areas in the project area, but there are 5 areas important for nature protection and these were proposed for protection in the national system of protected areas (fig.11). CWMP Meckuevci – Arbasanci is near to some proposed protected areas as follows:

- Lower Zletovska (distance cca 9,55km)

- Mangovica (distance cca 5,80 km)

- Ovce Pole (distance cca 9,70km)

- Nemanjica (distance cca 4,80 km)

- Vilcanin bombs (distance cca 9,40km)

Mangovica is one of proposed protected areas as Nature Park. The area Mangovica spreads on the western parts of the mountain Mangovica and occupies a surface of 3.271ha. The main goal for protection is the ornitho fauna represented by multiple significant bird species, especially birds of. The imperial eagle Aquila heliaca, golden eagle Aquila chrysaetos, lanner falcon Falco biarmicus, long-legged buzzard Buteo rufinis nest here and black stork Ciconia nigra is also present. A significant herpetofauna species registered in this area is the four-line snake Elaphe quatuorlineata.

Nemanjica is situated in the eastern part of Ovche Pole basin, north-east of Sveti Nikole near the village of Nemanjica. The area belongs to the eastern rim of Vardar zone towards Zletovo, Kratovo volcanic formation and in narrower terms it is part of Upper Eocene flysch complex. Geological profile of Paleogene flysch series was revealed on the site where sandstone horizon is rich in fossil flora and fauna. Fauna is marine, typical for Upper Eocene, while flora indicates existence of tropical climate conditions. Determined fossil fauna is typical of the development of this geological period and consists of the same species determined in other localities within flysch formation, in which shallow aquatic and sandbar organisms prevail: Cardium, Spondylus, Meretrix, Ostrea, Ceritium, Natica, Conus, various corals, etc. The locality is paleontologically important and also carries significant scientific, research and educational importance, especially given the fact that it is comparable with other finds in Central Europe and Dinarides. Presence of marine fauna from late Eocene has national importance.

Ecological network/ecological corridors The Project for development of the national ecological network in the Republic of Macedonia (known as  MAK‐NEN), implemented by the Macedonian Ecological Society (MES) and the European Centre for Nature  Conservation (ECNC) in cooperation with the Ministry of Environment and Physical Planning of the Republic of  Macedonia  includes  one  element of the ecological network – steppe core area Mangovica (fig.12). This steppe core area is proposed areas to be comprised in the MAKNEN and is yet to be approved by the Ministry of Environment.

and Physical Planning.

Figure 12. MAK‐NEN biocoridor Mangovica

The Mountain Mangovica (on east of Sveti Nikole up to Osogovo Mountains on east and Sredorek on north, and Ovce Pole on south) (Fig. 12). It is characterized with mild hills with plateaus and rounded peaks and continual areas under montane pastures. Its aim is providing vitality of organism’s populations that depend on grass habitats to survive. The size of the core area is 40.494 ha. The main goal for protection is the ornitofauna represented in this area by multiple significant bird species, especially birds of prey. Thus, all of the below identified mitigation and monitoring measures are covering concerns of this area too.

### 4.1.2 Protected Areas under International legislation

Important Bird Areas and Natura 2000

The Member States of the European Union, the protection of birds implement based on the provisions of the Directive on the protection of wild birds (2009/147/ EC). This Directive, in its original form since 1979 is the first document of the European Commission intended to preserve nature. Among other things, it stipulates habitats and species of wild fauna and flora (92/43 EEC) provides proclamation of special areas (SPA - Special Protected Areas) to protect some bird species. Another document of the European Union Directive on the protection of habitats and species of wild fauna and flora (92/43 EEC) provides proclamation of special areas of conservation (SAC - Special Areas for Conservation). Together, these two types of designated areas, cost the European ecological network Natura 2000.

Based on internationally established criteria, international organization BirdLife International (http://www.birdlife.org/) identifies the important areas for Birds (IBA - Important Bird Areas) of World, European and regional level.

Figure 13. Birdlife factsheet for IBA Ovce Pole

CWMF Meckuevci - Arbasanci is situated within the proposed IBA site – MK019 Ovce Pole (close to the northern boundary zone). On Figure 13 below, Birdlife Ovce Pole factsheet is given.

There are two other proposed IBA sites in the wider area:

- IBA MK012 River Zletovska Valley - at cca 2,50 km from CWMF

- IBA MK015 Preod-Gjugance - at cca 5,60 km from CWMF

These proposed IBA sites (IBA MK012 River Zletovska Valley and IBA MK015 Preod-Gjugance) consist of the same main bird species as proposed IBA Ovce Pole i.e. Aquila heliaca and/or Falco naumanni. The project may have impact on these IBA areas as presented in Chapter 4.4, so the identified mitigation, management, and monitoring measures are covering the concerns of this sites.

As presented in the Chapter 4.4., below, there are 2 active nesting sites of Aquila heliaca near the CWMF concerning this proposed IBA areas (distance 5 km and 5.1 km), one within the boundaries of proposed IBA MK012 River Zletovska Valley and one within the area of proposed IBA MK015 Preod-Gjugance.

In addition Birdlife River Zletovska Valley and Preod-Gjugance factsheets are given.			There is one proposed IPA (Important Plant Area) site near and it is “Ovce Pole-Bogoslovec” at distance cca 3,70 km and no PBA (Prime Butterfly Area) site.

Emerald sites (proposed)

Emerald Network is established on the territory of the countries Parties to the Bern Convention and important part in the preparation of candidate countries for EU membership for the implementation of the EU Birds and/or Habitats Directives, or an additional tool in the process of establishing the coherent European Environmental Network Natura 2000. 35 locations have been identified and proposed in the National Emerald network. Ovce Pole is one of the proposed sites with Site code MK0000035.

Key Biodiversity Areas (KBAs)

In 2011 Melovski Lj. et al. identified KBAs in Macedonia, using a combination of Important Plant Areas (IPAs) and Important Bird Areas (IBAs) that have already been identified. Ovce Pole (fig.14) was one of the 42 KBAs proposed in Macedonia.

Figure 14.  KBA Ovce Pole

Endangered species of Egyptian Vulture (Neophron percnopterus) has not been identified in the project during the survey nor in the literature data. Within the radius of 10 km of project area (CWMF) there are not any data of active nests of Egyptian vulture thus assessment of this impact has been avoided and it can be concluded that location of CWMF does not enter in the core area of these species.

## 4.2 Description of habitats

Figure 15.  Biomes (zonobiome and orobiome) in the Bregalnica River Watershed and the East Planning Region (modified from Matvejev & Puncer 1989).

The project area is under thermophile vegetation, where potentially Querco-Carpinetum orientalis forests could dominate and they represent the continental-sub Mediterranean area (pink area of fig.15). The East Planning Region as well as the project area CWMF site Meckuevci - Arbasanci  belong to the Balkan Highlands Province of the Palearctic Realm. This province actually, belongs to the central part of the Balkan Peninsula. A larger part of Eastern Macedonia including the project area belongs to the European Continental Bio-geographical Region according to the classification of the European Environment Agency.

The following section presents the summarized results of biodiversity assessment including field observations and literature data on flora and fauna composition of the region. It contains the habitats description, distribution and importance at the local and regional levels. Immediate surroundings of CWMF were analyzed too, as well as the wider area and range, for the vagile species of organisms such as birds (this is proposed IBA area), the distance of several kilometers could be just a small part of their basic living space. Therefore, the study reviewed the biotopic and biome distribution near CWMF, at maximum distance of 3 km.  Habitats in the radius of 3 km are presented on a map (see Annex 3).

The habitats in the area can be divided in two main categories according to their origin, namely, natural and anthropogenic habitats.

The main criterion to make the division within these categories was the division proposed by EUNIS Habitat Classification (European Commission, DG Environment http://eunis.eea.europa.eu/habitats-code-browser.jsp ), but other criteria such as presence of different plant communities, distribution, degradation level were also used.

The description of the habitats that is presented below follows the following pattern:

- At the beginning, a description of the general characteristics of the habitat followed by a reference to their general distribution in the region and their distribution in the project area;

- Description of the plant association. Dominant and most common plant species and characteristic fungi species;

- Description of the fauna within the habitat, presented by vertebrates (amphibians, reptiles, birds and mammals) and selected groups of invertebrates (dragonflies, ground beetles, grasshoppers and daily butterflies).

NATURAL HABITATS

Natural forests and shrubland

Forests and scrublands are divided into three main habitat types:

  - Oak forest (forest of pubescent oak and oriental hornbeam),

  - Beech forest, and

  - Riparian habitats.

OAK FOREST BELT

This type of EUNIS habitat matches with biome of Submediteranean-Balkans Forests (SBF) (fig.8) classified by Metvejev 1995. In the Balkans, it spreads over the highlands and the area before the highlands. Because of the human influence, it is turned into a farming area or in bare and fruitless screes. Therefore, the original territory of the SBF is decreased at the expense of extending the Eurasian steppes, cultural steppes and semi-deserts (which is the case in this study area). Main type of biocenosis are dryforests where the trees are quite apart (open), so that growth of shrubs and grasses is enabled.

A series of adaptations occurred, which helped life organisms to avoid unfavorable conditions, such as winter sleep, underground way of life and seasonal migration. Terofits and cryptofits prevail in the plants. Animals feed with dry grass (in winter) and dried mushrooms and pips - insured by the short term and partial winter cover. Under the influence of agricultural activities, irrational forestry and livestock, many biocenosis are partialy altered or destroyed. Thus the new biocenosis of submediteranean balkan type (communities and heats and shrubs) were created.

This biome in the project area is mainly represented by oak forests (Quercus pubescens) or communities of iatlian oak and hornbeam (Carpinus orientalis).

Main characteristics – As it was said before this community is a relic of well-developed oak-hornbeam forests. There is a lower percentage of deciduous species (Carpinus orientalis, Quercus pubescens, Fraxinus ornus, etc.) because of their excessive exploitation in the past and the present, which changed the physiognomy of natural community. These oak parts phytocenologically belong to the described community - ass. Paeonio-Quercetum pubescentis Riz. prov. or fragments of ass. Querco-Carpinetum orientalis macedonicum Rud. 1939 ap. Ht. 46, in whose composition there is absence of species Carpinus orientalis. Degraded belts are characterised by the presence of the, invasive species Paliurus spina-christi, Pyrus amygdaliformis, Prunus spinosa etc. The dominant plant community representing this habitat is Paliuretum submediterraneum with dominant species Paliurus spina-christi. The most important plant species in this community are Paliurus spina-christi, Quercus pubescens, Fraxinus ornus, Juniperus oxycedrus, and Pistacia terebinthus (in some places).

Distribution – Oak-hornbeam forests is widespread in the Adriatic and Aegean sub-Mediterranean region. In Macedonia, it is climate zone assigned to about 600 m altitude, and the southern slopes of the mountains reach a height up to 1000 m.

Distribution in the project area – This habitat is present in the surroundings of locality, mostly intermixed with hill pastures in the south part (see Habitat Map at Annex 3).

Flora – These degraded habitats are characterised with presence of Paliurus spina-christi, Prunus spinosa, Pyrus amygdaliformis, Juniperus oxycedrus, Coronilla emeroides, Crategus heldreichii etc. Representatives of grass plants are: Ajuga laxmanii, Minuartia glomerata, Euphorbia myrsinites, Knautia orientalis, Tunica illyrica, Althea sp.

Fungi – Lignicolous fungi that can be found in the area are: Peniophora cinerea (of Paliurus spina-christi), Peniophora junipericola (of Juniperus spp.) and Peniophora incarnata, Laeticorticium polygonioides at others (of Pyrus amygdaliformis).Characteristic for the degraded forests are also terricolous species which develop on grass areas, as Agaricus spp., Bovista plumbea, Hygrocybe conica, Marasmius oreades and others.

Mammals – Common species are the following: hedgehog (Erinaceus concolor), marbled polecat (Vormela peregusna), Levant vole (Microtus guentheri), also: Apodemus flavicolis, A. agrarius, Rattus rattus, Mus macedonicus, Lepus europeus, Canis lupus, Vulpes vulpes, Mustela nivalis, Meles meles, Felis sylvestris, Sus scrofa.

Birds – Birds in the area represented by Passer hispaniolensis, Hippolais pallida, Sylvia spp., Lanius collurio, L. minor, L. S enator, Parus lugubris, Dendrocopos syriacus, Ficedula semitorquata, Streptopelia decaocto, Accipiter brevipes, as well as some types of species Emberiza characteristic for hilly meadow.

Reptiles and amphibians – The amphibians found in this habitat are: Fire Salamander (Salamandra salamandra), Common Newt (Lissotriton vulgaris), Yellow Belied Toad (Bombina variegata), Common Toad (Bufo bufo), Green Toad (Pseudepidalea viridis). The reptiles found in this habitat are Hermann's Tortoise (Eurotestudo hermanni), Greek Tortoise (Testudo graeca), Erhard’s Wall Lizard (Lacerta erhardii), Green lizard (Lacerta viridis), Balkan Green Lizard (Lacerta trilineata), Snake‐eyed Skink (Ablepharus kitaibelii), Slow Worm (Anguis fragilis), Aesculapian Snake (Zamenis longissimus) and Nose‐horned Viper (Vipera ammodytes).

Invertebrates – Characteristic  species of butterflies in this dwelling consists of typical species that has a dry, shrubby vegetation as: Thymelicus sylvestris, Phengaris arion, Melitaea phoebe, Arethusana arethusa, as well as species that are common to a variety of habitats: Iphiclides podalirius, Papilio machaon , Aporia crataegi, Carcharodus alceae, Gonepteryx rhamni, Limenitis reducta, Nymphalis antiopa, N. polychloros, Erebia medusa, Argynnis niobe, Aglais io, Plebeius agestis, Vanessa cardui, V. atalanta, Melanargia larissa, Coenonympha pamphilus, Leptidea sinapis, Colias crocea , Satyrium acacia, Hamearris lucina etc.

The bug fauna (Carabidae) is represented by species typical of mountain pastures and forests of oak, because there are no specific habitat types in the degraded oak forests. This habitat with low vegetation allow the emergence of strains that develop in the trees just at the stage larvae (almost all types of the subfamily Lepturinae, such as S. bifasciata, S. melanura, S. nigra, S. septempunctata, P. livida and others) and remain in the same area as imago, because they can feed with flowers that grows near. Due to the open areas, with rare vegetation, here can be found crickets: Tylopsis lilifolia, Ancistrura nigrovittata, Poecilimon thoracicus, Polysarcus denticauda, ​​Tettigonia viridissima, Decticus albifrons, Platycleis affinis, Odontopodisma decipiens, Omocestus rufipes, Chorthippus bornhalmi, Euchorthippus declivus etc.

RIPARIAN HABITATS

Forests and shrublands have developed in narrow stretches along some of the riverbanks, channels or guillies found in the project area. Well-preserved forests of this type are currently very rare. People used to clear these stands for providing fertile alluvial soil for agriculture. In this area, the forest communities belong to Salicion albae Soó (30) 1940 alliance.

Main characteristics – This type of river forest belongs to the community Salicetum albae-fragilis Issler 1926. It represents a very narrow belt along the streams with willow domination and rare occurrence of poplars. The biotope is characterized by constant moisture. Most characteristic tree species of are Salix alba, or mixed communities of Salix alba and Salix fragilis. Tree species as: Populus nigra, Salix triandra, Sambucus nigra, Viburnum opulus, Cornus sanguinea, Amorpha fruticose, Ulmus sp. and others can be found in small groups or individually. Most characteristic grass species are: Poa trivialis, Poa palustris, Carex vulpina, Polygonum lapatifolium, Polygonum hidropiper, Rumex sanguineum, Veronica anagalis-aquatica, Scirpus lacustris and others.

Distribution – This biotope is present in almost all river and streams valleys in Macedonia.

Distribution in the project area – The location has natural gully Klobuč that can collect water after rains and in the rainy season. This community can be found as a belt along the natural gully.

Flora – Characteristic plants species are stated in the habitat description.

Fungi – Lignicolous fungi are present, as well as parasites and saprobes of Salix alba, Populus tremula and Alnus glutinosa. Species Laetiporus sulphureus, Phellinus igniarius and Panus tigrinus are charcteristc for willows. Representatives of parasite fungi are: Phellinus igniarius (of Salix alba), Phellinus tremulae (of Pupulus), Ganoderma applanatum, Polyporus squamosus and Pleurotus cornucopiae. Representatives of saprobes are Funalia trogii, Ganoderma adspersum, G. resinaceum, Pleurotus ostreatus, Trametes ochracea etc.

Mammals – Fauna consists of Apodemus sylvaticus, Crocidura suaveolans, Mustela nivalis, Sus scrofa, Sciurus vulgaris, Talpa europea and Vulpes vulpes.

Birds – Characteristic species for this habitat are warbler (Cettia cetti) and penduline tit (Remiz pendulinus). Several other kinds use the willows for protection and reproduction, most commonly: nightingale (Luscinia megarhynchos), European robin (Erithacus rubecula), blackcap (Sylvia atricapilla) and others.

Reptiles – Most common species are snakes Natrix natrix and N. tesselata,also Elaphe longissima and Vipera ammodytes. Other more common are turtles (Eurotestudo hermanni, Testudo graeca) and lizards (Lacerta erhardii riveti, L. viridis, L. trilineata, Anguis fragilis).

Amphibians – Representative species are marsh frog (Rana ridibunda) and Triturus carnifex, and there are also the fire salamander (Salamandra salamandra), the smooth newt (Lissotriton vulgaris), Bombina  variegata, Bufo bufo, Pseudepidalea viridis, Rana graeca and Pelophylax ridibundus.

Invertebrates – This habitat is suitable place for daytime butterflies. Most common species is Lycaena tityrus, other than Apatura ilia, Polygonia c-album, Maniola jurtina, Pieris mannii, Pararge aegeria, Leptidea sinapis, Limenitis reducta, Erebia ligea, Vanessa cardui, V. atalanta, Aglais urticae, Aglais io, Anthocharis cardamines, Colias crocea, Gonepteryx rhamni, Argynnis adippe, A. paphia, Melanargia larissa, Pyronia tithonus, Nymphalis antiopa, N. polychloros, Polyommatus icarus, Satyrium spini. Insects are also present, and the dominant species include the following: Carabus granulatus, Chlaenius nitidulus, Stenolophus mixtus, Agonum sexpunctatum. The dragonfly’s characteristic of the area are Calopteryx virgo, Calopteryx splendens, Libelulla depressa and Sympetrum sanguineum. Orthoptera are represented by grasshoppers of the Tetrix genus.

Main characteristics - The water flow in these streams exists only during the humid period of the year. They have high water level after snow melting in early spring, and a half of the year (more or less) these streams are characterised by a dry bed. That is the reason why these streams do not have great importance as water ecosystems. But, the ravines through which they flow are regularly covered by denser  or  sparser  willow  forests  or  belts,  thus  strongly  differing  from  surrounding  grassland  or  agricultural  habitats.

Distribution - Intermittent stream named Klobuč, running along the eastern site area, towards south.

Amphibians - Amphibian species present in this habitat are: Yellow Belied Toad (Bombina variegata),

Common Toad (Bufo bufo) and Green Toad (Pseudepidalea viridis).

ButterfliesCommon species for this habitat are: Carcharodus flocciferus, Pygus alveus, P. sidae, Spialia  orbifer,  Parnassius  mnemosyne,  Zerynthia  cerisy,  Apanthopus  hyperantus,  Arethusana  arethusa,  Lasiommata petropolitana, Pyronia tithonus,Vanessa atalanta etc.

OPEN TERRAIN – GRASSLANDS

The grasslands in the area are not characteristic vegetation types. This region is either covered by zonal pubescent oak and oriental hornbeam shrub land or it is turned into arable fields after clearing of shrub land. As a secondary formation, most of the grasslands are surrounded by sparse vegetation of different degradation stages. Almost all project area (approximately 90%) is presented by category hill pastures, which is characteristic of the hilly areas in all valleys and plateaus in Macedonia.

This type of EUNIS habitat matches with biome Mediterranean semi deserts (MSD) classified by  Metvejev 1995. According to Metvejev this type of biome is easy recognizable as a biome of open grassland with grass cover and rare shrubs. It is almost impossible to distinguish natural habitats from the man made in the territory of other biomes for livestock and agriculture needs. The type of life form is with specific life adjustments to open terrain, lack of moisture, with strong winds, high temperatures, and winter cold. The plants and animals there have an annual cycle of activities with two interruptions: summer and winter. In connection with this there were changes in the daily cycle: summer, many species are active at night and in the evening, and winter the same species is active only during the day. Many characteristic species are wanderers during the winter.

Main characteristics–Pseudo-steppe with grasses and annuals includes a variety of xeric, thermophilic and mostly open Mediterranean perennial and annual grasslands. As a consequence of the looseness of its definition, at least three broad subtypes should be considered within the 6220 habitat type, each of which are described below in terms of their habitat features, ecological requirements and variability.

Three major sub-types that should be considered are: one of perennial basophile rather hard short-grass communities, included in Lygeo-Stipetalia; another one of very dense and short but highly productive perennial summer drying swards, created by intense and continuous livestock activity, included in Poetalia bulbosae; and a last one of pioneer and ephemeral basophilous annual grasslands, included in Brachypodietalia (Trachynietalia) distachyae.

Subtype 1: Lygeo-Stipetalia

Mediterranean perennial basophile rather hard short-grass communities dominated by Brachypodium retusum (= B, ramosum. B. bosissieri ) is included in Order Lygeo-Stipetalia Br.-Bl. & O. Bolós 1958 (= Thero-Brachypodietalia); Class Lygeo-Stipetea (= Thero-Brachypodietea). The most widespread and important alliance within this subtype is Thero-Brachypodion retusi (ramosi) though other, such as Triseto-Brachypodion boissieri and Stipion parvifoliae, should also be considered.

This habitat is typical is usually found in areas of long summer drought period just like Ovcepole. Under semiarid conditions, the grass community usually “hides” under shrub cover. The habitat is found mostly on soils degraded by erosion. When this habitat subtype is in good ecological and management conditions, ground cover is often high, sometimes complete. Since it is considered to be the last substitution stage of sclerophyllous forests, Mediterranean shrublans, it usually occurs associated with those woody communities, both in small to medium size gaps or clearings and under their often light canopies. Its presence is usually linked with extensive grazing (sheep and goat) and frequent fires.

Subtype 2: Poetalia bulbosae

Very dense and short, but highly productive, Mediterranean perennial swards dominated by Poa bulbosa, and also including many annual species. They are created and maintained by intense and persistent livestock (usually sheep). Grazing and selection of species on the one hand, and fertilization and nutrient cycle acceleration through faeces on the other result in the optimum Mediterranean sward type for livestock grazing. It has been described as a `cultural´ pastoral climax (Montserrat & Fillat 1999). This subtype is included in Order Poetalia bulbosae Rivas Goday & Rivas Martínez in Rivas Goday & Ladero 1970; Class Poetea bulbosae.

This habitat is found on oligotrophic or eutrophic soils, but the upper soil horizon is always rich in matter and nutrients as a result of intense and persistent livestock activity.

When this habitat subtype is in good ecological and management conditions, ground cover is complete, woody vegetation is usually absent or scarce (due to intensive browsing in times of green grass shortage in summer and winter), and legumes are abundant.

Another essential feature is the high level of activity of soil meso and microfauna: e.g. worms, ants, bacteria, etc.

The most characteristic species of this sub-type are: Poa bulbosa, Bellis annua, Parentucellia latifolia, Trifolium subterraneum (oligotrophic soils), Trifolium scabrum, Plantago albicans, annual medics: Medicago sp. (eutrophic soils), Plantago serraria (clay soils).

Subtype 3: Brachypodietalia distachyi

Pioneer and ephemeral basophilous annual grasslands, usually growing on lithosols. This subtype is included in Order Brachypodietalia distachyi Rivas-Martínez 1978 (syn. Trachynietalia distachyae).

This habitat subtype is considered to be the last substitution stage of xeric Mediterranean forests and shrub communities. Since woody plants are stronger competitors than herbs under Mediterranean climate, this subtype usually occurs in the clearings of woody chamaephyte communities, forming a mosaic-like pattern in which the area occupied by its communities is usually small: a few square metres. Indeed, these patches get even smaller as climatic humidity increases.

The most characteristic species in this sub-type are: Brachypodium distachyon (syn. Trachynia distachya), Bombycilaena erecta, Echinaria capitata, Polygala mospelliaca, Scabiosa stellata, Stipa capensis.

This habitat features grass vegetation with surrounding oak forest in different degrees of degradation . This represents a secondary vegetation formation, created from gradual and long-term degradation of the forests which were widely distributed in this region in the past. Large trees characteristic of these degraded forests are: (Quercus pubescens, and Fraxinus ornus), smaller trees (Carpinus orientalis, Cornus mas, Crataegus monogyna, Pyrus amygdaliformis, Ulmus minor) or bushes (Prunus spinosa, Paliurus spina shristi, Rosa spp., Colutea arborescens, Coronilla .

Distribution in the project area – The hilly pastures with sparse shrubs (dominant Subtype 3: Brachypodietalia distachyi) are present in almost all project area .

Flora – Characteristic plants species are stated in the habitat description.

Fungi – This habitat is characterised by presence of terricolous fungi, while lignicolous are rarer due to scarce substrates and are mainly connected with Juniperus spp., Coronila emeroides, Prunus spinosa and others. Most of the species are non-mycorrhizal representatives of genera Agaricus, Entoloma, Stropharia, Hygro- cybe, Panaeolus, as well as some Gasteromycetes from genera: Calvatia, Bovista and Vascellum. Also found (though very rarely) are mycorrhizal representatives of the genera Amanita and Russula.

Mammals – Macedonian mouse (Mus macedonicus), Günther's Vole (Microthus guentheri) are characteristic of habitats with thin bushes, as well as marbled polecat (Vormela peregusna) and mammals which enter from neighboring habitats in search for food: e.g. the fox (Vulpes vulpes), marten (Martes sp.), European badger (Meles meles), and wild boar (Sus scropha).

Birds – Galerida cristata, Lanius collurio, L. senator, Buteo buteo, Merops apiaster, Columba livia, Oenanthe oenanthe, Corvus corax, Passer domesticus, Corvus cornix, Pica pica, Sylvia communis, Turdus viscivorus, Upupa epops Emberiza caesia, Sturnus roseus, Melanocorypha calandra, Burrhinus oedicnemus, Otis tetrax  and others.

Reptiles and amphibians – Characteristic types for this habitat are: Testudo graeca (Mediterranean tortoise), Lacerta erhardii riveti (Macedonian wall lizard), Elaphe quatorlineata (four-lined snake) and Vipera ammodytes (horned viper). Other than the characteristic ones, there can also be found species like Coluber caspius, Coluber najadum and Natrix tessellata.

Invertebrates – Butterfly species are: Iphiclides podalirius, Euchloe ausonia, Maniola jurtina, Colias alfacariensis, Plebeius agestis, and beetles species - Acinopus picipes and Dixus obscurus. Characteristic species of grasshoppers include: Calliptamus italicus, Dosciostaurus marrocanus, Oedipoda miniata, Glyphothmetis heldreichii, Asiothmetis limbatus, Paracalopternus caloptenoides, Acrida sp. Ground beetles: Harpalus metallinus, Carabus graecus morio, Pachycarus cyaneus, Brachinus brevicollis.

STEPPE-LIKE DRY GRASSLAND AND HALOPHYTIC COMMUNITIES

Steppe elements in this region occur within two vegetation types, namely halophytic steppe elements in salt valleys, and steppe elements which are not halotolerant on dry grasslands. Following these findings, two types of steppe vegetation can be distinguished. We have used in the report the term dry steppe for vegetation on non-salinised dry habitats and salt steppe for communities appearing on salty soils.

Main characteristics – Steppe-like vegetation is developed on Paleogene and Neogene marshes, with presence of steppe-like species like: Astragalus parnassi, Hedysarum macedonicum, Morina persica, Onobrychis hypargyrea. The most common is ass. Astragalo-Morinetum which usually develops on eroded slopes.

Distribution – These communities develop in the central parts of Macedonia, in the region between Negotino, Stip and Veles.

Distribution in area of the area – In the area, this habitat is represented  in fragments  or mixed with pastures  (see Habitat Map).

Reptiles and amphibians – Typical for this habitat is Balkan wall lizard (Podarcis taurica) and Javelin sand boa (Eryx jaculus).

Birds – Common quail (Coturnix coturnix), grey partridge (Perdix perdix), and little bustard (Tetrax tetrax).

Mammals – Geoffroy’s bat (Myotis emarginatus), whiskered bat (Myotis mystacinus), Levant vole (Microtus guentheri), southern vole (Microtus levis), yellow-necked mouse (Apodemus flavicollis), lesser mole-rat (Spalax leucodon), European mole (Talpa europaea), brown hare (Lepus europeus) and marbled polecat (Vormela peregusna).

Main characteristics – The reed belts found in the area are not a typical biotope. They develop as narrow passages along the slow waters of the natural gully. This type of reed belts usually represents fragments of the swampy plant community Scirpo- Phragmitetum W. Koch 1926. The bulrush Typha latifolia sets the physiognomy of this habitat, and also present is common reed (Phragmites australis). Common vascular plants which usually grow in water habitats are: Veronica anagalis-aquatica, Veronica beccabunga, Stelaria aquatica, Lycopus europaeus, Myosotis scorpioides, Alisma plantago-aquatica, Rumex cristatus, Polygonum hydropiper and Ranunculus repens. This type of biotope is very poor from the point of view of flora and fauna.

Distribution – This biotope appears in almost all lowland rivers in Macedonia

Distribution in the project area – They are present in small areas around the natural gully at the south-west part.

Flora – Characteristic types of plants are stated in the habitat description.

Mammals – Typical species for reed belts include the striped field mouse (Apodemus agrarius), and the Crocidura suaeveolans, Microtus rossiaemeridionalis.

Birds – Reed beds feature with a rather low diversity of birds. Only small number of bird species visit this habitat in search for food or migration.

Reptiles – Common reptiles include the grass snake (Natrix natrix) and dice snake (Natrix tessellata).

Amphibians – This habitat is suitable for frogs most of all: common toad (Bufo bufo), European green toad (Pseudepidalea viridis), European tree frog (Hyla arborea), Greek stream frog (Rana graeca) and marsh frog (Pelophylax ridibundus).

Invertebrates – Due to the presence of water, this habitat is suitable for development of some groups of invertebrates. Common butterflies are: Lycaena tityrus, L. candens, Aglais io, Apatura ilia, Argynnis pandora, A. paphia, Papilio machaon, Celastrina argiolus, Polyommatus icarus, Colias crocea, Pontia edusa and others. Dominant beetle species are: Carabus granulatus, Stenolophus mixtus, Poecilus cupreus, Chlaenius spp., Agonum spp., and there are also some species of dragonflies.

ANTHROPOGENIC HABITATS

Main characteristics – The fields and farmlands in the vicinity of the planned landfill are mainly represented by grain and agricultural cultivations. The most common industry plants are as well as grain plants especially wheat. Monoculture plantations have less biodiversity value than the individually planted fields. The monotypic structure of the communities, and the ecological conditions controlled by humans, using pesticides and fertilizers, dictate the development of biocenose with less species diversity. Contrary to the economic significance of the farmlands, their significance for biodiversity is minor. Some fields are divided with borderlines of different types of bushes and orchards. Most common are: Prunus cerasifera, Pyrus spp., Populus spp., Robinia pseudoacacia and Juglans regia.

Distribution in the project area – The fields and acres doesn’t appear along the project area ,they can be found in the vicinity, about 2,5 km away (see Habitat Map)

Fungi – Some fungi are characteristic of different types of agricultural land as: Agaricus hortensis, Coprinus spp., Anelaria semiovata, Volvariella spp. etc.

Mammals – Driven by the food supply, fields and acres are home for many mammal species such as: lesser white‐toaded shrew (Crocidura suaeveolens), common mole (Talpa europea), sibling vole (Microtus rossiaemeridionalis), guenther's vole (Microtus guentheri), lesser mole rat (Nanospalax leucodon), wood mouse (Apodemus sylvaticus), striped field mouse (Apodemus agrarius), fat dormaous (Glis glis), black rat (Rattus rattus), Balkan short‐tailed mouse (Mus macedonicus), brown hare (Lepus europeus), wolf (Canis lupus), fox (Vulpes vulpes), weasel (Mustela nivalis), badger (Meles meles), wild boar (Sus scrofa) and roe deer (Capreolus capreolus).

Birds – Common species are: Melanocorypha calandra, Galerida cristata, Oenanthe oenanthe, and various other species use them for nesting.

Reptiles – This habitat is rich with species similar like in the deserted farmlands.

Amphibians – Toads (Bufo bufo, B.viridis) are most common species of amphibians in this habitat.

Invertebrates – Common butterflies are: Gonepteryx rhamni, Nymphalis antiopa, Vanessa atalanta, Colias crocea, and beetles: Amara aenea, Brachinus explodens, Harpalus affinis, H. rufipes, Chlaenius vestitus, Pterostichus niger, P. nigrita, Carabus coriaceus, Bembidion lampros, Cicindela campestris.

Main characteristics – Conifer tree plantations are represented mostly by Cypress trees (Cupressus sempervirens, Cupresus arizonica etc) and Black pine (Pinus nigra) . Cypress and Black pine tree are very well acclimatised to the soil and climate conditions in the region under consideration, and it is very often used in plantations. It is planted mainly on the southern slopes of the hills to prevent soil erosion.

Distribution – The plantations of this type are very common in the Republic of Macedonia

Distribution in the project area – Tree plantations mostly of Cupressus arizonica are distributed at the northern parts, behind the asphalted road.

Fungia – The black pine plantations are characterised by the presence of some lignicolous fungi that are specific to different pine species, such are Meruliopsis taxicola, Peniophora pini, Phellinus pini etc. This biotope is characterised by the occurence of mycorrhizal terricolous fungal species, connected to the pine root systems. The most characteristic are Suilus granulatus, Suilus luteus, Lactarius deliciosus etc.

Mammals – Pine marten (Martes martes) and the rock mouse (Apodemus mystacinus) are the most typical species registered in the pine plantations. Presence of red squirrel (Sciurus vulgais) is also expected.

Birds – The size and structure of the Black pine forests in the area do not allow permanent presence of some specific bird species for coniferous forests. Thus, the fauna of birds originates from the neighbouring forests.

Reptiles and amphibians – There are no typical representatives from amphibians and reptilians in this habitat. The species from these classes are the same as the neighboring habitats.

Butterflies – Often recorded butterflies in conifer trees are: Kirinia roxelana, Hipparchia statilinus and H. syriaca. However, species characteristic for woodland clearings can be also spotted in this habitat: Coenonympha arcania, Pyronia tithonus, Pararge aegeria, Pyrgus alveus, Anthocharis cardamines, Satyrium spini, Vanessa atalanta, Phengaris arion.

URBAN AND URBANISED AREAS AS HABITATS

Main characteristics – Village Meckuevci which is 3 km away isn’t characterized by rural features, because there are no residents it’s abandoned. In the past the houses in this village were surrounded by small gardens and fruit trees even in their central part. In such conditions, many wild animal species are adapted to living close to human presence. Meckuevci is characterized by sparsely distributed small number of houses with small meadows, grasslands and sparse trees around. The participation of natural vegetation is high. Becouse there is noone to meintain the gardens, vegetation is primarily represented by elements from the neighbouring biotopes and ruderal and weed species.

Distribution in the project area – No settlements in the radius of 3km of the proposed landfill. Closest village Meckuevci (no residents) is 3 km away from the location toward Sveti Nikole.

Mammals – Villages are suitable mammal habitats. Most common species are: lesser white‐toaded shrew (Crocidura suaeveolans), red squirrel (Sciurus vulgaris), yellow‐necked mouse (Apodemus flavicollis), wood mouse (Apodemus sylvaticus), striped field mouse (Apodemus agrarius), fat dormaous (Glis glis), black rat (Rattus rattus), western house mouse (Mus domesticus) etc.

Birds – The most common are: Black‐billed Magpie Pica pica, Eurasian Jackdaw Corvus monedula, Hooded Crow Corvus cornix, Common Raven Corvus corax, House Sparrow Passer domesticus, Eurasian Tree Sparrow Passer montanus, White Stork Ciconia ciconia, Common Kestrel Falco tinnunculus, Rock Pigeon Columba livia, Eurasian Collared Dove Streptopelia decaocto, Barn Owl Tyto alba, Common Scops Owl Otus scops, Little Owl Athene noctua, Long‐eared Owl Asio otus, Barn Swallow Hirundo rustica, Red‐rumped Swallow Hirundo daurica, Northern House Martin Delichon urbica, Blackcap Sylvia atricapilla, Blue Tit Parus caeruleus, Great Tit Parus major, Eurasian Golden Oriole Oriolus oriolus and Spanish Sparrow Passer hispaniolensis.

Reptiles and amphibians – Amphibian species that could be found around rural settlements  are: Common Newt (Lissotriton vulgaris), Agile Frog (Rana dalmatina), Yellow Belly Toad (Bombina variegata), Common Toad (Bufo bufo), Green Toad (Pseudepidalea viridis), European Tree Frog (Hyla arborea) and Marsh Frog (Pelophylax ridibundus). These habitats are preferred by the reptiles as well. Seven species of reptiles are present such as Hermanns Tortoise (Eurotestudo hermanni), Slow Worm (Anguis fragilis), Erhard’s Wall Lizard (Lacerta erhardii), Wall Lizard (Podarcis muralis), Balkan Green Lizard (Lacerta trilineata), Green Lizard (Lacerta viridis), Whip Snake (Dolochopis caspius), Aesculapian snake (Zamenis longissimus) and Nose‐horned Viper (Vipera ammodytes).

Butterflies: Rural areas contain diverse butterfly fauna due to the natural environment around them and the unique features like the ruderal sites, which are common gathering places for butterflies. Typical and frequent dwellers in this habitat are: Lycaena virgaureae, Lycaena tityrus, Polyommatus belargus, P. icarus, Leptidea sinapis, Plebeius argus, Pieris brassicae, P. napi, Coenonympha papmphilus, C. arcania, Maniola jurtina, Argynnis paphia, Satyrium acaciae, Colias crocea, Arethusana arethusa, Nymphalis polychloros, Erebia medusa, Vanessa cardui, V. atalanta, Cupido osiris, Erynnis tages, Polygonia c‐album, Pseudophilotes vicrama, Hamearis lucina, Pyrgus alveus, Aglais urticae, Aporia crategi etc.

Ground beetles: It is interesting that the fauna of the villages is very similar to the one of the agricultural land (orchard, vineyards, fields and acres). All of these species are connected to human activities. The most common are: Amara aenea, Calathus melanocephalus, Calathus fuscipes, Microlestes fissuralis, Harpalus serripes, Harpalus rufipes and Poecilus cupreus.

This includes a variety of industrial facilities, glasshouses for early crops, dumps etc. Such facilities in the vicinity of project area are sheepfold (see Annex 3 - Habitat Map). However, these structures do not have greater relevance from biodiversity point of view.

At a distance of 1,2 to 1,5 kilometers from the CWMF site, there are three livestock facilities with over 3,500 sheep’s (this numeric  data is not verified).

## Valorisation

### Habitats

In the project area and its surrounding 10 types of habitat can be distinguish. Among them as the most important habitat are hill pastures. Almost  all of the habitats in  the  studied  area  are  grasslands  represented  by  the  category  “hill  pastures”,  which  is  characteristic of the hilly areas in all valleys and plateaus in Macedonia. As a secondary formation, hill pastures are surrounded by sparse vegetation of different degradation stages. Hill pastures with sparse vegetation are another habitat, and in the region of the landfill it is of very similar vegetation composition to that of typical hill pastures.

Grasslands (pseudo steppe) are listed as a priority habitat in Annex I of the Habitats Directive. There is no steppe vegetation in Macedonia (Micevski 1967), but northern boundary of several steppe plant species passes through the central part of Macedonia. Micevski (1967) named the grass vegetation in this region “steppe like” vegetation. In the area, this habitat is represented in a small space on the south slopes (see Habitat Map in Annex 3). Nevertheless; this is not a real steppe. It cannot be said that this part of Macedonia is a primary steppe, but only a steppe-like landscape which is a result of anthropogenic influences. The grasslands in Meckuevci don’t have characteristic halophytic vegetation.

These are considered priority biodiversity features as they are listed in Habitats Directive (Annex I: 92A0) Salix alba and Populus alba galleries. They are also listed in the Bern Convention. Well-preserved forests are not characteristic for this area because it is mostly dry no surface waters is present here. The location has natural gully that can collect water after rains and in the rainy season so this community can be found in fragments along the natural gully.

### Flora

The following plant species listed as present for part of the route are of particular interest:

- Species which are on the IUCN Global Red List of threatened species; none are known to be present in the Study Area;

- Species which are on the CORINE European List: Silene vulgaris (bladder campion) which is very common in Macedonia;

- IPA species in area: none are known to be present in the Study Area, only in vicinity ; and

- Macedonian endemic species present in space around the route: none are known to be present in the Study Area.

According to the literature review in this region can be found important plants as they are representatives of the steppe-like vegetation. They are: Astragalus parnassi, Convolvulus holosericeus, Morina persica, Onobrychis hypargyrea and Potentilla tridentulа. Among them only Morina persica was detected in the study area which is not rare plant species in Macedonia.

### Fauna

As it was said before target taxa were mammals and birds.

Mammals

The valorization of the mammals was made according to several international conventions and directives, which have been ratified by the Republic of Macedonia such as: the world Red list, the annexes IIb and IVb from the Directive for habitats, the CORINE list of species, the Bon convention for protecting the migratory species of wild animals and the Bern convention for protecting the European life world and the natural habitats.

Within the study area Meckuevci-Arbasanci, about 22 species of mammals could be found. According to the IUCN World red list, 16 species belong to the category of the least affected (LC), the bats (Pipistrellus pipistrellus, Rhinolophus blasii and Rhinolophus ferrumequinum) of the category close affected (NT), and only the dwarfish bat (Rhinolophus euryale) is a vulnerable species (VU). In the annexes II and III of the Bern convention, seventeen (17) species have been included.

A special attention should be paid to the occurrence of the Macedonian mouse, and the stony field mouse, the wild cat and several species of bats. A similar conclusion may be also made from the analysis of the species represented in the Directive for Habitats. All the species of bats present a part of the annexes of the Bon convention or of the Directive for habitats.the bats haven’t been included within the description of the habitats because of a lack of concrete data about their distribution. It is important to point out that they occur in almost all habitats in the area , searching for food.

Signs of mammal’s

Dung, feeding signs, footprints, burrows and dens were noted as evidence of the presence of mammals in the project area.

This little trails (fig.16) are very specific to Southern vole - Microtus rossiaemeridionalis. Voles create tunnels under and through the grass. This none is exposed and disappears into the opening of the tunnel. This animal is very active in winter at daylight, digging many short, shallow burrows.

Figure 16. Trails of Southern vole

In the next pictures we see the vole’s underground nests of grass, stems, and leaves, as it could be seen bellow (fig.17 and 18).

Figure 17.  Vole’s underground nests

Figure 18. Vole’s underground nests in southern part of the project area

Fox scats are another very prolific and noticeable clue. Because part of their function is to mark territory, they are often deposited in the showiest of places: like in this case vertical on grass (figure 19). Their color varies from shades of grey to black, but it is their shape which is their most distinctive feature.

Figure 19. Fox scats (probably)                                                  Figure 20. Roe deer droppings (probably)

On the figure 20, roe deer (Carpeolus carpeolus) droppings are showed. Deer scat is usually small, round pellets, similar to those produced by rabbits, goats and others. Depending on what the deer is eating and its size, the scat could also be a bit clumpier and more stuck together. Diet can also affect the hardness of the poop, and its color, which can range from light brown to black. Additionally, deer droppings usually have a slight indent on one end and a bit of a point on the other like on the picture. The droppings were found in an open area, which may be a sign that that place is a feeding area of a deer. Based on the black color and wet and shiny look, droppings were relatively fresh, left within the last 6 to 12 hours. This statement was confirmed by a hunter who said that he saw two roe dears in the surrounding.

Birds

In total, 88 species of birds were recorded included literature data. Dominating birds belongs to the order Passeriformes, which represents 62% of the total number of recorded species. The results are presented in Annex 2.1. During the monitoring (bird survey), presented in chapter 2.2.5, 36 bird species were observed and verified; these species are presented in Annex 2.3.

In accordance with EU Birds Directive 2009/147/EC, there are 9 species of birds that require special protection of the habitat, as indicated in Annex 2.2.

Convention on the Conservation of European Wildlife and Natural Habitats (Bern Convention) imposes strict legal protection (according to Appendix II: Strictly protected species) for all 9 species already protected by the Wild Birds Directive by extending the list of one additional species.

Bonn Convention recognizes that endangered migratory species can adequately protect if appropriate action is taken to prevent, eliminate, minimize or compensate the adverse effects of activities or obstacles that seriously impair or prevent the migration of species.

In terms of the conservation status prescribed by IUCN Red List of endangered species (Europe) there are 6 species classified as vulnerable (category VU).

Because of the characteristic habitat on the selected location for CWMF and its surroundings, proposed as Important Bird Area (IBA), as well as the presence of birds with a higher degree of protection, preservation of the surrounding areas identified as important for feeding and breeding, should be treated with the highest importance.

Bird species with a higher degree of protection under the IUCN Red List, EU Birds Directive 2009/147/EC, Bern and Bonn Convention (ratified in Macedonia) are presented in Annex 2.2..

Part of the species presented in Annex 2.2 globally have a lower level of protection because of the increased populations in some parts of Europe, but unfortunately if there was a red list of endangered species in Macedonia, they would be at the top of the list. Such is the case with kestrel (Falco naumanni) and Imperial Eagle (Aquila heliaca).

Below in table 1 are presented the most important bird species in the area.

Table 1  The most important bird species in the area

| Aquila heliaca | This species has a small population globally, in a continuous decline, primarily due to the loss and degradation of habitat, mortality of adults through persecution and collision with power lines, robbing the nests and lack of prey (Birdlife international). According to IUCN this species is categorized as Vulnerable (Vulnerable). The growing population of imperial eagles in Central Europe (Hungary, Czech Republic, and Slovakia) is a result of a strong conservation measures undertaken. This contributed at European level, this species to be classified as Near Threatened. Although the population of Imperial Eagles in Macedonia seems stable or only slightly declining in recent years, however, their population has history of declining- since the 80s of the 20th century to today. On the territory of Macedonia, imperial eagles are facing poaching, electrocution and poisoning, drying or cutting the stems of the nest, and regular harassment The Imperial eagle prefers rolling terrains and plains where forested areas or groups of trees alternate with open spaces such as pastures, agricultural lands and fallow lands. The Imperial eagle nests on single or in groups of tall trees growing alongside rivers, frequently in the immediate proximity to settlements, roads and arable lands. In the past, the species had nested in orchards as well. Its preferred altitude varies between 25 and 1,230 m. The Imperial eagle is a monogamous species. The pair is strongly attached to its nest and some nesting areas have been occupied by Imperial eagles for many years. The minimal distance (established using GPS) between the nests of various pairs is 4,700 m (confirmed on a current location of nests at Figure 22). The mating displays begin in February and are most intensive in March. Both birds participate in construction of the nest, using dry branches. The home of the eagle reaches 1.2 to 2.2 meters. It is 30 to 90 cm thick and can weigh as much as 200 kilos. Often, the pairs have more than one nest, using one and keeping the others as spare. The main trees used for nesting are hybrid poplars (Populus sp.) followed by various oak species (Quercus frainetto, Quercus pubescens, Quercus cerris, Quercus petraea). Less often, the nests are built on Scots pine (Pinus silvestris), European beech (Fagus sylvatica) and gumarabic acacia (Robinia pseudoacacia). Usually, the female lays 2 eggs, or, less often, 1 or 3 eggs, in the second half of March or in early April. Nests with 4 eggs have been known, although they are exceptional. Mainly the female sits on the eggs, replaced by the male to feed, or having food brought to the nest by the male. The incubation period is around 43 days long. The young are fed mainly by the female, while the male is responsible for delivering the food. The young leave the nest after the second half of July and early August. For some time they return to the nest to spend the night or stay near-by, still fed and trained to hunt by their parents. The birds remain in their nest region until the second half of September - the end of October. After that, the young birds migrate, usually reaching as far as Turkey, Israel, or Syria. One of the juvenile birds with a satellite transmitter was followed during the project as far as the Sudan. The adult birds stay and winter here permanently, because they are experienced hunters, unlike the juveniles, and are capable of feeding even during the cold months. The Imperial eagle hunts in open areas such as pastures, meadows, and bare hills. Most often, the bird surveys, for hours in some cases, its hunting territory from a suitable observation point. It swoops on any prey flying precipitously low. In areas with abundance of sousliks, the eagles fly low, several centimeters above the area most densely occupied by these rodents. In some cases the Imperial eagles may be observed perching next to the entrances toward the shelters of their prey. Their population in the region of Ovce Pole has been reduced (from 25 couples in 1990, to 13-15 pairs in 2015). Due to the optimal conditions for the survival of this species in Ovcepolska Valley, despite breeding couples during the winter can be recorded and individuals that come even from the Pannonian Plain (Status of Birds in Macedonia, Metodi Velevski and others; Macedonian Ecological Society, 2012/2013). The imperial eagles are breeding species in Ovce pole. Satellite tracking of three young birds in 2013 was fitted with transmitters in Ovce pole region (monitored continuously since 2013 by the MME-Hungary and NCA - Aquila), shows that all of them use the region of Ovce Pole as a hunting ground throughout the year. About the location of the CWMF at a distance of three to seven kilometers there are five breeding territories of Imperial eagles, and all of these couples use as hill pasture grounds and food source. Please find more details regarding the important habitats within the Project area in Chapter 4.4, below. |
| --- | --- |
| Falco naumanni | Kestrel is a migratory species that nests in Europe, central Asia and winters in Africa. They are kind of small falcon that primarily makes its nests in colonies under the eaves and ceilings of houses. Basic food for Kestrel are insects, but also small birds, reptiles and rodents (especially mice), which often hunts height of land. Recent studies suggest that their populations are stable or show slightly positive trend of the overall population over the last three generations. Consequently, the qualification of Kestrel has been changed, and from VU (Vulnerable) now it is qualified as LC (Least Concern) type, because it is not close to any of the thresholds for vulnerable species according to IUCN criteria. The discovery of numerous populations of steppe Kestrels Falco naumanni especially in Ovcepole, Vardar and Pelagonia in the census in 2002 (FWFF-Macedonia, unpublished data), unfortunately, did not last long. Repeat census in 2012 (MED-CAP L'Aquila), showed a drastic decline in their numbers in Pelagonia (59%, from about 850 to 350 couples) and a larger (90-95%) in Ovce pole (from 230-300 to 20-30 pairs). Along with the observed decline in Vardar region, it’s been assessed that the overall population has declined by over 70% in just 10 years, which makes this species endangered in Macedonia. The reasons for the drastic decline in population are potential nest abandonment in many villages, loss of pastures and excessive use of pesticides in agriculture leading to loss of their’s source of food. Please find more details regarding the important habitats within the Project area in Chapter 4.4, below. |
| Alectoris graeca | The rock partridge (Alectoris graeca) is a gamebird in the pheasant family, Phasianidae, of the order Galliformes (gallinaceous birds). It is widespread in southwest Asia and south eastern Europe. Partridge has a round shape and colours it is similar to the colours of the stones. Mostly grey, with light brown back, grey breast and cream-colored belly. Her face is white with black lines, and the wings have square patterns. Her feet are red. When upset, she rather run than fly, but if necessary it will fly short distances. Has a length of 36 cm and span of its wings can reach 60 cm. This is a resident breeder in dry, open and often hilly country. It nests in a scantily lined ground scrape laying 5-21 eggs. The rock partridge takes a wide variety of seeds and some insect food. This species is declining in parts of its range due to habitat loss and especially excessive hunting. The rock partridge is found northwest of the selected location at a distance of 1 to 3 km. The partridge is also an ideal source of food for the Imperial eagles during the winter period. |
| Circus cianeus | The hen harrier (Circus cyaneus) or northern harrier is a bird of prey. The male is mainly grey above and white below except for the upper breast, which is grey like the upperparts, and the rump, which is white; the wings are grey with black wingtips. The female is brown above with white upper tail coverts, hence females, and the similar juveniles, are often called "ringtails". Their underparts are buff streaked with brown. This medium-sized raptor breedson moorland, bogs, prairies, farmland coastal prairies, marshes, grasslands, swamps and other assorted open areas is typical of prey that hunts with long wings in a shallow V position and low flight. Mostly hunt small mammals and birds. The diet occasionally includes amphibians, especially frogs and reptiles, insects and more. Hen harrier hunts with flying very low to the ground in open spaces, floating over the fields and hills. Unlike many of prey, the hen Harrier, historically favored by farmers because they eat potential thieves of eggs, mice and other pests. |
| Coracias garrulus | The European roller is big bird, 29-32 cm in length with a wingspan of 52-58 cm. It is predominantly blue and orange-brown back. The European roller often sit prominently on trees, poles and overhead wires, thus requiring large insects, small reptiles, rodents and frogs to eat. It is a bird of warm, dry, open country with scattered trees, preferring low land and open countryside with patches of oak Quercus forest, mature pine Pinus woodland with heathery clearings, orchards, mixed farmland, river valleys, and plains with scattered thorny or leafy trees. It nests in holes in trees, river banks or steep hills. The hole in the ground digging themselves and it is half a meter deep in the expansion of the bottom, where it placed the nest. The European roller has big global population. However, in the period 1970-1990, the number began to decline, especially in Europe. For three generations (15 years) the decline exceeded 30%. In Russia it disappeared from the northern parts. The use of pesticides, and changing agricultural and forestry practices affect these birds or reduce their availability to find food. For these and other reasons, its status at least threatened to change in almost an endangered species. Please find more details on this species concerning the Project area in Chapter 4.4, below. |
| Streptopelia turtur | The turtle dove is a migratory species is a member of the bird family Columbidae. It inhabits Europe and western Asia and winters in Africa. Turtule dove is long 24-30 cm, wingspan 47-55cm. The migration begins in late September and returns in the second half of April. According to a report on the state of European birds from 2007, the population of wild turtledoves in Europe decreased by 62% between 1980 and 2005. Partial reason for such decrease in the population is changing agricultural practices for which has come to reduce the number of plants that feed and excessive hunting of birds during migration through the Mediterranean areas (Birdlife international). Because of this it is classified as Vulnerable (Vulnerable) species in the red list of Europe. It is a bird of open rather than dense woodlands, and frequently feeds on the ground. It will occasionally nest in large gardens, but is usually extremely timid, probably due to the heavy hunting pressure it faces during migration. The flight is often described as arrowy, but is not remarkably swift. Water habitats are also necessary for dove. So in the summer days are observed larger clusters of turtledoves along river valleys. The surroundings of the selected CWMF is one of the few forested areas in Ovce Pole region with present surface water sources and as such is suitable for resting and nesting place for turtle doves. South and west of the CWMF spreads large fields planted with cereal plants which represent source of food for this species. |
| Ciconia ciconia | The white stork (Ciconia ciconia) is a large bird in the stork family Ciconiidae. Its plumage is mainly white, with black on its wings. Adults have long red legs and long pointed red beaks, and measure on average 100–115 cm (39–45 in) from beak tip to end of tail, with a 155–215 cm (61–85 in) wingspan. White storks consume a wide variety of animal prey. They prefer to forage in meadows that are within roughly 5 km of their nest and sites where the vegetation is shorter so that their prey is more accessible Their diet varies according to season, locality and prey availability. Common food items include insects (primarily beetles, grasshoppers, locusts and crickets), earthworms, reptiles, amphibians, particularly frog species such as the commonfrog as well small mammals such as voles, moles and shrews. It lives in swamps, wetlands and floodplain areas to rivers and lakes, and the nests except the trees, often shrinking them in neighborhoods, the roofs of houses or power lines. Great danger for this species and for all wetland birds, a drying of wetlands and high use of pesticides. Because of population stable or rose, storks are categorized as Least Concern species with (Least Concern). The population of storks in Macedonia probably has a slight rise, if one can judge by increasing the number in the largest sub-populations (Pelagonia) in the period 2002-2015 year. The number of breeding couples increased from 220 to 320 (an increase of 31%), which are mainly concentrated around the wet meadows in the north and central part of the Pelagonia Valley (unpublished data from the census data storks MES, etc.). Stork's most recognizable symbol of nature in human settlements and people often independently take care of its protection. |
| Vanellus vanellus | It is migratory, which winters in northern Africa, northern India, Pakistan, and parts of China. When moving flies by day, often in large flocks. The nun is 28-33 cm long, with a wingspan of 67-87 cm and weighing 128 to 330 grams. It has rounded wings and a thin, long crest (chair) of the head. Her feet are the shortest of its kind. Color is black and white, but her back is greenish glow. The male has a long (crest) and black crown, throat and chest that are in contrast with the white face. Females and young have smaller crest and less distinctive face and the rest of the body are similar to males. When flying, heard the clear sound of moving wings, as if defending him from attack. The nun is a territorial bird that actively defends the territory of nesting and feeding. It is active at night, and defends the nest from all animals deemed to threaten. They feed on wet meadows and arable land, and in shallow waters. Eat insects and other small invertebrates. Because of the natural habitat of the nun, which is usually a field, meadow, pasture, etc.. and which has recently been burdened with the progress of civilization and the use of various pesticides, the number of these birds is reduced.For this bird Agreement for protection of Africa and Asia migratory water birds is applicable. |
| Lanius excubitor | The great grey shrike, northern grey shrike, or northern shrike (Lanius excubitor) is a large songbird species in the shrike family An adult great grey shrike is a medium-sized passerine about as large as a big thrush, measuring from 22 to 26 cm (8.7 to 10.2 in) long. It typically weighs around 60 to 70 g. The general color of the upperparts is pearl grey, tinged brownish towards the east of its Eurasian range. The cheeks and chin as well as a thin and often hard-to-see stripe above the eye are white, and a deep black mask extends from the beak through the eye to the ear coverts; the area immediately above the beak is grey. The scapulars (shoulder feathers) are white, and the wings are black with a white bar made up by the bases of the primary remiges, continuing slightly offset onto the bases of the secondary remiges in some regions. The tail is black, long, and pointed at the tip; the outer retraces have white outer vanes. The preferred habitat is generally open grassland, perhaps with shrubs interspersed, and adjacent lookout points. These are normally trees – at forest edges in much of the habitat, but single trees or small stands at the taiga-tundra border. In steppe, it will utilize any isolated perch, be it fence posts, power lines or rocks. In general, some 5–15 perching sites per hectare habitat seem to be required. It avoids low grassland with no lookouts and nesting opportunities (trees or large shrubs), as well as dense forest with no hunting ground. Apart from grassland, the birds will utilize a variety of hunting habitats, including bogs, clearings or non-industrially farmed fields. Breeding birds appear to have different microhabitat desires, but little detail is known yet. Eats small vertebrates and large invertebrates. To catch this bird boast the highest branch of the tree, telegraph pole or similar prominent places. Sometimes the fly and hunt small birds, usually approaching their bottom or side and hooking them on foot with his beak. His legs aren’t adapted for tearing the prey, so it prickles the prey on thorns, sharp branch or barbed wire. Great grey shrike recently becomes extinct in Switzerland and the Czech Republic. Overall, in Europe the number is declining. |

### 4.3.4 Ecosystem function of habitats

The principal ecosystem functions (defined as biological system properties or processes of ecosystems) include primary productivity, nutrient cycling, and decomposition. Many of these ecosystem functions provide ecosystem services of value to humans (Table 1; Costanza et al. 1997). Although we might be focused primarily on productive output in managed grasslands, the ecosystem goods and services provided by grazing ecosystems must be considered as well. As the grasslands are dominant habitat in project area (fig.21), their functions or ecological importance is described below. Grasslands, mixture of grass, clover and other leguminous species, dicotyledonous, herbs and shrubs, contribute to a high degree to the struggle against erosion and to the regularizing of water regimes, to the purification of fertilizers and pesticides and to biodiversity and they have aesthetic role and recreational function. But even for grassland it is very difficult to create a good frame for its different tasks the provision of forage for livestock protection and conservation of soil and water resources, furnishing a habitat for wildlife, both flora and fauna and contribution to the attractiveness of the landscape. Nevertheless it is the only crop able to fulfill so many tasks and to fit so many requirements. The area provides some ecosystem services of value to the local communities mostly for collection of Medicinal and aromatic plants. The most common medicinal and aromatic plants in the area are: Achillea millefolium, Althаea officinalis, Chamomilla recutita, Crataegus monogyna, Cornus mas, Equisetum arvense, Rosa canina, Hypericum perforatum, Malva silvestris, Onionis spinosa, Plantago lanceolata, Plantago major etc.

Figure 21.  Pasture

## Significant habitat areas and affected species

As stated in this document, beside site survey, literature data on the region were used obtained in previous research projects conducted by the authors of the EBS.

From 2003 to 2008 monitoring were carried out throughout the year with a frequency of at least once a week covering the location of CWMF and its surrounding. During the monitoring, data on the complete fauna of birds in the area were collected. Also, since 2004 until now (2017), a project has been implemented for conservation of Imperial eagles (Aquila heliaca) in Republic of Macedonia. Therefore, it can be stated that literature data are supported with the performed bird monitoring as indicated in the Chapter 2.2.5 as well with data from previously performed monitoring on the location of CWMF and its surrounding.

As per obtained monitoring data during these years, in the vicinity of the selected location for CWMF, at a distance of 3 km to 7 km, there are 5 active nesting couples of Aquila heliaca (please see the figure 22 and table, below). Also, affection of the nearby proposed IBA sites has been determined in regards of the nests location.

| Nests of Aquila heliaca | Distance from nests to location of CWMF | Affected proposed IBA area |
| --- | --- | --- |
| AH23 | 5.3 km | MK019 Ovce Pole |
| AH24 | 3.6 km | MK019 Ovce Pole |
| AH25 | 5.1 km | MK015 Preod - Gjugjance |
| AH26 | 5 km | MK012 River Zletovska Valley |
| AH29 | 6.3 km | MK019 Ovce Pole |

* GPS coordinates will be available to the Designer and Operator due to safety concerns

Figure 22.  Locations of active nests of Imperial eagle (Aquila heliaca) near the CWMF  – E. Lisichanets

As per studies obtained in the last 10 years, the basic food for Aquila heliaca in Macedonia is: hedgehog, turtles, snakes, rabbits, partridges, some smaller rodents and mammals (young foxes in some cases) etc.

Accordingly, the habitats important for hunting ate the ones are grassy surfaces through which are spread scattered shurbs and trees i.e. Habitat type 4: Hill pastures with sparse shrubs presented in Chapter 4.2., Habitat map in Annex 3, in MIT-O-06 and on the photo below.  It can be stated that beside on location for CWMF, these habitats are also widely spread on the surrounding areas (see MIT-O-06).

Figure 23.  Hill pastures affected by CWMF

To make the prey easier to access, for these bird species it is important surfaces covered with vegetation with lower height. Such areas are pastures where sheep and other large cattle are harvested.

During the rearing of the young, the Imperial eagles hunting grounds is within a radius of 5 kilometers (sometimes more) around the nest, while in the rest period of year the hunting radius increases depending on the food availability. According to the above presented, in the surrounding of the CWMF (3 - 7 km), there are five active nests of Aquila heliaca in a hunting distance, which means that some of the couples may be using the CWMF territory as hunting ground.

During the census of the Falco naumanni (conducted in 2002 and 2012), in the villages located in vicinity of the selected CWMF (Meckuevci, Ranchinci, Kukovo, Patetino etc.) were found nesting colonies of the Falco naumanni as presented in the table an photo below.

|  | Year of conducted census |  |  |
| --- | --- | --- | --- |
| Location | 2002 | 2012 | Proposed IBA site |
| Meckuevci (2 km from CWMF) | 19 | 7 | MK019 Ovce Pole |
| Rancinci (3 km from CWMF) | 23 | 5 | Near the border line of MK019 Ovce Pole |
| Kukovo (2.5 km from CWMF) | 36 | 8 | Near the border line of MK012 River Zletovska Valley |
| Patetino (4.5 km from CWMF) | 29 | 3 | MK012 River Zletovska Valley |

Figure 24.  Villages in surrounding of CWMF where nesting colonies of Falco naumanni can be found

Decline of the population of Falco naumanni in the region of Ovce Pole (including the surrounding location of CWMF) is obvious as per obtained data, so it is important their conservation to be threated appropriately. Breeding areas for Falco naumanni are manly, but not limited, villages i.e. Habitat type 9: Rural settlement - village presented in Chapter 4.2.

The most suitable areas for finding food are grasslands in the surroundings of the nests. Such as Aquila heliaca, for Falco naumanni species it is important surfaces covered with vegetation with lower height.

Pretty similar hunting territories as Falco naumanni are used by Coracias garrulus (European roller).

# 5. Impact of CWMF on biodiversity

## 5.1 Overview of CWMF elements

This section provides an assessment of the potential positive and negative impacts on flora and fauna resulting from the development of Central Waste Management Facility Meckuevci – Arbasanci identified for construction, operational and post-operational phase.

The Central Waste Management Facility includes the following main elements and equipment:

- Materials Recycling Facility

  - Construction of building, Electrical works, Mechanical works, Fire protection, fencing, security, loading and unloading docks and platforms etc.;

  - Utilities and connections (toilets and hygienic facilities, sewerage, water and power supply etc.);

  - Waste handling equipment (loader, tractor, shredder, separators, conveyer belts and lines, bailer etc.).

- Waste Treatment Facility

  - Construction of platforms and buildings, Electrical works, Mechanical works, Fire protection, fencing, security, loading and unloading docks and platforms;

  - Utilities and connections (toilets and hygienic facilities, sewerage, water and power supply;

  - Waste handling equipment (loader, shredder, turning machine, conveyer belts and lines, membranes, etc.).

- Landfill

  - Landfill cell;

  - Internal roads;

  - Leachate collection and treatment system;

  - Construction of buildings (Offices, maintenance, etc.);

  - Electrical works, Mechanical works, Fire protection, fencing, security, weighbridge, etc.);

  - Monitoring system;

  - Utilities and connections (toilets and hygienic facilities,  sewerage, water and power supply);

  - Waste handling equipment (Waste compactor, waste loader, Bulldozer, etc.);

  - Waste inspection materials and tools;

  - Office and office equipment;

  - PPE and other protective equipment.

The operational phase will include the following main elements:

- Personnel;

- Fuel;

- Energy;

- Maintenance;

- Consumables;

- Insurance;

- Monitoring;

- Wastewater management;

- Administrative cost.

## 5.2 Impact on flora and fauna

The identified impacts on flora and fauna can be rated within the following scale:

- Zero impact (there is not any impact on flora or fauna)

- Minor to moderate impact (there is a minor or moderate impact on flora, fauna or habitat, with usage of appropriate mitigation measures the impact will be reduced or avoided)

- Significant impact (the impact can lead to damages on habitats, flora or fauna; usage of mitigation measures, alternatives and/or compensation measures shall be imperative)

- Critical impact (this impact will likely cause an irreversible damage on flora or fauna, such activities/Projects should be avoided within the affected area)

- Positive impact (such operations will likely lead to positive impact on some segment of biodiversity)

### 5.2.1 Construction phase

Construction activities usually generate variety of impacts. Site preparation, earthworks, grading, construction of access roads and incorporation/construction of installation units is likely to lead to:

- IMP-C-01 emission of dust and other air pollutants

- IMP-C-02 noise and vibration

- IMP-C-03 vegetation removal and degradation of habitats; loss of conservation value

- IMP-C-04 disturbance and hunting of fauna by workers

- IMP-C-05 potential pollution releases to soil, surface water and groundwater

- IMP-C-06 generation of hazardous and non-hazardous

Earthworks are the main source of dust emissions while emission of other air pollutants (SO2, NOx, CO, CO2) is inevitable during the operation of construction mechanization and vehicles. Regarding emissions (dust) from the construction activities, a potential impact on the surrounding habitats can be occurred.

Noise and vibration will be generated by the machinery during the construction activities.  It is assumed that the landfill construction will be limited to daytime (7am to 6pm) periods. The most significant period of construction noise generation will be noted during the initial site infrastructure works. Following this initial phase, the construction activities will extend throughout the lifetime of the development, as new cells and phases of the site will be required progressively as the site develops.

Noise and vibration have a direct impact on fauna. With the disturbance, natural comfort is in disorder and it can lead to negative effects of fauna (nesting failure, leaving the territory – loss of habitat etc.). Since there are not breeding habitats of Aquila heliaca or Falco naumanni near the location of CWMF (at least 2 km distance from Falco naumanni breeding area and 3.6km from Aquila helliaca breeding site), this impact is assessed as minor to moderate short-term. Specific mitigation measures are presented in Chapter 6.1 so this impact to be reduced.

The construction of the CWMF Meckuevci - Arbasanci, in regards of the clearing of vegetation leading to degradation of habitat, will have a potentially direct long-term impact on flora and fauna if we consider CWMF will be permanent facility. It represents a “Greenfield” project and because of the characteristic habitat on the selected location for CWMF and its surroundings, proposed as Important Bird Area (IBA). Anyway, main target should be preservation of bird species Aquila heliaca and Falco naumanni i.e. species due to which this area has been proposed as IBA. Intensive pastures represent and ideal hunting grounds for almost all species of birds as well surface water is necessary for survival of all living organisms. Intensive grazing has maintained a low level of grass on pastures and it allows the birds to reach their prey. .

Clearing of existing vegetation during preparatory works will result in a loss of associated ecological habitats and their fauna, within the footprint of the CWMF development (the site and its surroundings, access road). Two land plots estimated for CWMF have area of 38.28 ha (equivalent 382 800 m2). This is the total area so landfill footprint area and buffer zone is still to be determined while design phase. Since this area did not maintain specific protected plant species, affects only 0.08% from total area of proposed IBA where it is identified existence of widely spread similar habitats located in a hunting distance that may serve as a feeding areas for the affected bird species (please see MIT-O-06), this impact will be classified as minor to moderate with recommendation for precaution measures to be achieved during pre-construction, construction and operational phase. Details for precaution measures are provided in Annex 5 of EBS.

Disturbance and hunting of fauna by workers should be treated as one of the possible impact on fauna while construction activities, especially because of conservation value of the area and its surrounding. While one of the performed site visits, it were observed some hunting activities in the area.

Different waste types can be expected to be generated during the works:  hazardous waste (used waste oil from machinery), non-hazardous (wood, paper, glass, plastic) and communal waste. This shall be managed properly in order to do not cause any potential pollution releases to soil, surface water and groundwater.

In the Table 02 below are presented the main impact associated with construction phase of CWMF Meckuevci - Arbasanci included assessment of potential significance.

Table 02 Main biodiversity impacts of constructional phase (IMP-C-XX; Impact-Constructional phase-Number)

| Impact | Receptor | Existence of mitigation measures | Potential significance of impact |
| --- | --- | --- | --- |
| IMP-C-01 Emission of dust and other air pollutants | Flora and fauna | √ | Minor impact |
| IMP-C-02 Noise and vibration | Fauna (birds and mammals) | √ | Minor to moderate impact |
| IMP-C-03 Vegetation removal and degradation of habitats; loss of conservation value; | Direct impact on flora Indirect impact on fauna | √ | Minor to moderate impact |
| IMP-C-04 Disturbance and hunting of fauna by workers | Fauna (birds and mammals especially) | √ | Minor impact |
| IMP-C-05 Potential pollution releases to soil, surface water and groundwater | Flora and fauna | √ | Minor impact |
| IMP-C-06 Generation of hazardous and non-hazardous | Flora and fauna | √ | Minor impact |

### 5.2.2 Operational phase (waste collection, transport, processing, landfilling)

Waste deposition is relatively least troublesome method of its utilization. However, this method is related to biodiversity impacts, among which the most important are as follows:

- IMP-O-01 formation of landfill gas

- IMP-O-02 leachate from the landfill

- IMP-O-03 dust, carried small materials and odour

- IMP-O-04 concentrated presence of rodents and birds

- IMP-O-05 noise due to landfill operation and increased traffic

- IMP-0-06 Habitat loss; loss of conservation value

Formation of landfill gas

The preliminary estimated quantity of produced landfill gas will be in the range from 800 m3/hour (in the first 15 years of operation) to 1,000 m3/hour (after 20 years of operation). As per Vaverková, gas emitted from landfill often contains compounds, which concentration considerably exceeds the concentration of the surrounding environment. Such concentrations may lead to the development of ecosystem with specific organisms. The main components of landfill gas are methane (from 40% to 60%), carbon dioxide (from 35% to 50%), nitrogen (from 0% to 20%), oxygen (from 0% to 1%) and hydrogen sulphide (from 50 to 200 ppm) [Bove and Lunghi, 2006]. Hypothetically, plants (plant communities) in the ecosystem can be assumed to induce emissions and occurrence of polluted areas under the influence of landfill gas. The pollution may be indicated by:

- the development of specific species content and/or external reactions of organisms;

- accumulation of contamination in plants.

Methane is typically burned away using either a flare-up (candle) or an enclosed flare. Flares can burn intermittently or continually, depending on the amount of gas produced. Some intermittent flares are automated to turn on when gas reaches a certain level in the system. Others may be ignited by a switch that produces a spark at short intervals as the gas passively seeps out of the structure. Either system produces a nearly invisible flame of up to 900 degrees Celsius (Wentley 2005). Visible flares (e.g., those yellow in color or producing smoke) have contaminants such as hydrogen sulphide. In most cases, these contaminants are filtered or “scrubbed” out before the gas reaches the burner, resulting in a nearly invisible flame.

Burners located in favorable habitat can be problematic to birds. Birds perched on, or flying near, a stack can be seriously injured or killed when a flare suddenly ignites. In some cases, birds may fly over or even through an almost invisible burner flame. If a bird survives such an encounter, its burned feathers may render it unable to fly. In such situations, the bird is likely to die from starvation, infection, exposure, or predation (Moller 2009, Siftar 2008). The figure 25 below shows some examples of bird injuries cause by the flare (burned foot and beak).

Figure 25. Burned foot (left) and burned beak (right) from landfill flare-up (open method)

(Source: Rocky mountain raptors program)

Leachate and runoff from the landfill

Landfill leachate is the product of water that has percolated through waste deposits that have undergone aerobic and anaerobic microbial decomposition. Landfill leachate contains dissolved constituents of the disposed waste and its degradation products. It is typically very high in nitrogen (ammonium), chloride, potassium, suspended solids, pathogens, dissolved BOD and COD organics.

Anyway, depending of the compounds, leachate and runoff, if discharge in the surrounding area or surface water, can have negative impact on aquatic flora and fauna, riparian flora and terrestrial fauna (including domestic animals) that use the surface water (i.e. for drinking). In the area of CWMF there are no significant surface water bodies on the site or its vicinity, only an intermittent stream named Klobuč, running along the eastern site area, towards south.

So, the proposed landfill shall be equipped with a system for leachate collection and treatment on-site, before its final discharge.

Dust, carried small materials and odour

Dust and odour emissions will be generated during the waste unloading and disposal. They may be a problem especially during the summer months or in case of disposal of dusty or malodours waste. Regarding constant emissions (dust) from the landfill, this will probably have an impact on the above-ground plant parts, especially due to shading, mechanical clogging or covering of stomata what can result in slowing down the photosynthesis, overheating of leaves, adsorption changes and the reflection of heat radiation or mechanical damage of leaf surface. Thus, it can directly affect biomass.

Dust and odour emissions will be generated during the waste unloading and disposal. They may be a problem especially during the summer months or in case of disposal of dusty or malodours waste.

Concentrated presence of rodents and birds

Because landfills provide a continuous supply of food, that makes a perfect place for birds and rodents. As the CWMF Meckuevci - Arbasanci is positioned within the proposed IBA site – MK019 Ovce Pole (close to the northern boundary zone), it is from high importance managing with the landfill affection on birds i.e. proliferation of birds that can have potential impact on other bird species.

In most cases, the most common birds at landfills (far from the coast) are ravens, crows, and magpies. The presence of ravens and crows will sometimes attract some species of eagles.

Crows may cause damage to agricultural crops, such as maize and ground nuts. Some conservationists are concerned that increasing numbers of crows may have a negative effect on raptor populations. They compete for the same food, may usurp or even rob their nests, and sometime aggressively mob eagles, vultures and other birds of prey. Crows are very effective, opportunistic predators of tortoises and birds’ nests. There is growing concern that increasing numbers of crows may cause widespread decreases in reptile and bird populations. Thus, proliferation of birds may have negative impact the bird species as Aquila heliaca, Falco naumanni, Coracias garrulus etc.

Figure 26. Bird species attracted by landfill (Source: Evan Schneider, UN Photo/Flickr)

Also, recent studies show changing in the behavior of some birds such as becoming depended of landfill food or even changing in theirs migratory behavior.

Waste in landfill can and often does provide harborage (living space) and food for rodents. Rodent populations can proliferate if landfills allow garbage and fill areas to remain exposed and undisturbed. With proper landfilling procedures, however, rodent populations can be controlled.

There is some risk to birds of prey if such birds consume poisoned rodents (from the landfill control techniques). The degree of risk is dependent upon the number of poisoned rodents consumed by the predatory bird in a given period of time, the amount of poison consumed by each rodent and the size of the bird, amongst other factors.

Moreover, hazardous/poisoned waste from landfill in some cases can be fatal for birds if they are using as a food.

Noise due to landfill operation and increased traffic

Noise in operational phase is result of the operation of the landfill equipment, mechanization for landfill maintenance as well increased traffic with heavy vehicles (approximately 29 per day) loading and unloading the collected waste. These can lead to impact on behaviour on some fauna species as well disturbances of wildlife and bird nesting leading to certain decrease on the bird population. With usage of appropriate mitigation measures for noise management as well avoidance of traffic routes within significant habitats it is expected minor to moderate impact on fauna, mainly concerning bird fauna. Traffic routes leading to the CWMF are presented on Figure 31.

In general, the highest noise levels expected from the land-filling operations will be approximately equal to the worst-case construction noise.

Habitat loss; Loss of conservation value

As stated before, CWMF Meckueci - Arbasanci is situated within the internationally designated IBA site – MK019 Ovce Pole, so conservation value of the area and its surrounding shall be priority.

In relation to this, 0.08% from the total territory of the proposed IBA site Ovce Pole can be physically affected by construction and operational activities of CWMF. This percentage includes the footprint area and buffer zone so the real affection will be even less.

| Proposed IBA site - MK019 Ovce Pole | CWMF Meckuevci - Arbasanci |
| --- | --- |
| 48183 ha | 38.28 ha |
| Territory of CWMF will affect 0.08 % from the total area of proposed IBA Ovce Pole |  |

Anyway, part of the habitat of CWMF as described in Chapter 4.2, Chapter 4.4 as well on a Habitat map provided in Annex 3, can be classified as a feeding ground for the affected bird species. Breeding sites for Aquila heliaca and Falco naumanni would not be directly affected by the location of CWMF but some part of the hunting/feeding ground identified in the habitat of hill pastures will be directly impacted with the location of CWMF.

The food availability (including density of prey populations) is the predominating successful breeding precondition for Imperial Eagle. On Figure 32 below are presented hill pastures that are in the vicinity of the CWMF which are also in the hunting distance for the determined breeding sites of Aquila heliaca and would not be affected by construction activities.

Because the CWMF is considered as permanent facility so this impact will be transferred during operational phase, please find more explanation on the feeding/hunting areas for Aquila heliaca and Falco naumanni in Chapter 4.4 as well area covered with the same habitat located nearby proposed CWMF in Chapter 6.2 MIT-O-06.

While operational phase some part of the CWMF area will be fenced and as it will act as a barrier of wildlife movement.

In the Table 03 below are summarized main impacts on flora and fauna associated with operational phase of CWMF Meckuevci - Arbasanci included assessment of potential significance.

Table 03 Main biodiversity impacts of operational phase (IMP-O-XX; Impact-Operational phase-Number)

| Impact | Receptor | Existence of mitigation measures | Potential significance of impact |
| --- | --- | --- | --- |
| IMP-O-01 Formation of landfill gas | Flora and fauna Especially Birds (in case of open flare method) | √ | Minor effect |
| IMP-O-02 Leachate from the landfill | Aquatic flora and fauna, riparian flora and terrestrial fauna (including livestock) | √ | Minor effect |
| IMP-O-03 Dust, carried small materials and odour | Flora and fauna Dust sensitive areas | √ | Minor effect |
| IMP-O-04 Concentrated presence of birds | Birds | √ | Minor effects |
| IMP-O-05 Noise due to landfill operation and increased traffic | Fauna | √ | Minor to moderate effect |
| IMP-O-06 Habitat loss; Loss of conservation value | Flora and fauna (birds especially) | √ | Minor to moderate effect |

### 5.2.3 Post-operational phase

Post-operational phase is related to maintenance of environmental pollution control systems installed as part of the landfill (CWMF) closure. Creation of new habitats (post-restoration) as one of the major arguments in favour of land filling is that following completion of infilling activities; the site can be restored and used for recreation and/or very low intensity developments. Although such restoration represents a sizeable proportion of the overall costs of developing sanitary landfill facilities, it allows the land to be used for some alternative purpose. As the landfill is developed and progressively restored, a research program should be conducted to monitor the introduction of different species, and assess their relative success.

Table 04 Main biodiversity impacts of post-operational phase (IMP-PO-XX; Impact-Post Operational phase-Number)

| Impact | Receptor | Existence of mitigation measures | Potential significance of impact |
| --- | --- | --- | --- |
| IMP-PO-01 Post restoration | Flora (restoration process) Indirect effect on fauna | n/a | Positive impact |

# 6. Mitigation measures

Biodiversity is not just about the presence of a wide variety of vegetation and fauna, it is also about protecting the environment in which they exist.

With regards to biodiversity management, this document sets the following objectives:

  - to protect and conserve biodiversity

  - to avoid, minimize and mitigate impacts on biodiversity in construction and operational phase including offset significant

  - residual impacts, where appropriate, with the aim of achieving no net loss or a net gain of biodiversity

Codes of mitigation measures for impact on flora and fauna are compatible with the codes of the identified impacts presented in Chapter 3 of this document, as follows:

Construction phase	Operational phase

IMP-C-01 	MIT-C-01	IMP-O-01  	           MIT-O-01

IMP-C-02		MIT-C-02	IMP-O-02  	           MIT-O-02

IMP-C-03		MIT-C-03	IMP-O-03 	           MIT-O-03

IMP-C-04		MIT-C-04	IMP-O-04 	           MIT-O-04

IMP-C-05		MIT-C-05	IMP-O-05 	           MIT-O-05

IMP-C-06		MIT-C-06	IMP-O-06 	           MIT-O-06

IMP-C/O-XX Impact - Construction or Operational phase - Number

MIT-C/O-XX Mitigation measure- Construction or Operational phase – Number

## 6.1 Mitigation measures during construction phase

MIT-C-01 The following measures have been proposed in order to reduce the impact of dust and other air pollutants on the environment (biodiversity including), which can also reduce the air pollution during the construction of the CWMF Meckuevci - Arbasanci:

- Strict control of the construction methods, mechanical equipment and other equipment used;

- Careful planning of the construction works including the works near sensitive areas (please see Chapter 4.4, above) and area near boundary limit (prohibition of construction in certain periods of the day);

- Speed limitation of the construction vehicles on the construction site and in the populated areas;

- Sprinkling water or other appropriate material on the contrition site in order to reduce the emission of dust;

- Proper maintenance and covering of the vehicles delivering construction materials due to the reduction of the emission and dispersion of pollutants.

- Covering of the excess earth material and storage on specific places for further re-use in operational phase; to be stabilized in order to avoid erosion of the material.

MIT-C-02 As present in the Chapter 5.2.1 noise and vibration can lead to disturbance and disorder in natural comfort of fauna in the affected region.

The measures for mitigation of the impacts caused by noise and vibrations in construction phase include:

- Careful planning on preparation works in order to decrease the noise;

- Avoiding equipment which will emit noise more than 90 dB;

- Control on constructional methods and usage of mechanization and regular maintenance of the equipment because of the possible decrease of the noise;

- Trucks on site will not be left standing with engines idling;

- Equipment including bulldozers, graders, excavators and trucks will have all reasonable and feasible noise controls fitted to reduce noise emissions;

- Careful planning on time of the works (ex: seasonal bans during the mating of certain animal species and/or birds);

- Limit the speed of the constructional vehicles (up to 20 km/h).

- Selecting of traffic routes in regards of avoidance sensitive breeding area presented in Chapter 4.4 (Contractor in consultation with Environmental manager)

MIT-C-03 The animal species community is adapted to dynamic land use, so there is no additional disturbance to be expected beyond the impact zone. There will no additional land be used for the construction phase. Regarding impact on vegetation removal and degradation of habitats; loss of conservation value; the Designer shall pay special attention on requirements and findings in this document.

The Contractor will develop a specific Site Clearance Plan based on results of EIA and Ecological survey. The Plan should be prepared based on the EBS finding and should include commitment to protect specific vulnerable and sensitive habitats and species, and maintain ecosystem services. Where appropriate, measures should include avoidance (no-go areas to the Contractor), seasonal working (e.g. to avoid nesting of vulnerable birds species), pre-construction relocation of vegetation or fauna and rehabilitation of damaged habitats at of landfill zone. Implementation of this Plan will include periodically supervision and monitoring by experienced Biologist and Ornithologist (at least twice per month). Most significant habitats presented in Chapter 4.4 should be strictly taken into consideration while development of this plan.

Below are presented some general measures that need to be taken:

- Prior to construction, the layout plan must properly show the construction area so it is clearly demarcated.

- All development footprint areas should remain as small as possible and should not encroach onto surrounding natural areas.

- Workers must be instructed on the environmental/biodiversity requirements (forbidden hunting activities, setting fire etc.)

- Machinery and equipment used onsite must be in good working order in order to avoid negative noise impacts.

- If the use of explosive is planned (not recommended), it should be limited only to the period from November to February, in order to avoid the nesting period of the birds.

- Careful placement, harmonization and designing of the accompanying facilities (landfill objects, access roads, location for maintenance of the construction mechanization etc.) for the reduction of the impact;

- Careful selection of the location for the construction materials, storage/disposal of construction waste, i.e. selection of  a location which does not impact the protective green belt;

Design for the landfill site should incorporate an undisturbed buffer zone (within the expropriation limit) to be established around the site using existing and in some places planted trees and other vegetation, in order to reduce the effects of disturbance to surrounding facilities and wildlife populations.

Since the CWMF is considered as e permanent facility, this mitigation measures can be bonded with MIT-0-06 as well precaution measure for establishemt of two feeding places for Aquila heliacal explained in Annex 5. The Contractor will be responsible for establishment (during pre-construction) and maintenance of two controlled feeding places during the construction phase (for more info on mitigation and monitoring measures please see MIT-C-06 and Annex5).

MIT-C-04 The Contractor shall established strict biodiversity policy while construction phase. The Contractor’s personnel, visitors including, shall be introduced on fauna of surrounding area and prohibition of any disturbance or hunting. A special training on EIA and Biodiversity requirements shall be organized by Contractor’s Environmental Manager.

MIT-C-05 Proper waste management to be implemented by Contractor in order not to cause any potential pollution releases to soil, surface water and groundwater (please see MIT-C-06, below).

MIT-C-06 The waste management on the Project is going to be implemented in order to prevent soil and water from different hazardous and non-hazardous substances contained in different kind of wastes that will be created while carrying on construction activities. It is very important that the waste is going to be minimized, categorized, collected properly, recycled if possible, treated and disposed on appropriate locations.

The primary forms of waste which will be generated during construction phase will be:

- Excess excavated soil and sand

- Packing waste

- Debris and construction waste

- Waste tires (of constructional mechanization)

- Waste oils and greases (from construction mechanization), filter, adsorbents etc.

- Communal waste of working site camps (construction sites)

- Excess concrete material and washing outs of mixers.

Prior any working activity, the Contractor shall develop Waste Management Plan where each waste stream will be identifies including temporary or final disposal.

Appropriate assessment matrix of the identified mitigation measures during construction Project phase is presented below.

| Appropriate assessment: Mitigation measures (construction phase) |  |  |  |
| --- | --- | --- | --- |
| List of mitigation measures | How the measures will avoid or reduce the adverse effects on the integrity of the site |  | How will be implemented and by whom |
| MIT-C-01 | It has been assessed as a minor impact so it is not expecting any adverse effects on the site’s integrity outside the Project area. Implementation of the proposed mitigation measures (watering of the soil area, proper maintenance of the mechanization, covering earth material etc.) will reduce the impact of dust and other air pollutants on the surrounding habitats during construction phase. |  | It shall be implemented by the Contractor during the construction phase of the Project |
| MIT-C-02 | Since the CWMF is on a decent distance (identified in Chapter 4.4) from the breeding sites of identified affected with higher protection status (Chapter 4.4), minor to moderate impact is expecting from noise and vibration during construction phase. Mitigation measures will reduce and limit the impact, thus integrity of the site outside Project area would not be affected. |  | It shall be implemented by the Contractor during the construction phase of the Project |
| MIT-C-03 | In order to assure preservation of the breeding sites for Aquila heliaca and Falco naumanni presented in Chapter 4.4 as well possible feeding locations identified with MIT-O-06 (outside of the Project area), the Contractor will develop and implement Site clearance plan based on the EBS findings. Establishment of undisturbed buffer zone (see also MIT-O-06) by the Contractor after initial construction activities will be finished. Full establishment will be achieved during operational phase. Widely spread similar habitat (feeding areas for birds - mainly Aquila heliaca) are identified nearby CWMF as presented in MIT-O-06. The construction phase would not have any impact on these areas. Since the Project will have direct impact on the small percentage of the habitat identified as possible feeding location for birds (mainly Aquila heliaca) as presented in Chapter 4.4, precaution mitigation measures are identified during preconstruction phase and construction phase (please see MIT-O-06 and Annex 5) i.e. establishment and mainteain of two controlled feeding areas for Aquila heliacal. |  | It shall be implemented by the Contractor during the pre-construction or construction phase of the Project |
| MIT-C-04 | All of the Contractor’s personnel will attend on a short biodiversity training prior construction phase so to avoid any interference with the fauna in the wider region. |  | It shall be implemented by the Contractor during the pre-construction phase of the Project |
| MIT-C-05 | The Contractor shall work in accordance with National Law on Waste Management so different waste streams will be suitable managed and disposed (development of Waste Management Plan by the Contractor) Out of Project area there will be forbidden to locate any workers camps or temporary facility in order not to occupy anymore area from the surrounding habitat. |  | It shall be implemented by the Contractor during the construction phase of the Project |
| MIT-C-06 |  |  | It shall be implemented by the Contractor during the construction phase of the Project |
| List of mitigation measures | Degree of confidence in their likely success | Timescale when they will be implemented | Proposed monitoring scheme |
| MIT-C-01 | Likely success is expecting | During construction phase | Please see Chapter 7.2; baseline monitoring and monthly monitoring should be performed by the Contractor to assure proper implementation of mitigation measures |
| MIT-C-02 | Likely success is expecting | During construction phase | Please see Chapter 7.2; baseline monitoring and monthly monitoring (on CWMF and traffic routes) should be performed by the Contractor in order to assure proper implementation of mitigation measures |
| MIT-C-03 | Likely success is expecting for preserving the important habitats outside of the Project area Precaution measures are identified (see MIT-O-06 as well details provided in Annex 5) | During construction phase Prior construction phase | Implementation of the Site Clearance Plan will be monitored by Biologist at least twice per month. Keeping records by the Contractor and handing over to the Operator in further phase 3 monitoring periods (prior construction, during construction and after completion of construction of CWMF – at least 10 monitoring days per session; data on breeding success/failure and feeding locations should be monitored) |
| MIT-C-04 | Likely success is expecting | During construction phase | Records and photo documentation form training events |
| MIT-C-05 | Likely success is expecting | During construction phase | Appropriate records on handling of a different waste streams |
| MIT-C-06 | Likely success is expecting | During construction phase |  |

## 6.2 Mitigation measures during operational phase

MIT-O-01 The landfill should be equipped with the gas collection system which should be designed in compliance with the Macedonian and EU standards and should include thermal destruction on a closed-type flare. The efficiency of the gas collection system will be appropriately monitored. Migration of the gas through soil substrata will be monitored by surrounding boreholes.

In case of open-flare the following will be applicable:

A collar of spikes can be placed around the top burner rim to prevent raptors from perching on the stack (Huntington-Oyster Bay Audubon Society and The Solid Waste Association of North America 2010, Moller 2009, Siftar 2008). Even simple spikes affixed to a burner pipe would make perching more difficult for larger birds.

The triangular spikes must be tall enough (50 cm or more) so a raptor can't straddle the triangles to perch and be narrow enough between, so a smaller raptor can't squeeze in between to perch. Placing upright rods around the rim is not recommended since they may resemble reeds and smaller species can perch on or between them.

Figure 27. Collar of spikes around the top burner rim

Source: Daniele Peters; Methane Burner Impact on Raptors

Methane burners may be installed in a way to minimize the risk to raptors. Some burner manufacturers suggest utility poles near burners should be no closer than four times the height of the burner (e.g., a 7 m burner must be at least 30 m from the nearest utility pole) (Shaw LFG Specialties 2008).

When placing burners, it also is important to consider burner alignment relative to other poles. Burners should not be placed between poles (i.e., raptors flying in a straight line from one pole to another should not pass over the flame).

Figure 28. Improper placing of a burner relative to perching sites

(A) Pole to pole flight path. (B) Flame origin (usually invisible). (C) Area of dangerous intense heat.

Source: S.E. Bowen; Methane Burner Impact on Raptors

Since many raptors use perches to hunt from and roost on, it may be preferable to remove selective perches near a burner. Although the proper distance has not been scientifically studied, installation of alternative perching poles near a burner is not recommended. In some areas, perching trees may be plentiful around landfills, so removal of potential perches near the burner might also be prudent. In other areas, there may be few trees or structures for perching. In such areas, installation of alternate perches might reduce the chance of raptors perching on the flare, as long as the perch is placed far enough away from the burner.

MIT-O-02 The proposed landfill should be equipped with a system for leachate collection and treatment on-site, before its final discharge. Part of the leachate will be recirculate to the landfill body thus assisting the biodegradation process and shortening the period required for stabilization of the landfill. Based on a preliminary calculation (Feasibility Study), it is estimated that about 23.5 m3 will be generated daily.

According to the Macedonian legislation, the treated leachate can be discharged either into the natural recipient at the site or into the municipal sewer system outside of the landfill. The detailed technical design and EIA Study will define where the treated leachate will be discharged. The closest surface water to the landfill is the Klobuc, an intermittent stream running east of the site. The Macedonian effluent standards are established for the discharge both into the natural recipient and sewage.

As identified in MIT-C-03 and MIT-O-06, surface water is very important for keeping of the conservation value of this area, for both flora and fauna. So, discharge into natural recipient should be avoided or strictly monitored by relevant appropriate parameters such as T, pH, el. conductivity, ammonia, total N, BOD, COD, metals, sulphate, chloride, total phosphorus, cyanide, fluoride, trace organic substances.

MIT-O-03 Nuisance control (dust, litter, odour emissions and fire) will be achieved through effective compacting and regular covering of waste, minimization of open tipping area, landfill fencing and landscaping. Landscaping will consider planting of the green barrier around the landfill site.

Monitoring the impacts from dust generating activities will be undertaken using visual inspections of landfill activities as well laboratory measurements. This risk based approach will highlight the effectiveness of implemented dust controls and the need for any additional measures.

Visual inspections will be undertaken by Operator periodically to ensure operational activities are not generating excessive amounts of dust with the potential to adversely impact receivers in close proximity. This includes monitoring the following key mitigation measures:

- Dust covers on haulage trucks are in place prior to leaving the construction site and whenever the load is in transit

- Stockpiles, spoil movements  and cleared  areas are  being appropriately managed

- Inspection of access roads

Inspections of plant and vehicles will be undertaken to ensure that they have appropriate emission controls that are being maintained correctly. The following monitoring requirements will be implemented:

- Mechanical inspections of plant and vehicles will be undertaken regularly during use on the Project to ensure all equipment have appropriate emission control devices and are in good working order

- Operator will undertake inspections of plant and haulage trucks to ensure there is no unnecessary running of plant and vehicles or any excessive emission of fumes or smoke

MIT-O-04 Some landfill operators are using set of noises with different frequencies as well flashlight in order to prevent from the presence of birds. When birds are hell-bent on getting the best treats from the landfill, they will quickly become insusceptible to traditional methods like loud noises and flashing lights. Also, these methods can be contra-effective and can cause disturbance of fauna at its surrounding.

During long summer days, or on weekends when the landfill site may be closed, birds may be able to gain access to feed at the site. So, human presence, ate least on periodic basis is very important in bird control management.

Habitat alteration involves eliminating or restricting access to any of the basic survival requirements of birds within footprint operational area. This can often be accomplished by minimizing the surface area of the active landfill, providing good daily coverage of refuse and managing nearby vegetation. In some cases, it may be necessary to restrict access to waste with overhead wires or other physical barriers. Details on these methods are provided below:

- Minimize Surface Area of Active Landfill - The landfill should be designed and operated so as to minimize the surface area where waste is deposited. This will decrease the site's attractiveness to any birds.

- Preventing access to edible waste (Daily Coverage) - Conventionally, landfill coverage is accomplished with soil materials. When soil materials are used, the coverage must be at least 20 cm deep. Regular coverage of waste throughout the day is desirable since it minimizes the availability of food. In most cases, good coverage of waste is the only means of preventing gull feeding during times when the landfill is closed. In practice however, complete coverage of waste is difficult to achieve, although it should still be a goal of landfill operations. In situations where it is determined that conventional coverage methods is not preventing birds from gaining access to waste after hours, alternative coverage methods (e.g., tightly-woven plastic mesh) that can be pulled or rolled out over the active cell should be considered.

- Vegetation Management – Some birds prefer, and are attracted to large open areas that are either devoid of vegetation (e.g., paved areas, gravel areas, exposed soil areas etc.), or have closely mown vegetation (pastures, agricultural fields etc.). All areas of exposed soil (except active cells, roads, and other necessary openings) on the landfill property which are presently devoid of vegetation (less than 25 percent plant cover) should be hydro-seeded or cultivated with an appropriate mixture of seed, fertilizer and mulch. The particular species of plant is not important, however, it should be a fast-growing species which grows rank (dense) and tall (>25 cm), grows on sterile soils with little cultivation, and has stout stems that resist lodging (laying down under rain, snow, wind). Potentilla has been suggested in the literature as suitable, however, quack grass (Elmus repens), or other species recommended by the Natural Resource Conservation Service for this purpose and in these soils can be used. The resultant vegetation should be allowed to grow, and be maintained at a height of at least 25 cm where possible. The goal is to have the site's vegetation tall and dense. One cutting per year will usually be adequate to prevent the growth of woody vegetation. This cutting should be done in late April (i.e., prior to grassland bird nesting seasons) so that the vegetation is rank and at least ten inches tall for the fall and winter months. As the landfill operations progress, all areas that are recently filled and covered, and expected to remain inactive for a least one month, should be planted  and treated as described above. Efforts should be made to maintain as much of the landfill property as possible with vegetation at least 25 cm tall.

- Water Elimination from active landfill - Every attempt should be made to avoid, minimize and/or eliminate non-jurisdictional and non-required surface waters on the landfill site, including ponds, borrow pits and puddle areas. Grading activities conducted during landfill operations should be done in a manner to avoid the creation of surface water bodies. Please pay attention that this shall be limited only in landfill footprint area and it is not applicable in buffer zone where conservation is priority.

- Physical Barriers - Several means of physically excluding bird from landfill areas can been used, including wire grids and metal buildings constructed over the waste area. These tend to be expensive and/or labor intensive however. Wire grids, consisting of a single series of parallel lines, but more commonly two series of parallel lines erected at right angles to each other to form a mesh or grid pattern, can be erected above the active cell of the landfill.

Harassment and lethal methods shall be strictly prohibited in area.

When fencing is applicable, a ‘paper fence’ is preferable to prevent trash from blowing outside the landfill area. This fencing usually consists of netting that may be as tall as a utility pole. Poles suspending this netting serve as perching opportunities and can be fitted with anti-perch devices, if needed. An example of the method one landfill used to discourage perching is shown in Fig. 29 below. The rods at the top of the poles are set at an angle, making them less attractive perches.

Figure 29. Example of retrofitting nets to better contain blowing trash and keep large bird from perching.

Source: Joey Mason; Methane Burner Impact on Raptors

MIT-O-05 As good positioned landfill equipment and access roads during design and construction phase, the Operator shall pay attention to:

- Limit the speed within the landfill area, its surrounding and traffic routes near CWMF.

- Choosing proper equipment that cause with minimal noise pollution.

- Regular maintenance of the equipment and vehicles because of the possible decrease of the noise.

- Performing periodically monitoring on noise on the boundaries of CWMF as well near the traffic routes and access roads as stated in Chapter 7, below.

- Plant trees (buffer zone) on the landfill boundaries in order to minimize the noise effect on the surrounding area (see MIT-O-06).

- Effective exhaust mufflers in proper working condition will be maintained on engine-powered equipment at the site. Mufflers found to be defective will be replaced promptly.

- Physical noise barriers such as soil berms may be done to provide shielding of landfill operating equipment noise to limit offsite noise propagation.

- Landfill operations and waste placement may be staged such waste lifts provide additional shielding of the landfill working face equipment noise to limit offsite noise propagation.

- Operation of two (2) separate working face areas to limit the additive effect of working face equipment noise.

- Implementation of alternative “white noise” backup alarms to reduce noise.

- Use electro power vehicles which generate much lower noise, if applicable.

Mainly, the Designer, Contractor and Operator shall take into consideration the main findings provided within the Chapter 4.4 so traffic routes shall be limited near these sensitive receptors (breeding sites of Aquila heliaca) i.e. buffer zone to be established for minimization of noise disturbance.

As per study “A review of disturbance distances in selected bird species - M. Ruddock & D.P. Whitfield”,  recommendation of a minimal protection buffer of 500 m (based on 95 % prediction of flight probability) with a vulnerable zone extending to 800 m (based on 99 % prediction of flight probability) where some activities, such as vehicles, should be allowed. This study was obtained on Spanish Imperial eagle but it has been mentioned that the results can be a model for other Aquila species too.

As per figure 22, nearest breeding site of Aquila heliaca (AH26) is located approximately 1000 meters from the existing road which will be used as one of the traffic routes.  Thus, it can be predict that the traffic routes would not have negative impact on the breeding sites of Aquila heliaca.

Figure 30. Traffic route leading to the CWMF (yellow line presents the road)

Noise pollution due to landfill operation can be assessed with minor to moderate impact, especially with implementation of undisturbed buffer zone (see MIT-O-06) as well requirements presented above.

MIT-O-06 The design for the CWMF landfill site should incorporates an undisturbed buffer zone established around the site using existing vegetation as well planting trees and other vegetation. Native trees i.e. Quercus pubescens or communities of Italian oak and hornbeam (Carpinus orientalis) did not show some satisfactory results on planting so Conifer tree plantations represented mostly by Cypress trees (Cupressus sempervirens, Cupressus arizonica etc) and Black pine (Pinus nigra) should be used. Cypress and Black pine tree are very well acclimatized to the soil and climate conditions in the region under consideration, and it is very often used in plantations, which support a wide range of wildlife.

This activity will reduce the effects of noise, dust and other disturbance to surrounding area.

The buffer zone is an area of restricted activities, depending on the application and abutting uses. That is, the conduct of waste operations would be prohibited, but there is no reason why other compatible revenue-generating activities may not be allowed on the buffer. For example, tree planting would provide a visual screen and limit transport of litter and dust from the operations area, and may well provide a substantial income to the operator at the end of the active life of the landfill.

Figure 31. Buffer zone established on the Landfill; Source: Website Landfill Conservancies

One positive example of established buffer zone on landfill is provided on a photo below.

Fencing of the site shall be kept on minimum operational area so as to facilitate wildlife dispersion through the buffer zone areas.

Ovce Pole is open and hilly terrain with the elevation ranging between 150 and 400m. The region consists of dry grasslands, arable lands and small forests of oak species.

Part of the habitats in  the  project area are  grasslands  represented  by  the  category  “hill  pastures”,  which is characteristic of the hilly areas in all valleys and plateaus in Macedonia.

Hill pastures could be found in the vicinity of the CWMF (see figure 32 below), especially on the hilly terrain south-eastern to the village Pestrishino on the locality Pikev rid, Alachuga, Stoilev rid, Livada and Govedarov Kamen.

As explained, these habitats are predominately used as feeding areas for Imperial eagle (Aquila heliaca) as well Falco naumanni. Since part it will be loss by footprint area of CWMF (0.08% from total IBA area), still figure 32 shown widely spread habitat of hill pastures in the surrounding that would not be covered or affected by construction or operational phase of CWMF so it can handle a suitable feeding process of the affected bird species. Hence, with implementation of the proposed mitigation measures, it may be assumed that the Project will not adversely affect the integrity of the site.

Figure 32. Hill pastures nearby the proposed location for CWMF and elevation of the terrain (*- CWMF; O – selected nearby hill pastures);

Moreover, due to loss of the part from the feeding/hunting area, precaution mitigation measure for establishment of two controlled feeding places is estimated and more details are provided in Annex 5 of EBS. The feeding places will target population of Aquila heliaca to cover necessary demands mainly on identified breeding locations presented in Chapter 4.4, but also may serve to other species of Imperial eagle and other birds in Ovce pole region.

These feeding places will be established during pre-construction phase by the Contractor and handed over to Operator during operational phase. The Contractor will have responsibility for theirs maintain during construction phase and respectively Operator during operational phase (at least five years as explained in Annex 5). All of the characteristics for establishment, technical specification and monitoring requirements for the controlled feeding areas are provided in Annex 5.

Feeding of birds on specially prepared feeding places is a very important measure which will attract as well keep birds within the feeding and nesting territories. It will provide a number of benefits for the birds such as the increase of food availability, increase of survival rates, improvement of breeding performance, increase of micronutrients availability such as calcium, reduction of the risk of poisoning as well as some environmental benefits. Especially for species of Imperial eagles, controlled feeding places may increase the population viability and also contribute to the settlement of new breeding territories as their abundance of food has been found to be closely related with the formation of communal roosts that hold congregations of non-breeding individuals.

Hence, proper implementation of this precaution mitigation measure will contribute for keeping stable population of Aquila heliaca in the region of Ovce pole in order to assure preserve and maintain conservation status of the IBA region.

The Contractor and Operator will implement specific bird monitoring as presented in the Chapter 7.2, below.

Appropriate assessment matrix of the identified mitigation measures during operational Project phase is presented below.

| Appropriate assessment: Mitigation measures (operational phase) |  |  |  |
| --- | --- | --- | --- |
| List of mitigation measures | How the measures will avoid or reduce the adverse effects on the integrity of the site |  | How will be implemented and by whom |
| MIT-O-01 | Applicable only for open-flare method. Collar spikes will be installed around top burner rim so to discourage raptors perching on it. Consideration for the location of burner so to avoid any injuries of the birds - perch management strategy. With implementation of presented mitigation measures possible impact will be avoided. |  | Designer to consider into the Project. Implemented by the Contractor during the construction phase so further impact to be avoided during operational phase. |
| MIT-O-02 | Leachate collection system and appropriate treatment before its final discharge so to avoid any pollution of surface water or groundwater. This impact is considered in the EIA study too. |  | Implemented by the Contractor during the construction phase Proper monitoring of the discharge by Operator |
| MIT-O-03 | Dust, litter and odour emissions will be reduced with appropriate compacting and regular covering of waste, minimization of tipping are, landfill fencing and landscaping. Also, buffer zone established around the landfill footprint area will act as a barrier to avoid this impact to be transferred on the surrounding habitat. |  | Buffer zone – primary to be established by the Contractor Maintain of buffer zone and implementation of further mitigation measures as identified in MIT-O-03 by the Operator |
| MIT-O-04 | Concentrated presence of birds will be avoided with the following bird control managements measures: - Minimize of surface area of active landfill - Preventing access to edible waste - Vegetation management - Water elimination - Physical barriers - Human presence All of these techniques will decrease attractiveness of landfill to the birds. |  | Operator during operational phase |
| MIT-O-05 | Undisturbed buffer zone as explained in MIT-O-06 and MIT-O-03 will secure minimization of the noise impact from CFMF elements on the surrounding habitats. Moreover, the Operator shall implement predicted mitigation measures (presented above) so the impact will be reduced. Since the traffic routes are located in a distance more than 1000 meters from the identified breeding sites of Aquila heliaca, it may be assumed that would not have any negative impact on these sites nor hunting territories. |  | Operator during operational phase |
| MIT-O-06 | Maintain of buffer zone in order to act as a barrier between landfill footprint area and surrounding habitat. Minimization of footprint area in accordance with the Design. This will allow preservation minimization of habitat loss as much as possible. Worst case scenario is loss of 0.08% from the total area of proposed IBA. Preservation of surrounding habitats important for feeding of Aquila heliaca (and other bird species) through proper implementation of the package of mitigation measures identified for construction and operational phase. Implementation of precaution mitigation measures of establishment of two feeding areas for Aquila heliaca; as mentioned this measure is mainly precaution so to keep stable population of the Imperial eagle in the region. With these proposed measures, it was assessed that this impact will not cause significant effects on the proposed IBA site. |  | Contractor during construction phase Contractor/Operator The Contractor will establish two feeding areas during pre-construction phase and appropriately will handed over to Operator as identified in Annex 5 |
| List of mitigation measures | Degree of confidence in their likely success | Timescale when they will be implemented | Proposed monitoring scheme |
| MIT-O-01 | Likely success is expecting | During construction phase | Photo documentation kept by the Contractor/ Supervision to oversee |
| MIT-O-02 | Likely success is expecting | During construction and operational phase | Proper monitoring of the discharge to meet National requirements by Operator |
| MIT-O-03 | Likely success is expecting | During operational phase | Appropriate monitoring as presented in the Chapter 7.2; quarterly measurements to be organized by the Operator |
| MIT-O-04 | Likely success is expecting | During operational phase | In accordance with Chapter 7.2; twice per year monitoring by Ornithologist - to be organized by the Operator |
| MIT-O-05 | Likely success is expecting | During operational phase | In accordance with Chapter 7.2; quarterly measurements near the CWMF and on traffic routes near sensitive receptors |
| MIT-O-06 | Likely success is expecting (precaution mitigation measures are proposed to assure the necessary preservation) | Buffer zone will be established by the Contractor; Operator to maintain it appropriately during operational phase Precaution measures implemented by the Contractor and handed over to Operator during operational phase (as described in Annex 5) | In accordance with Chapter 7.2; Operator/Bird monitoring - Before starting of operational phase (at least 15 monitoring days) Operator/Bird monitoring - Twice per year during the first five years of operation (at least 15 monitoring days per session) Monitoring to confirm any breeding failure/success, hunting areas and presence of bird species in the CWMF and its surrounding |

# 7. Biodiversity Management and monitoring plan

## 7.1 Biodiversity Management consideration

A Biodiversity Management Plan (BMP) is produced subsequent to an EIA with the principle aim of managing biodiversity assets identified within this document, as well as what and where project-specific mitigation measures should be implemented. Mitigation is defined as follows:

• Avoid the impact altogether by not taking a certain action or parts of an action

• Minimizing impacts by limiting the degree or magnitude of the action and its implementation, e.g., by implementing construction/operational measures to minimize impact

• Rectifying the impact by repairing, rehabilitating or restoring the affected environment

• Reducing or eliminating the impact over time by preservation and maintenance operations during the life of the action, e.g., by changing behavior

• Compensating for the impact by replacing or providing substitute resources or environments

The sequence of preference for measures is:

The BMP gives mitigating measures and monitoring requirements, and identifies the organizations assigned to implement them. Mitigating measures for the impacts that are likely to arise from implementing the project are given in the Chapter 6.

During the design phase, the Consultant (Designer) assigned to design the Landfill project (CWMF) will be responsible for incorporating the recommended mitigation measures from this document into the design and the technical specifications of the bidding document.

During construction phase, the Contractor will be responsible for implementing mitigation measures included in the design and technical specifications. The construction supervisor will monitor the proper implementation of mitigating measures at the right time, particularly for the impacts of the biodiversity. The Contractor will be fully responsible for ensuring that all the work will be carried out as per the environmental requirements indicated in the design and technical specification.

It is also envisaged that a Biologist/Ecologist, with a broad range of experience and will be intermittently assigned by both Contractor and Supervisor for a proper implementation of the mitigating measures. Biologist or Ecologist will be responsible for the overall coordination of the biodiversity management activities, advise the contractors, construction supervisors and meet with the Operator and authorities regarding the implementation of the environmental mitigating measures and monitoring of impacts.

During the operation period, the biodiversity issues will be managed and controlled by the Operator. In particular, MoEPP and/or State Environmental Inspectorate may overview and supervise the implementation as per National Regulation.

Proper implementation of the requirements of this document will ensure that the proposed Landfill will meet regulatory and operational technical biodiversity best techniques.

Environmental management/monitoring is essential for ensuring that identified impacts are maintained within the allowable levels, unanticipated impacts are mitigated at an early stage (before they become a problem), and the expected project benefits are realized. Thus, the aim of BMP is to assist in the systematic and prompt recognition of problems and the effective actions to correct them, and ultimately good environmental performance is achieved.

A good understanding of environmental priorities and policies, proper management of the facility (at the National, municipality and the local levels), knowledge of regulatory requirements and keeping up-to-date operational information are basic to good environmental performance.

Before starting of operational phase, an Operator shall organize a compliance monitoring performed by outsourced company/experts. Compliance monitoring provides for the control point of this document, and the initial report from this monitoring should show a progress, implementation and effectiveness of the measures stipulated in this document. This shall also include bird monitoring as per information provided in this document. Afterwards, such monitoring shall be performed twice per year during the first three years of operation (at least 15 monitoring days to be allocated for the Bird Expert per one monitoring session – see Chapter 7.2; also details regarding monitoring are provided in Annex 5).

The Operator’s staff that shall attend training programs to improve their qualifications and update their information on environment requirements (including biodiversity). Both Contractors and Consultants would be involved in knowledge transfer to operators and management through regular assistance

## 7.2 Biodiversity Monitoring Plan

| Medium | Project Phase | Measured Parameters | Location of the measurement place | Mode/Type of measurement | Frequency of measurement | Responsibility |
| --- | --- | --- | --- | --- | --- | --- |
| Air Quality (especially PM10 particles | Construction/Operational | According to the legal regulations on the quality of ambient air - especially solid particles (PM10) | Total 4 measurement points (on the each side of landfill) | Visual Monitoring with appropriate measurement equipment /laboratory tests | Monthly (construction phase, one baseline monitoring during pre-construction phase) Quarterly (operation phase) | Contractor/Operator |
| Noise | Construction/Operational | Level of noise (dB) | 2 measurement points on the each side of landfill as well as noise from traffic routes near sensitive receptors (MIT-O-05) Mainly in the period from March to September due to breeding season of the birds | Monitoring with appropriate measurement equipment (accredited laboratory) | Monthly (construction phase, one baseline monitoring during pre-construction phase) Quarterly (operation phase) | Contractor/ Operator |
| Establishment of buffer zone | Design/Construction/Operational | In accordance with MIT-O-06 | Around the landfill footprint area in expropriation zone | Area established/Visual/ Photo documentation prior and post establishment | While construction (twice per month by Biologist) While operation (once per month during the initial year) | Contractor/ Supervision to oversee/ Operator |
| Burner installation (applicable in case of open flare) | Construction | In accordance with MIT-O-01 | On spot | Visual |  | Supervisor to oversee/ Operator |
| Site Clearance | Construction | In accordance with MIT-C-03 | Along the landfill area | Visual/ Photo documentation | Twice per Week inspection by Supervision staff | Contractor/ to be approved by the Supervision |
| Waste Management | Construction | Type and amount of waste, types and fractions of waste (hazardous, communal, inert etc.) | Along the landfill area | Keeping records regarding the type and amount, as well as the mode of disposal of different types of waste | While construction phase | Contractor |
| Bird Control Monitoring and Management | Construction | Number and type of bird species (special attention to the species of Aquila heliaca and Falco naumanni) | Along the landfill area and its vicinity (including breeding sites appoint in Chapter 4.4) | Photo documentation / Ornithological Survey (to be handed over to Operator) To confirm breeding failure/success, hunting areas and presence of bird species in the CWMF and its surrounding | 3 monitoring periods (prior construction, during construction and after completion of construction of CWMF – at least 10 monitoring days per session) | Contractor |
|  | Operational | In accordance with MIT-O-04 / Number and type of bird species (special attention to the species of Aquila heliaca and Falco naumanni) | Along the landfill area and its vicinity (including breeding sites appoint in Chapter 4.4) | Photo documentation of implemented measures/ Ornithological Survey To confirm breeding failure/success, hunting areas and presence of bird species in the CWMF and its surrounding | Before starting with operational phase (at least 15 monitoring days) Twice per year during the first five years of operation (at least 15 monitoring days per session) | Operator |

# 8. Discussion

This document has established baseline conditions of the natural environment of CWMF Meckuevci-Arbasinci including its vicinity.

CWMF Meckuevci - Arbasanci is situated within the internationally designated IBA site – MK019 Ovce Pole (close to the northern boundary zone). Because of the characteristic habitat on the selected location for CWMF and its surroundings, proposed as Important Bird Area (IBA), as well as the presence of 12 species of birds with a higher degree of protection (presented in Annex 2.2), its preservation should be treated with the highest importance. In this document mainly has been targeted bird species of Aquila heliacal and Falco naumanni as the most important ones for the proposed IBA site.

The purpose of a habitat survey was to identify different types of habitats found within a project area and to delineate their coverage. The results are consolidated in form of a habitat map and photographs shown in Annex 3. Additionally, most significant habitat areas area for affected bird species are presented in Chapter 4.4.

The document recognizes the biodiversity impact during construction and operational phase of the project as well as sets an appropriate measures in order mitigation on the identified impact.

Biodiversity management consideration for Designer, Contractor, Supervision and Operator is identified including monitoring plan with parameters, monitoring frequences, location and responsibilities while each phase.

In general, the Project for establishment of integrated and finacial self-sustanaibale waste management system will have a positive biodiversity impact in East and North-East region of Macedonia, taking into consideration current negative status of waste management and impact of non-standard landfill sites on flora and fauna.

On a local level, since the location of CWMF is within proposed Important Bird Area, the main importance is proper impmelementation of proposed mitigation measures indetified in EIA/EBS in order to avoid/reduce any negative impact on the surrounding habitat so to preserve its conservation value..

# 9. ANNEXES

ANNEX 1. MAMMALS

| No | Species | Bern Convention – Appendices | Habitat Directive – Annexes | Bonn Convention | CITES Convention - Appendices | IUCN Red List |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Canis lupus | II | II IV |  | II | LC |
| 2 | Capreolus capreolus | III |  |  |  | LC |
| 3 | Crocidura suaeveolans | III |  |  |  | LC |
| 4 | Erinaceus concolor | III |  |  |  | LC |
| 5 | Felis sylvestris | II | IV |  | II | LC |
| 6 | Glis glis | III |  |  |  | LC |
| 7 | Lepus europeus | III |  |  |  | LC |
| 8 | Lutra lutra | II | II IV |  | I | NT |
| 9 | Martes foina | III |  |  |  | LC |
| 10 | Martes martes | III |  |  |  | LC |
| 11 | Meles meles | III |  |  |  | LC |
| 12 | Mustela nivalis | III |  |  |  | LC |
| 13 | Myotis myotis | II | II, IV | II |  | LC |
| 14 | Myotis emarginatus |  | IV |  |  | LC |
| 15 | Myotis mystacinus |  | IV |  |  | LC |
| 16 | Pipistrellus nathusii | II | IV | II |  | LC |
| 17 | Pipistrellus pipistrellus | III | IV | II |  | LC |
| 18 | Rhinolophus ferrumequinum | II | II, IV | II |  | LC |
| 19 | Sciurus vulgaris | III |  |  |  | LC |
| 20 | Microtus rossiaemeridionalis |  |  |  |  | LC |
| 21 | Vormela peregusna | II |  |  |  | VU |
| 22 | Sus scrofa | III |  |  |  | LC |

ANNEX 2. BIRDS

Annex 2.1 Bird species  - CWMF Meckuevci-Arbasanci and its vicinity

|  | SPECIES | PRESENCE | IUCN | IUCN EU | BIRD DIRECTIVE | BERN CONVENTION |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Accipiter gentilis | Nbr. | LC | LC | not included | annex II |
| 2 | Accipiter nisus | Nbr. | LC | LC | not included | annex II |
| 3 | Alcedo atthis | br. | LC | LC | Annex II/B | annex III |
| 4 | Alectoris greaca | br. | VU | VU | Annex II/A | annex II |
| 5 | Anthus campestris | Nbr. | LC | LC | Annex I | annex III |
| 6 | Ardea alba | br. | LC | LC | Annex I | annex II |
| 7 | Apus apus | Nbr. | LC | LC | not included | annex II |
| 8 | Athene noctua | br. | LC | LC | not included | annex II |
| 9 | Aquila chrysaetos | br. | LC | LC | Annex I | annex II |
| 10 | Aquila heliaca | br. | LC | VU | Annex I | annex II |
| 11 | Burhinus oedicnemus | br. | LC | LC | Annex I | annex II |
| 12 | Buteo buteo | br. | LC | LC | not included | annex II |
| 13 | Buteo rufinus | br. | LC | LC | Annex I | annex II |
| 14 | Carduelis carduelis | br. | LC | LC | not included | annex II |
| 15 | Cecropis daurica | br. | LC | LC | not included | annex II |
| 16 | Cettia cetti | br. | LC | LC | not included | annex II |
| 17 | Chloris chloris | Nbr. | LC | LC | not included | annex II |
| 18 | Ciconia ciconia | br. | LC | LC | Annex I | annex II |
| 19 | Circus cyaneus | Nbr. | LC | NT | Annex I | annex II |
| 20 | Circus pygargus | Nbr. | LC | NT | Annex I | annex II |
| 21 | Circaetus gallicus | br. | LC | LC | Annex I | annex II |
| 22 | Coccosthraustes coccosthraust | br. | LC | LC | not included | annex III |
| 23 | Columba livia | br. | LC | LC | Annexe II/A | annex II |
| 24 | Columba palumbus | Nbr. | LC | LC | Annexes II/A; III/A | annex III |
| 25 | Coracias garrulus | br. | LC | NT | Annex I | annex II |
| 26 | Corvus corax | br. | LC | LC | not included | not included |
| 27 | Corvus corone | Nbr. | LC | LC | Annex II/B | annex III |
| 28 | Corvus monedula | br. | LC | LC | Annex II/B | not included |
| 29 | Coturnix coturnix | br. | LC | LC | Annex II/B | not included |
| 30 | Cuculus canorus | br. | LC | LC | not included | annex III |
| 31 | Curruca communis | br. | LC | LC | not included | annex III |
| 32 | Curruca crassirostris | br. | LC | LC | not included | annex III |
| 33 | Curruca melanocephala | br. | LC | LC | not included | annex II |
| 34 | Delichon urbicum | br. | LC | LC | not included | annex II |
| 35 | Dendrocopos minor | br. | LC | LC | not included | annex II |
| 36 | Dendrocopos syriacus | br. | LC | LC | Annex I | annex II |
| 37 | Emberiza calandra | Nbr. | LC | LC | not included | annex II |
| 38 | Emberiza cirlus | br. | LC | LC | not included | annex III |
| 39 | Emberiza citrinella | br. | LC | LC | not included | annex II |
| 40 | Falco biarmicus | br. | LC | LC | Annex I | annex II |
| 41 | Falco naumanni | br. | LC | LC | Annex I | annex II |
| 42 | Falco tinnunculus | br. | LC | LC | not included | annex II |
| 43 | Falco peregrinus | br. | LC | LC | Annex I | annex II |
| 44 | Falco subbuteo | br. | LC | LC | not included | annex II |
| 45 | Fringilla coelebs | Nbr. | NT | NT | not included | annex II |
| 46 | Galerida cristata | Nbr. | LC | LC | not included | annex III |
| 47 | Garrulus glandarius | br. | LC | LC | Annex II/B | annex III |
| 48 | Hippolais olivetorum | br. | LC | LC | Annex I | annex II |
| 49 | Hirundo rustica | br. | LC | LC | not included | annex II |
| 50 | Lanius collurio | br. | LC | LC | Annex I | annex II |
| 51 | Lanius excubitor | br. | LC | VU | not included | annex II |
| 52 | Lanius minor | br. | LC | LC | Annex I | annex II |
| 53 | Lanius senator | br. | LC | LC | not included | annex II |
| 54 | Linaria cannabina | br. | LC | LC | not included | annex II |
| 55 | Lullula arborea | br. | LC | LC | Annex I | annex II |
| 56 | Motacilla alba | br. | LC | LC | not included | annex II |
| 57 | Motacilla cinerea | br. | LC | LC | not included | annex II |
| 58 | Motacilla flava | Nbr. | LC | LC | not included | annex II |
| 59 | Muscicapa striata | br. | LC | LC | not included | annex II |
| 60 | Oenanthe oenanthe | Nbr. | LC | LC | not included | annex II |
| 61 | Oriolus oriolus | br. | LC | LC | not included | annex II |
| 62 | Otus scops | br. | LC | LC | not included | annex II |
| 63 | Panurus biarmicus | br. | LC | LC | not included | annex II |
| 64 | Parus major | Nbr. | LC | LC | not included | annex II |
| 65 | Passer domesticus | br. | LC | LC | not included | annex II |
| 66 | Passer hispaniolensis | br. | LC | LC | not included | not included |
| 67 | Passer montanus | br. | LC | LC | not included | annex III |
| 68 | Perdix perdix | br. | LC | LC | Annexes II/A; III/A | annex II |
| 69 | Pernis apivorus |  | LC | LC | Annex I | annex II |
| 70 | Phoenicurus ochruros | br. | LC | LC | not included | annex III |
| 71 | Phylloscopus collybita | Nbr. | LC | LC | not included | annex II |
| 72 | Phylloscopus trochilus | Nbr. | LC | LC | not included | annex II |
| 73 | Pica pica | Nbr. | LC | LC | Annex II/B | annex II |
| 74 | Picus viridis | br. | LC | LC | not included | annex II |
| 75 | Saxicola rubetra | br. | LC | LC | not included | annex III |
| 76 | Saxsicola rubicola | br. | LC | LC | not included | annex II |
| 77 | Sitta neumayer | br. | LC | LC | not included | annex II |
| 78 | Streptopelia decaocto | Nbr. | LC | LC | Annex II/B | annex II |
| 79 | Streptopelia turtur | br. | LC | VU | Annex II/B | annex III |
| 80 | Sturnus vulgaris | br. | VU | VU | Annex II/B | annex III |
| 81 | Sylvia atricapilla | br. | LC | LC | not included | not included |
| 82 | Tringa ochropus | Nbr. | LC | LC | not included | annex II |
| 83 | Turdus merula | br. | LC | LC | Annex II/B | annex II |
| 84 | Turdus philomelos | br. | LC | LC | Annex II/B | annex III |
| 85 | Turdus viscivorus | Nbr. | LC | LC | Annex II/B | annex III |
| 86 | Tyto alba | br. | LC | LC | not included | annex III |
| 87 | Upupa epops | br. | LC | LC | not included | annex II |
| 88 | Vanellus vanellus | br. | LC | VU | Annex II/B | annex II |

Legend:

Br- Breeding

Nbr- Not breeding

Annex 2.2 Species of birds that require special protection

|  | SPECIES | PRESENCE | IUCN | IUCN EU | BIRD DIRECTIVE | BERN CONVENTION |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Alectoris greaca | br. | VU | VU | Annex I | annex II |
| 2 | Aquila heliaca | br. | LC | VU | Annex I | annex II |
| 3 | Circus cyaneus | Nbr. | LC | NT | Annex I | annex II |
| 4 | Circus pygargus | Nbr. | LC | NT | Annex I | annex II |
| 5 | Ciconia ciconia | br. | LC | LC | Annex I | annex II |
| 6 | Coracias garrulus | br. | LC | NT | Annex I | annex II |
| 7 | Falco naumanni | br. | LC | LC | Annex I | annex II |
| 8 | Fringilla coelebs | Nbr. | NT | NT | Annex I | annex II |
| 9 | Lanius excubitor | br. | LC | VU | Annex I | annex II |
| 10 | Streptopelia turtur | br. | LC | VU | Annex II/B | annex III |
| 11 | Sturnus vulgaris | br. | VU | VU | Annex II/B | annex III |
| 12 | Vanellus vanellus | br. | LC | VU | Annex II/B | annex II |

Legend:

Br- Breeding

Nbr- Not breeding

Annex 2.3 Species of birds observed during the survey (December 2016 – February 2017)

|  | Species | Presence | IUCN | IUCN Europe | Bird Directive | Bern Convention |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Accipiter gentilis | Nbr. | LC | LC | Not included | Annex II |
| 2 | Ardea alba | Br. | LC | LC | Annex I | Annex II |
| 3 | Athene noctua | Br. | LC | LC | Not included | Annex II |
| 4 | Aquila chrysaetos | Br. | LC | LC | Annex I | Annex II |
| 5 | Aquila heliaca | Br. | LC | VU | Annex I | Annex II |
| 6 | Buteo buteo | Br. | LC | LC | Not included | Annex II |
| 7 | Buteo rufinus | Br. | LC | LC | Annex I | Annex II |
| 8 | Carduelis carduelis | Br. | LC | LC | Not included | Annex II |
| 9 | Chloris chloris | Nbr. | LC | LC | Not included | Annex II |
| 10 | Circus cyaneus | Nbr. | LC | NT | Annex I | Annex II |
| 11 | Coccosthraustes coccosthraust | Br. | LC | LC | Not included | Annex III |
| 12 | Columba livia | Br. | LC | LC | Not included | Annex II |
| 13 | Columba palumbus | Nbr. | LC | LC | Annex II/A; III/A | Annex III |
| 14 | Corvus corax | Br. | LC | LC | Not included | Not included |
| 15 | Corvus corone | Nbr. | LC | LC | Annex II/B | Annex III |
| 16 | Corvus monedula | Br. | LC | LC | Annex II/B | Not included |
| 17 | Dendrocopos minor | Br. | LC | LC | Not included | Annex II |
| 18 | Dendrocopos syriacus | Br. | LC | LC | Annex I | Annex II |
| 19 | Falco tinnunculus | Br. | LC | LC | Not included | Annex II |
| 20 | Falco peregrinus | Br. | LC | LC | Annex I | Annex II |
| 21 | Galerida cristata | Br. | LC | LC | Not included | Annex II |
| 22 | Motacilla alba | Br. | LC | LC | Not included | Annex II |
| 23 | Motacilla flava | Nbr. | LC | LC | Not included | Annex II |
| 24 | Parus major | Nbr. | LC | LC | Not included | Annex II |
| 25 | Passer domesticus | Br. | LC | LC | Not included | Annex II |
| 26 | Passer hispaniolensis | Br. | LC | LC | Not included | Not included |
| 27 | Perdix perdix | Br. | LC | LC | Annex II/A; III/A | Annex II |
| 28 | Phylloscopus collybita | Nbr. | LC | LC | Not included | Annex II |
| 29 | Pica pica | Nbr. | LC | LC | Annex II/B | Annex II |
| 30 | Picus viridis | Br. | LC | LC | Not included | Annex II |
| 31 | Sitta neumayer | Br. | LC | LC | Not included | Annex II |
| 32 | Streptopelia decaocto | Nbr. | LC | LC | Annex II/B | Annex II |
| 33 | Sturnus vulgaris | Br. | VU | VU | Annex II/B | Annex III |
| 34 | Turdus merula | Br. | LC | LC | Annex II/B | Annex II |
| 35 | Turdus philomelos | Br. | LC | LC | Annex II/B | Annex III |
| 36 | Turdus viscivorus | Nbr. | LC | LC | Annex II/B | Annex III |

ANNEX 3. Photographs from the habitats in the project area & Habitat map

ANNEX 4. Photographs of birds in the project area

| Circus cyaneus (female) | Sitta neumayer |
| --- | --- |
| Picus viridis | Carduelis carduelis |
| Dendrocopos syriacus | Carduelis chloris |
| Carduelis cannabina and Carduelis carduelis | Emberiza cirlus |
| Aquila chrysaetos | Aquila chrysaetos |
| Buteo buteo | Buteo buteo |
| Buteo rufinus and Corvus corax | Circus cyaneus |
| Circus cyaneus | Circus cyaneus |
| Circus cyaneus | Circus cyaneus |
| Imperial eagle – Aquila heliaca | Imperial eagle – Aquila heliaca |
| Egretta alba | Galerida cristata |
| Columba livia | Sturnus vulgaris |

ANNEX 5. Precaution mitigation measures (MIT-O-06)

Establishment and maintenance of two controlled feeding places for Aquila heliaca (and other species of birds)

Description:

Controlled feeding in specific places known as supplementary feeding is considered a key management tool for the conservation of scavenger and bird of prey populations. This provision of safe and high quality food to target species can decrease the effects of mainly anthropogenic threats.

Insight and experience has shown that feeding of birds on specially prepared feeding places is a very important measure which will attract and keep birds of prey and vultures in the areas of the feeding and nesting territories. It provides a number of benefits for the birds such as the increase of food availability, increase of survival rates, improvement of breeding performance, increase of micronutrients availability such as calcium, reduction of the risk of poisoning as well as some environmental benefits. Specifically for the case of the Imperial eagles, supplementary feeding stations may increase the population viability and also contribute to the settlement of new breeding territories as their abundance of food has been found to be closely related with the formation of communal roosts that hold congregations of non-breeding individuals.

Controlled feeding location will also provide a valuable food source for other affected raptors i.e. Golden eagles, Kites and Buzzards. Moreover, it will give opportunity to local livestock farmers to dispose of unwanted animal carcasses in an environmental friendly way and it serves as a fulcrum for awareness in the local community on conservation issues.

Regulations and Permits:

The latest Commission Regulation (EC 142/2011) comes with derogations from certain provisions of Regulation 1069/2009 which are applicable for vultures. In contrast to previous regulations, Article 14 in EC 142/2011 describes the possibility of feeding certain species at feeding stations, promoting and highlighting the natural role of vultures as “cleaners” of the ecosystem. In addition, the Imperial eagles, (along with Bearded Vulture, Black Vulture, Griffon Vulture, Egyptian Vulture Golden Eagle, White-tailed Eagle and Black Kite) is one of the species of necrophagous birds that can be fed with Category 1 material.

Regulation EC 1774/2002 – lays down rules regarding classification, collection, transport, storage, handling, processing and use of animal by-products.

Regulation EC  1069/2009 Facilitates  the  efficient  management  of  animal  by-‐ products,  while  it  maintains  the  high  level  of   protection  against  risks  to  public  health.

Regulation EU 142/2011 implements the EC 1069/2009 and actually replaces EC 1774/2002.

For the establishment of a feeding station a set of permits from the relevant authorities are required. First, an authorization has to be granted by the competent authority (this would be the Veterinary Directorate). For this, an application has to be submitted to the Directorate along with an official approval by the Ministry of Environment and Physical Planning, as well as a technical description of the controlled feeding station and the targeted species with program for feeding.

In addition, depending on the ownership of the land (public or private) as well as the use of the land (forest land, farmland, etc.) another set of permits might be required.

Technical requirements:

An important factor during creating a controlled feeding place is the selection of the site. The location of the feeding stations should be studied carefully in order to increase the possibility of its use by certain species. Feeding sites must be located in areas that will enable the attendance and benefits for the target species. Hence, many different parameters should be considered and evaluated beforehand. These include variables such as geographic characteristics, distance to the breeding territories, absence of infrastructure that may be dangerous to birds (such as power lines or wind farms) etc. Farmers should be consulted first in order to ensure their collaboration and avoid future problems concerning their sensitivity regarding disease transmission. Moreover, a feeding site could not operate properly without the acceptance from local communities. So a series of awareness activities such as meetings and presentations with local stakeholders should take place before the creation of the controlled feeding places.

Supplementary feeding stations should be optimized in order to favor a target-species within the scavenger guild. In that way their conservation function is channeled to the most threatened populations which in our case are medium sized birds like the Imperial eagles.

Fencing is essential for a controlled feeding station in order to avoid mammal scavengers to approach and consume the remains intended for birds. The ideal fence should be erected to create an area of 100x100 metres but a smaller area (minimum of 50x50 meters) could also be acceptable (Alivizatos 2004). Any smaller area could affect the ability of larger birds to land as well fly away. In addition, the fence should be placed at least 1 meter away from trees, bushes or rocks in order to avoid access to animals that can jump over it. The fence should be at least 1.8 meters high above the ground. Stakes of 2.5 meters should be buried 0.5 m into the ground and placed at distance of 2.5 meters. The mesh should be strong enough to restrain animals from breaking into the feeding station. Thus, it is recommendable to use a mesh with a maximum opening of 50 mm. The mesh should also be buried into the ground in 0.2 to 0.5 meters and enhanced (covered) with concrete, to prevent mammals from digging their way through. The entrance of the feeding station should be 2. 5 meters wide to permit the entrance of the vehicle that will transfer the carcasses or any other animal by-products. Finally, a concreted pool with dimensions 3x2x0.2 meters should be constructed at the feeding station.

Explanations on feeding process:

How much and what kind food should be deposited is crucial question when designing a supplementary feeding program for some target species. Smaller carcasses benefit smaller scavengers. The disposal of many small pieces of remains dispersed in the feeding site appears to be an action that would favor the Imperial eagles. Providing lots of scattered remains not only allows rapid access to the food but also leads to adult birds to not dominate the food sources. Thus, not only existing pairs can feed in order to improve their breeding performance but it is also a way to favor sub-adult survival. Various animal remains can be used such as pieces of goats, sheep, pigs and chickens. In case of large remains like cattle, the carcass should be partially skinned/opened to allow immediate access to Imperial eagles.

Regular supplementary feeding stations should be supplied frequently (depending on the rate remains are consumed by the birds visiting the site) in order to ensure the constant availability of carrion but no less than once per week. Stable food sources are very important since they have been associated with the presence of communal roosts and they could maximize the probability of attracting new individuals – breeding units (floaters) to recolonize the abandoned territories. More efficient way of managing supplementary feeding would be the creation of a network of feeding stations which would be supplied sporadically with food resembling the traditional practices of the past.

Locations:

1st feeding place – two macro locations were proposed on the photo below. The Contractor to consult relevant Bird Expert during selection of micro location for the feeding place.

2nd feeding place – micro location is selected (photo below) since it has been active feeding place in the past. Contact with the previous operator (National conservation association AQUILA - nca.aquila@gmail.com) of the feeding place was established and positive feedback was received on the idea to reboot the feeding place.

Period of implementation:

Two controlled feeding places shall be established by the Contractor during pre-construction phase i.e. prior the start of initial construction activities of CWMF. Responsible for total maintenance and monitoring of feeding places during the construction phase will be Contractor.

Contractor is responsible to hand over these feeding places to the Operator prior start of operational phase including all of the documentation for the purchased equipment as well monitoring data.

Operator is responsible to maintain and monitoring the feeding places during first five years of operational phase. Afterwards, based on the monitoring on the feeding places as well results from the monitoring presented in Chapter 7.2, the Operator in consultations with NGOs, independent bird experts and MoEPP will decide whether  is essential the feeding places to continue with operation.

Monitoring requirements:

Monitoring of feeding place is crucial in order to assess its functionality. It can provide data on number of birds attending the feeding place, seasonal changes in their presence, age structure, and also follow the breeding success (fresh juveniles also use the feeding site after their first flights).

In order to monitor the use of the feeding station by birds, a minimum of one visit should be conducted the same day the carrion is disposed, although subsequent visits are recommended. In some occasions (adverse weather condition, large number of birds present in the region, presence of marked birds), two day monitoring might be needed.

The observation point should be at least 300-400 metres away from the remains so that the behavior of birds is not altered.

Records from the monitoring must include the following: date, start and end of survey, the amount of available food (kg) (including the amount deposited that day, if any) and the approximate amount consumed by the birds (%). Also any disturbance has to be noted. In either case, the species and the number of individuals present at the station and in a radius of 200 meters should be noted as well as a distinction between the birds that are actively feeding and the ones that are not, but found in the vicinity of the feeding area like the fly-overs.

Financial projections for establishment, maintenance and monitoring of two feeding places:

| Costs for establishment of two feeding place and equipment |  |  |
| --- | --- | --- |
| No | Item | Cost in EUR |
| 1 | Construction of the area | 6000 |
| 2 | Access road (paved) 1 km approximate | 4000 |
| 3 | Material for fencing | 3000 |
| 4 | Equipment (Camera traps) | 1000 |
| 5 | Optical Devices (Binoculars, Telescope, Tripod | 3000 |
| 6 | Vehicle 4x4 Purchase and Modification (optional, Contractor/Operator may provide vehicle) | 10.000 |
| 7. | Trailer purchase | 1000 |
| Total |  | 28.000 EUR for two feeding places |
| Yearly Operational Costs for two feeding places (including human resources) |  |  |
| No | Item | Cost in EUR |
| 1 | Daily Labour - feeding (60x25eur) | 3000/year for 2 locations |
| 2 | Daily Labour - monitoring (60x30eur) | 3600/year for 2 locations |
| 3 | Car Running Costs (gas) | 1500/year |
| 4 | Car spare / Maintenance | 800/year |
| 5 | Car Insurance, tax, etc. | 250/year |
| 6 | Maintenance of fence | 350/year |
|  | Total | 9.500 EUR |

Summary costs for construction, maintenance and monitoring of two feeding placed during construction and first 5 years of operational phase

| Establishment of two feeding places | 28.000 |
| --- | --- |
| Maintenance during construction (for projection that construction phase will take 1 year) | 9.500 |
| Maintenance during operation (first 5 years) | 47.500 |
| Total | 85.000 EUR |

Examples of establishing and maintaining controlled feeding places in Macedonia (photos)

# References

- Project “Wings over the Balkans: Preparation of Western Balkan countries to implement the Directive on wild birds European Union”- Macedonian Ecological Society

- http://www.moepp.gov.mk/?page_id=4920&lang=en

- Melovski, Lj., M. Velevski, V. Matevski, V. Avukatov & A. Sarov (2012). Using important plant areas and important bird areas to identify Key Biodiversity Areas in the Republic of Macedonia. Journal of Threatened Taxa 4(8): 2766–2778

- San Miguel A. 2008. Management of Natura 2000 habitats. 6220 *Pseudo-steppe with grasses and annuals of the Thero-Brachypodietea. European Commission

- Brajanoska, R., Melovski, Lj., Hristovski, S., Sarov, А., Avukatov, V. (2011). Brown Bear Corridors management Plan. Report under the Project: “Development of the National Ecological Network in the Republic of Macedonia (MAK-NEN)”. Macedonian Ecological Society, Skopje.

- Magdalena Vaverková, Dana Adamcová. (2011). POTENTIAL IMPACT OF TWO LANDFILLS ON THE NEAR VICINITY WITH THE USE OF BIOINDICATORS, Mendel University in Brno

- https://www.ewt.org.za/species%20factsheets/bop/new/Crows%20and%20raptors.pdf

- R1412: Rodent Control at Landfill Sites, General Manager Engineering

- Methane burner impacts on raptors. (2013). Keeping Company with Kestrels, Inc. and EDM Internacional, INC.

- MANUAL FOR GULL CONTROL AT MASSACHUSETTS LANDFILLS, Metropolitan District Commission, Division of Watershed Management Massachusetts Department of Environmental Protection, Bureau of Waste Prevention Massachusetts Division of Fisheries & Wildlife USDA APHIS Wildlife Services, May 1998

- www.landfillconservancies.com

- European Commission, DG Environment http://eunis.eea.europa.eu/habitats-code-browser.jsp

- The Regional Waste Management Plan of North-East Region - Enviroplan S.A., 2014;

- The Regional Waste Management Plan of East Region - Enviroplan S.A., 2014;

- Bregalnica River Basin Management Plan - Ministry of Environment and Physical Planning of Macedonia, State Secretariat of Econonomic Affairs, Switzerland, final version, August 2016;

- Environmental Impact Assessment. Regional Sanitary Landfill at the Možura Site, for the Municipalities of Bar and Ulcinj Coastal Region, April 2010

- Allawuna Landfill Vegetation and Fauna Assessment; Bowman & Associates; ENV Australia Pty Ltd; October 2012

Project “Wings over the Balkans: Preparation of Western Balkan countries to implement the Directive on wild birds European Union”- Macedonian Ecological Society

http://www.moepp.gov.mk/?page_id=4920&lang=en

Melovski, Lj., M. Velevski, V. Matevski, V. Avukatov & A. Sarov (2012). Using important plant areas and important bird areas to identify Key Biodiversity Areas in the Republic of Macedonia. Journal of Threatened Taxa 4(8): 2766–2778

San Miguel A. 2008. Management of Natura 2000 habitats. 6220 *Pseudo-steppe with grasses and annuals of the Thero-Brachypodietea. European Commission

Brajanoska, R., Melovski, Lj., Hristovski, S., Sarov, А., Avukatov, V. (2011). Brown Bear Corridors management Plan. Report under the Project: “Development of the National Ecological Network in the Republic of Macedonia (MAK-NEN)”. Macedonian Ecological Society, Skopje.

Magdalena Vaverková, Dana Adamcová. (2011). POTENTIAL IMPACT OF TWO LANDFILLS ON THE NEAR VICINITY WITH THE USE OF BIOINDICATORS, Mendel University in Brno

https://www.ewt.org.za/species%20factsheets/bop/new/Crows%20and%20raptors.pdf

R1412 : Rodent Control at Landfill Sites, General Manager Engineering

Methane burner impacts on raptors. (2013). Keeping Company with Kestrels, Inc. and EDM Internacional, INC.

MANUAL FOR GULL CONTROL AT MASSACHUSETTS LANDFILLS, Metropolitan District Commission, Division of Watershed Management Massachusetts Department of Environmental Protection, Bureau of Waste Prevention Massachusetts Division of Fisheries & Wildlife USDA APHIS Wildlife Services, May 1998

www.landfillconservancies.com